| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Turin, IT, 14-22 July 2011 | Document: JCTVC-F162 |
| --- | --- |

| Title: | Entropy coding performance simulations |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Thomas Davies Arild Fuldseth | Email: | thdavies@cisco.com arilfuld@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

Various coding conditions are simulated with both CAVLC and CABAC. Under common conditions it is reported that CABAC provides gain between 6.1% and 7.6%. The performance of the entropy coders is also investigated with various realistic encoder restrictions: no RDOQ, no adaption during RDO mode search, and tile-based coding. These assumptions individually reduce the average gap between CABAC and CAVLC by 0.9%, 1.75% and 0.75% respectively in LC settings. When RDOQ is off, RDO adaption is off  and tiles are also enabled the gap is reported to be between 2.0% and 3.8% on average in LC settings.

# Introduction

Five experiments have been performed. The purpose was to determine the performance of the two entropy coders in a range of conditions, including the common conditions [1] but also including conditions which might better match realistic encoding tools and techniques. We had observed in the past that in our experience of real AVC implementations the gap between entropy coding methods is lower than that observed in the JM. We had also observed that the gap between entropy encoders in HM seemed to be lower when the pictures are segmented into tiles and slices. Since fast RDO techniques and tiles or slices are likely to be features of a great many implementations, we have investigated performance under more realistic conditions.

## The experiments

In the first experiment, HM3.2 was used to determine the performance of CAVLC and CABAC under all common conditions. From this it appeared that there was no significant difference in the gap between LC and HE conditions, and therefore the LC settings only were used for the remaining experiments.

Overall an average of 6.1-7.6% gap was found between CAVLC and CABAC across all conditions.

In the second experiment, HM3.2 was used with RDOQ off for CAVLC and CABAC on all LC conditions. A gap of 5.4-5.8% was found between CAVLC and CABAC for these configurations, with an average reduction of 0.9% in the difference.

In experiment 3, HM3.0 modified to support tiles was used with CAVLC and CABAC on all LC configurations. The number of tiles varied according to the picture dimensions. A gap of 5.0-6.4% was found.

In experiment 4, HM3.2 was modified so that RDO search was performed without context adaption within an LCU – contexts are re-initialised at the beginning of LCU RDO search only. This is already the case for CALVC, so the CABAC bin coding engine was subclassed to perform coding without updating LPS probabilities, and this subclass used in RDO search. Although encoding was still run (to minimize the amount of software changes), this is very nearly equivalent to using fixed entropy costs for each syntax element within an LCU. A gap of 3.9-5.8% was found, representing an average reduction of 1.75% in the difference.

In experiment 5, the experiment 4 modifications were also applied to the HM3.0 tiles software, and experiments 2, 3, and 4 combined. A gap of 2.0-3.8% was found between CABAC and CAVLC. This demonstrates that the losses incurred in experiments 2, 3 and 4 accumulate.

# Results

## Simulation environment

Experiments 1, 2 and 4 were run on a homogeneous cluster with 64-bit Intel Xeon X7560 2.27GHz processors. The operating system was 64 bit Debian, and a single process was used per core (no hyperthreading).

Experiments 3 and 5 were … TBC

## Experiment 1: switching entropy coders under common conditions

Software: HM3.2

Configurations tested: All common conditions for 3.2 [1].

Simulation parameters: --SymbolMode=0 on HE settings; --SymbolMode=1 on LC settings

Summary results:

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 6.3 | -1.9 | -4.3 | -5.9 | 2.8 | 4.7 |
| Class B | 7.5 | -2.9 | -4.7 | -6.7 | 3.6 | 5.5 |
| Class C | 6.0 | -0.9 | -1.3 | -5.7 | 1.3 | 1.7 |
| Class D | 5.3 | -0.3 | 0.0 | -5.0 | 0.6 | 0.4 |
| Class E | 9.1 | -8.8 | -7.3 | -8.3 | 10.9 | 9.0 |
| Overall | 6.7 | -2.7 | -3.4 | -6.3 | 3.5 | 4.1 |
| Enc Time[%] | 91% |  |  | 118% |  |  |
| Dec Time[%] | 94% |  |  | 109% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 9.3 | -7.7 | -8.0 | -8.2 | 9.4 | 8.2 |
| Class B | 9.0 | -5.7 | -7.0 | -8.1 | 6.6 | 8.4 |
| Class C | 6.4 | -3.3 | -3.5 | -6.2 | 3.4 | 3.8 |
| Class D | 5.4 | -4.2 | -3.9 | -5.2 | 5.0 | 4.6 |
| Class E |  |  |  |  |  |  |
| Overall | 7.6 | -5.2 | -5.7 | -7.0 | 6.1 | 6.4 |
| Enc Time[%] | 99% |  |  | 110% |  |  |
| Dec Time[%] | 97% |  |  | 104% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 8.5 | -8.2 | -11.8 | -8.0 | 9.5 | 14.7 |
| Class C | 5.9 | -6.6 | -8.0 | -5.9 | 6.7 | 8.3 |
| Class D | 5.0 | -10.7 | -10.9 | -4.8 | 12.8 | 13.7 |
| Class E | 7.1 | -12.4 | -11.3 | -6.9 | 14.6 | 13.3 |
| Overall | 6.7 | -9.2 | -10.5 | -6.5 | 10.6 | 12.6 |
| Enc Time[%] | 99% |  |  | 112% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 8.1 | -9.2 | -13.0 | -7.2 | 11.6 | 16.5 |
| Class C | 5.9 | -7.0 | -8.3 | -5.7 | 7.4 | 9.1 |
| Class D | 4.9 | -10.3 | -9.8 | -4.8 | 12.0 | 12.8 |
| Class E | 7.0 | -12.2 | -11.9 | -6.4 | 15.4 | 14.8 |
| Overall | 6.5 | -9.5 | -10.8 | -6.1 | 11.4 | 13.4 |
| Enc Time[%] | 95% |  |  | 114% |  |  |
| Dec Time[%] | 98% |  |  | 101% |  |  |

## Experiment 2: RDOQ off

Software: HM3.2

Configurations tested: LC common conditions for 3.2

Simulation parameters: --RDOQ=0 and --SymbolMode=0 or --SymbolMode=1

Summary results:

|  | All Intra LC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -4.8 | -3.3 | -3.8 |
| Class B | -5.8 | -4.7 | -4.2 |
| Class C | -4.3 | -3.0 | -2.6 |
| Class D | -3.7 | -3.5 | -3.2 |
| Class E | -7.2 | -1.5 | -1.4 |
| Overall | -5.1 | -3.4 | -3.2 |
| Enc Time[%] | 129% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access LC |  |  |
|  | Y | U | V |
| Class A | -5.1 | 3.5 | 2.9 |
| Class B | -6.6 | -0.6 | -0.1 |
| Class C | -4.7 | -0.6 | -0.1 |
| Class D | -3.7 | -0.3 | -0.2 |
| Class E |  |  |  |
| Overall | -5.1 | 0.4 | 0.6 |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 94% |  |  |
|  | Low delay B LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -6.8 | 3.9 | 6.9 |
| Class C | -4.7 | 3.2 | 4.3 |
| Class D | -3.7 | 6.6 | 7.8 |
| Class E | -6.5 | 4.2 | 5.0 |
| Overall | -5.4 | 4.5 | 6.1 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 93% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -6.6 | 4.4 | 7.3 |
| Class C | -4.7 | 3.1 | 4.0 |
| Class D | -3.7 | 5.9 | 7.6 |
| Class E | -6.4 | 3.3 | 4.2 |
| Overall | -5.4 | 4.3 | 6.0 |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 95% |  |  |

The mean difference between these luma BD-rate results and those for experiment 1 is 0.9%.

## Experiment 3: Tiles

Software: HM-3.0-tiles (attached)

Configurations tested: LC common conditions

Simulation parameters: Uniform tile distribution, numbers of tiles as shown below:

| Class | Number of tiles |  | Tile dimensions (in units of LCUs) |  |
| --- | --- | --- | --- | --- |
|  | horizontally | vertically | horizontally | vertically |
| A | 5 | 5 | 8 | 5 |
| B | 6 | 3 | 5 | 6 |
| C | 2 | 2 | 7 | 4 |
| D | 1 | 1 | 7 | 4 |
| E | 4 | 3 | 5 | 4 |

Summary results:

|  | All Intra LC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -5.8 | 1.9 | 4.8 |
| Class B | -6.7 | 2.8 | 4.3 |
| Class C | -5.7 | 0.8 | 1.2 |
| Class D | -5.0 | 0.6 | 0.4 |
| Class E | -7.8 | 7.4 | 5.9 |
| Overall | -6.2 | 2.5 | 3.2 |
| Enc Time[%] | 167% |  |  |
| Dec Time[%] | 127% |  |  |
|  | Random Access LC |  |  |
|  | Y | U | V |
| Class A | -6.8 | 9.5 | 8.5 |
| Class B | -7.4 | 7.8 | 8.5 |
| Class C | -6.1 | 3.8 | 4.0 |
| Class D | -5.2 | 4.6 | 4.4 |
| Class E |  |  |  |
| Overall | -6.4 | 6.5 | 6.5 |
| Enc Time[%] | 123% |  |  |
| Dec Time[%] | 113% |  |  |
|  | Low delay B LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -6.6 | 12.8 | 16.9 |
| Class C | -5.5 | 7.9 | 9.3 |
| Class D | -5.0 | 12.6 | 13.0 |
| Class E | -3.0 | 13.8 | 14.3 |
| Overall | -5.3 | 11.7 | 13.5 |
| Enc Time[%] | 128% |  |  |
| Dec Time[%] | 128% |  |  |
|  | Low delay P LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -6.1 | 12.8 | 15.8 |
| Class C | -5.5 | 7.6 | 8.6 |
| Class D | -4.8 | 11.9 | 13.2 |
| Class E | -2.6 | 12.0 | 13.1 |
| Overall | -5.0 | 11.1 | 12.8 |
| Enc Time[%] | 147% |  |  |
| Dec Time[%] | 138% |  |  |

The mean difference between these luma BD-rate results and those for experiment 1 is 0.75%.

## Experiment 4: statistical adaption in RDO off

Software: HM3.2-RDO-adaption-off-patch (attached)

Configurations tested: LC common conditions

Simulation parameters: --SymbolMode=0 or --SymbolMode=1

Summary results:

|  | All Intra LC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -4.8 | -0.3 | 0.5 |
| Class B | -5.3 | 0.9 | 1.1 |
| Class C | -4.7 | 0.6 | 0.6 |
| Class D | -4.4 | 0.2 | 0.1 |
| Class E | -5.9 | 0.9 | 0.5 |
| Overall | -5.0 | 0.5 | 0.6 |
| Enc Time[%] | 109% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access LC |  |  |
|  | Y | U | V |
| Class A | -6.1 | 9.4 | 6.7 |
| Class B | -7.2 | 7.3 | 5.7 |
| Class C | -5.3 | 6.3 | 6.2 |
| Class D | -4.3 | 6.1 | 5.8 |
| Class E |  |  |  |
| Overall | -5.8 | 7.3 | 6.1 |
| Enc Time[%] | 108% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -5.8 | 16.8 | 19.7 |
| Class C | -4.0 | 13.9 | 15.5 |
| Class D | -2.8 | 18.8 | 19.9 |
| Class E | -4.9 | 8.8 | 5.2 |
| Overall | -4.4 | 15.1 | 16.0 |
| Enc Time[%] | 109% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Low delay P LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -4.9 | 13.1 | 14.5 |
| Class C | -3.7 | 10.4 | 11.6 |
| Class D | -3.0 | 12.6 | 12.6 |
| Class E | -3.8 | 6.1 | 3.5 |
| Overall | -3.9 | 11.0 | 11.2 |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 95% |  |  |

The mean difference between these luma BD-rate results and those for experiment 1 is 1.75%.

## Experiment 5: RDOQ off + RDO statistical adaption off + tiles

Software: HM3.0-tiles-RDO-adaption-off-patch (attached)

Configurations tested: LC common conditions

Simulation parameters: --RDOQ=0 and --SymbolMode=0 or –SymbolMode=1. Uniform tile distribution with numbers of tiles as shown below:

| Class | Number of tiles |  | Tile dimensions (in units of LCUs) |  |
| --- | --- | --- | --- | --- |
|  | horizontally | vertically | horizontally | vertically |
| A | 5 | 5 | 8 | 5 |
| B | 6 | 3 | 5 | 6 |
| C | 2 | 2 | 7 | 4 |
| D | 1 | 1 | 7 | 4 |
| E | 4 | 3 | 5 | 4 |

Summary results:

|  | All Intra LC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -3.7 | -6.6 | -7.5 |
| Class B | -4.2 | -6.5 | -7.2 |
| Class C | -2.9 | -3.6 | -3.4 |
| Class D | -2.7 | -3.6 | -3.3 |
| Class E | -5.3 | -9.0 | -8.3 |
| Overall | -3.7 | -5.7 | -5.9 |
| Enc Time[%] | 150% |  |  |
| Dec Time[%] | 108% |  |  |
|  | Random Access LC |  |  |
|  | Y | U | V |
| Class A | -3.9 | 0.1 | -1.9 |
| Class B | -4.9 | -2.6 | -4.0 |
| Class C | -3.5 | -0.3 | -0.5 |
| Class D | -2.7 | -1.2 | -1.3 |
| Class E |  |  |  |
| Overall | -3.8 | -1.1 | -2.1 |
| Enc Time[%] | 112% |  |  |
| Dec Time[%] | 104% |  |  |
|  | Low delay B LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -3.2 | 9.8 | 10.3 |
| Class C | -2.0 | 9.2 | 9.7 |
| Class D | -1.2 | 9.7 | 10.7 |
| Class E | -1.2 | -0.3 | -0.5 |
| Overall | -2.0 | 7.7 | 8.2 |
| Enc Time[%] | 110% |  |  |
| Dec Time[%] | 103% |  |  |
|  | Low delay P LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -3.4 | 5.3 | 5.4 |
| Class C | -2.1 | 5.3 | 5.4 |
| Class D | -1.4 | 4.0 | 5.1 |
| Class E | -1.0 | -4.7 | -3.3 |
| Overall | -2.1 | 3.1 | 3.7 |
| Enc Time[%] | 111% |  |  |
| Dec Time[%] | 100% |  |  |

The mean difference between these luma BD-rate results and those for experiment 1 is 3.6%.

# Patent rights declaration(s)

Cisco Systems may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JCTVC-E700, “Common test conditions and software reference configuration”, F. Bossen (DoCoMo), 5th JCT-VC Meeting, Geneva, CH, 16-23 March, 2011