| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F166 |
| --- | --- |

| Title: | CE9:Cross-verification on disabling spatial MVP scaling process for smaller PUs(SP01/02) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yusuke Itani Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Itani.Yusuke@dx.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution reports performance cross-check results on disabling spatial MVP scaling process for smaller PUs proposed by TI in JCTVC-E481 [3] according to the description of CE9 [1] at last Geneva meeting.. There are two versions based on PU size. In this contribution, results of all versions are reported. The verification tests have been conducted using common test conditions[2]. It is confirmed that the results perfectly match with the report made by the proponent.

# Introduction

Performance verification on CE9 approved at the last Geneva meeting [1] was conducted. The verified method is disabling spatial MVP scaling process for smaller PUs proposed by TI in JCTVC-E481 [3]. This work was conducted using modified HM3.0 software provided by TI.

# Simulation Results

The following two settings recommended by TI are cross-verified:

  - Only for 4x4 PUs (SP01)

  - PUs smaller than or equal to 8x8 (SP02)

The results of two settings are provided in Table 1,.Table2. It has been verified the results presented in this section perfectly match with the results provided by TI. In average, SP01 does not cause performance degradation, while 0.1% coding loss can be seen with SP02.

Table 1. only for 4x4 PUs (SP01)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | -0.1 | 0.0 | 0.0 | -0.1 | 0.1 |
| Class D | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | -0.2 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | -0.2 | 0.0 | -0.2 | -0.2 |
| Class C | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | -0.2 |
| Class D | -0.1 | -0.1 | 0.2 | 0.0 | -0.2 | -0.2 |
| Class E | 0.0 | 0.3 | 0.8 | 0.0 | 0.2 | 0.6 |
| Overall | 0.0 | 0.1 | 0.2 | 0.0 | -0.1 | -0.1 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2. PUs smaller than or equal to 8x8 (SP02)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| Class B | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Class C | 0.1 | 0.1 | 0.1 | 0.2 | 0.1 | 0.2 |
| Class D | 0.2 | 0.3 | 0.1 | 0.2 | 0.2 | 0.1 |
| Class E |  |  |  |  |  |  |
| Overall | 0.1 | 0.1 | 0.0 | 0.1 | 0.1 | 0.1 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | -0.2 |
| Class C | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Class D | -0.1 | 0.3 | 0.6 | 0.0 | 0.0 | 0.2 |
| Class E | 0.0 | 0.2 | 0.3 | 0.0 | 0.1 | 0.8 |
| Overall | 0.0 | 0.1 | 0.2 | 0.0 | 0.0 | 0.2 |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Conclusion

We confirmed that the implementation of TI’s proposal in HM-3.0 is consistent with the text, and all test results perfectly matched with the results provided by the proponent.

References:

- Yu-Wen, et.al. “Description of Core Experiment 9: MV Coding and Skip/Merge Operations,” JCTVC-E709, Mar. 2011.

- Frank Bossen, “Common test conditions and software reference configurations,” JCTVC- E700, Mar. 2011.

- B.Bross, et.al. “BoG report of CE9: MV coding and Skip/Merge operations,” JCTVC-E481, Mar. 2011