| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F168 |
| --- | --- |

| Title: | CE9:Modified AMVP Scaling Process(SP05) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yusuke Itani Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Itani.Yusuke@dx.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution reports performance verification results on modified AMVP scaling process proposed in JCTVC-E064 [3] according to the description of CE9 [1] at the last Geneva meeting. A performance evaluation using HM-3.0 was conducted using common test conditions [2]. The performance shows, almost the same as that of the anchor for HE LC and LC LD and 0.1% and 0.2% coding loss for HE RA and LC RA. The decoding time and encoding time are almost the same as that of HM3.0 anchor.

# Introduction

In this contribution, we report simulation results on modified AMVP scaling process proposed in JCTVC-E064 [3]. The evaluated technique provides a simplified design of AMVP process and reduces encoder and decoder complexity. This work was conducted using modified HM3.0 software.

# Simulation results

Simulations were conducted for the above proposal using the common conditions defined in [2]. The result is provided in Table 1. In average, the performance with the technique shows almost the same as that of the anchor for HE LC and LC LD and 0.1% and 0.2% coding loss for HE RA and LC RA.

Table 1. Simulation result

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1 | 0.3 | 0.2 | 0.3 | 0.2 | 0.2 |
| Class B | 0.1 | 0.1 | 0.0 | 0.2 | 0.1 | 0.2 |
| Class C | 0.1 | 0.2 | 0.2 | 0.2 | 0.2 | 0.3 |
| Class D | 0.2 | 0.2 | 0.1 | 0.2 | 0.2 | 0.1 |
| Class E |  |  |  |  |  |  |
| Overall | 0.1 | 0.2 | 0.1 | 0.2 | 0.2 | 0.2 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | 0.0 | 0.1 | -0.1 | -0.2 |
| Class C | 0.0 | 0.0 | -0.1 | 0.0 | -0.1 | -0.2 |
| Class D | -0.1 | -0.4 | -0.1 | 0.0 | 0.0 | -0.1 |
| Class E | 0.0 | 0.7 | -0.5 | -0.1 | 0.2 | 0.1 |
| Overall | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | -0.1 |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

# Conclusion

In this contribution, the simulation results of modified AMVP scaling process have been reported. From the experimental results, it has been confirmed that the coding loss relative to anchor is less than 0.2% in average with almost the same encoding and decoding time as that of the anchor. We propose to consider this verification result to conclude CE9 in Torino.

References:

- Yu-Wen, et.al. “Description of Core Experiment 9: MV Coding and Skip/Merge Operations,” JCTVC-E709, Mar. 2011.

- Frank Bossen, “Common test conditions and software reference configurations,” JCTVC- E700, Mar. 2011.

- Y.Itani, S.Sekiguchi, “Improvement to AMVP/Merge process,” JCTVC-E064, Mar. 2011.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix: Normative descriptions to be reflected to WD text

Add the following text with yellow marker into section 8.4.2.1.5

##### 8.4.2.1.5 Derivation process for motion vector predictor candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are (with N being replaced by A,or B)

- the motion vectors mvLXN of the neighbouring prediction units,

- the availability flags availableFlagLXN of the neighbouring prediction units.

Figure 8-1 – Spatial motion vector neighbours

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-exisiting.”

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- Let a set of two sample locations be (xAk, yAk), with k = 0 .. 1, specifies sample locations with xAk = xP – 1, yA0 = yP + nPSH and yA1 = yA0 - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

- For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- When the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ] and ListA is set equal to LX.

- When availableFlagLXA is equal to 0, for ( xA0, yA0 ) the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and RefPicOrderCnt( currPic, refIdxLY[ xAk ][ yAk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ] and ListA is set equal to LY.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ], ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ], ListA is set equal to LY.

- When availableFlagLXA is equal to 1, the following applies.

- If RefPicOrderCnt( currPic , refIdxA, ListA ) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ), mvLXA is set equal to mvLXA

- Otherwise, mvLXA is derived as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-1)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-2)

mvLXA = ClipMv( ( DistScaleFactor * mvLXA + 128 ) >> 8 )	(8-3)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxA, ListA ) )	(8-4)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-5)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPSW, xB1 = xB0 MinPuSize , xB2 = xP - MinPuSize and yBk = yP – 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- When the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX and mvLX[ xBk ][ yBk ] is not identical to mvLXA, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ] and ListB is set equal to LX.

- When availableFlagLXB is equal to 0, for ( xB0, yB0 ) the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ](with Y = !X) is equal to 1 and RefPicOrderCnt( currPic , refIdxLY[ xBk ][ yBk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ), availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ] and ListB is set equal to LY.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to LY.

- If availableFlagLXB is equal to 1 and RefPicOrderCnt( currPic , refIdxB, ListB ) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ), mvLXB is set equal to mvLXB.

- Otherwise if availableFlagLXB is equal to 1 and RefPicOrderCnt( currPic , refIdxB, ListB ) is not equal to RefPicOrderCnt( currPic, refIdxLX, LX ),, mvLXB is derived as specified below.

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-6)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-7)

mvLXB = ClipMv( ( DistScaleFactor * mvLXB + 128 ) >> 8 )	(8-8)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxB, ListB ) )	(8-9)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-10)

- When availableFlagLXB is equal to 1 and mvLXB is identical to mvLXA, availableFlagLXB is set equal to 0.