| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F169 |
| --- | --- |

| Title: | CE9: Temporal vector restriction for small PUs (SP07/SP09/SP12) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yusuke Itani Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Itani.Yusuke@dx.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution reports performance verification results on temporal vector restriction for small PUs proposed in JCTVC-E064 [3] and E097 [4] according to the description of CE9 [1] at the last Geneva meeting. There are 6 variants in this evaluation consisting of three tests on PU size threshold ranging among 4x4(SP07), 8x4,4x8(SP08), and 8x8(SP09), and another three tests on no limitation on the use of temporal MVP in motion estimation process(SP10-SP12). In this contribution, results of SP07/SP09/SP12 are reported. The results show that coding performance is almost the same as that of the anchor for SP07, and in SP09, 0.8% and 0.8% coding loss for HE LC and LC LD and 0.4% and 0.3% coding loss for HE RA and LC RA are observed. For SP12, 0.7% and 0.7% coding loss for HE LC and LC LD and 0.3% and 0.3% coding loss for HE RA and LC RA are confirmed.

# Introduction

Performance verification on CE9 approved at the last Geneva meeting [1] was conducted..We report simulation results on temporal vector restriction for small PUs proposed in CTVC-E064 [3] and E097 [4] This work was conducted using modified HM3.0 software.

# Simulation results

We modified HM-3.0 by limiting the size of temporal vector restriction according to Table 1. ○ means temporal MVPs are used both in candidate lists and in ME. △ means temporal MVPs are used only in ME. × means temporal MVPs are NOT used in candidate lists or ME. We evaluated SP07/S09/S12. In average, the results show that coding performance is almost the same as that of the anchor for SP07, and in SP09, 0.8% and 0.8% coding loss for HE LC and LC LD and 0.4% and 0.3% coding loss for HE RA and LC RA are observed. For SP12, 0.7% and 0.7% coding loss for HE LC and LC LD and 0.3% and 0.3% coding loss for HE RA and LC RA are confirmed.

Table1: simulation condition

|  | 4x4 | 8x4/4x8 | 8x8 | 16x8/8x16 | 16x16 | 32x16/16x32 | 32x32 | 64x32/32x64 | 64x64 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SP07 | × | ○ | ○ | ○ | ○ | ○ | ○ | ○ | ○ |
| SP08 | × | × | ○ | ○ | ○ | ○ | ○ | ○ | ○ |
| SP09 | × | × | × | ○ | ○ | ○ | ○ | ○ | ○ |
| SP10 | △ | ○ | ○ | ○ | ○ | ○ | ○ | ○ | ○ |
| SP11 | △ | △ | ○ | ○ | ○ | ○ | ○ | ○ | ○ |
| SP12 | △ | △ | △ | ○ | ○ | ○ | ○ | ○ | ○ |

The simulation results are provided in Table 2-4.

Table 2. SP07

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | -0.1 | 0.0 | -0.1 | -0.1 |
| Class C | 0.0 | -0.1 | 0.1 | 0.0 | -0.1 | -0.1 |
| Class D | -0.1 | -0.2 | 0.4 | 0.0 | -0.1 | -0.3 |
| Class E | -0.1 | -0.2 | -0.2 | 0.0 | -0.5 | 0.6 |
| Overall | 0.0 | -0.1 | 0.1 | 0.0 | -0.2 | 0.0 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3. SP09

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.4 | 0.4 | 0.7 | 0.3 | 0.3 | 0.2 |
| Class B | 0.2 | 0.2 | 0.1 | 0.0 | 0.0 | 0.1 |
| Class C | 0.4 | 0.3 | 0.4 | 0.3 | 0.4 | 0.5 |
| Class D | 0.5 | 0.5 | 0.5 | 0.5 | 0.4 | 0.4 |
| Class E |  |  |  |  |  |  |
| Overall | 0.4 | 0.3 | 0.4 | 0.3 | 0.3 | 0.3 |
| Enc Time[%] | 97% |  |  | 96% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.5 | 0.3 | 0.0 | 0.3 | 0.3 | 0.2 |
| Class C | 1.0 | 0.8 | 0.8 | 1.0 | 1.1 | 1.0 |
| Class D | 0.9 | 0.2 | 1.3 | 1.0 | 0.6 | 0.8 |
| Class E | 0.7 | 0.5 | 0.6 | 0.5 | 0.4 | 0.6 |
| Overall | 0.8 | 0.4 | 0.6 | 0.7 | 0.6 | 0.6 |
| Enc Time[%] | 97% |  |  | 96% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

Table 4. SP12

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.4 | 0.5 | 0.6 | 0.4 | 0.3 | 0.2 |
| Class B | 0.2 | 0.2 | 0.2 | 0.0 | 0.0 | 0.0 |
| Class C | 0.3 | 0.4 | 0.5 | 0.3 | 0.3 | 0.4 |
| Class D | 0.5 | 0.4 | 0.4 | 0.4 | 0.4 | 0.3 |
| Class E |  |  |  |  |  |  |
| Overall | 0.3 | 0.4 | 0.4 | 0.3 | 0.2 | 0.2 |
| Enc Time[%] | 103% |  |  | 104% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.5 | 0.2 | -0.1 | 0.3 | 0.3 | 0.2 |
| Class C | 0.9 | 0.8 | 0.8 | 0.9 | 0.8 | 1.0 |
| Class D | 0.8 | 0.5 | 1.5 | 0.9 | 0.7 | 0.8 |
| Class E | 0.7 | 0.7 | 0.4 | 0.4 | 0.5 | 1.0 |
| Overall | 0.7 | 0.5 | 0.6 | 0.6 | 0.6 | 0.7 |
| Enc Time[%] | 104% |  |  | 106% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

# Conclusion

Temporal vector restriction for small PUs was evaluated under the common test condition specified in CE9 .According to the test results, temporal vector restriction only for 4x4 PUs does not cause performance degradation, while reducing normative complexity on MV scaling required for temporal MVP generation. We recommend adoption of temporal vector restriction only for 4x4 PUs.

References:

- Yu-Wen, et.al. “Description of Core Experiment 9: MV Coding and Skip/Merge Operations,” JCTVC-E709, Mar. 2011.

- Frank Bossen, “Common test conditions and software reference configurations,” JCTVC- E700, Mar. 2011.

- Y.Itani, S.Sekiguchi, “Improvement to AMVP/Merge process,” JCTVC-E064, Mar. 2011.

- H.Takahara, S.Fukushima, “Temporal predictor restriction harmonized with motion vector compression,” JCTVC-E097, Mar.2011

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix: Normative descriptions to be reflected to WD text

(A) Add the following text with yellow marker into section 8.4.2.1.1.

##### 8.4.2.1.1 Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the motion vectors mvL0N and mvL1N, the reference indices refIdxL0N and refIdxL1N and the prediction list utilization flags predFlagL0N and predFlagL1N with N being replaced by A, B, C or D.

- The derivation process of reference indices for temporal merging candidate in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), nPSW, nPSH as the inputs and the output is directly assigned to refIdxLX.

- If both nPSW and nPSH are equal to 4, availableFlagCol is set equal to 0. Otherwise, The derivation process for temporal luma motion vector prediction in subclause 8.4.2.1.6 is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col	(8-1)

predFlagLXCol = availableFlagLXCol	(8-2)

- The merging candidate list, mergeCandList, is constructed of which elements are given as specified order:

  - A, if availableFlagA is equal to 1

  - B, if availableFlagB is equal to 1

  - Col, if availableFlagCol is equal to 1

  - C, if availableFlagC is equal to 1

  - D, if availableFlagD is equal to 1

- If several merging candidates have the motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.

- If the number of elements NumMergeCand within the mergeCandList is equal to 1, mergeIdx is set equal to 0, otherwise, mergeIdx is set equal to merge_idx[ xP][ yP ].

- The following assignments are made with N being the candidate at position mergeIdx in the merging candidate list mergeCandList ( N = mergeCandList[ mergeIdx ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]	(8-3)

mvLX[ 1 ] = mvLXN[ 1 ]	(8-4)

refIdxLX = refIdxLXN	(8-5)

predFlagLX = predFlagLXN	(8-6)

- If all availability flags availableFlagN (with N being replaced by A, B, Col, C, or D) are equal to 0, mergeIdx is set equal to 0 and the variables mvLX, refIdxLX and predFlagLX (with X being replaced by 0 or 1) are inferred as follows.

If slice_type is equal to P, the following applies.

mvLX[ 0 ] = 0	(8-7)

mvLX[ 1 ] = 0	(8-8)

refIdxL0 = 0		(8-9)

refIdxL1 = -1	(8-10)

predFlagL0 = 1	(8-11)

prefFlagL1 = 0	(8-12)

Otherwise ( slice_type is equal to B ), the following applies.

mvLX[ 0 ] = 0	(8-13)

mvLX[ 1 ] = 0	(8-14)

refIdxL0 = 0		(8-15)

refIdxL1 = 0		(8-16)

predFlagL0 = 1	(8-17)

prefFlagL1 = 1	(8-18)

(B) Add the following text with yellow marker into section 8.4.2.1.4.

##### 4 Derivation process for luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Output of this process is

- the prediction mvpLX of the motion vector mvLX (with X being 0 or 1).

The motion vector predictor mvpLX is derived in the following ordered steps.

- The derivation process for motion vector predictor candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as inputs and the availability flags availableFlagLXN and the motion vectors mvLXN with N being replaced by A, B as the output.

- When one of the width and the height of the prediction unit nPSW and nPSH is larger than 4, the If both nPSW and nPSH are equal to 4, availableFlagLXCol is set equal to 0. Otherwise, the derivation process for temporal luma motion vector prediction in subclause 8.4.2.1.6 is invoked with luma location ( xP, yP ) , the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector predictor mvLXCol.

- The motion vector predictor list, mvpListLX, is constructed of which elements are given as specified order:

- mvLXCol, if availableFlagLXCol is equal to 1

- mvLXA, if availableFlagLXA is equal to 1

- mvLXB, if availableFlagLXB is equal to 1

- If several motion vectors have the same value, the motion vectors are removed from the list except the motion vector which has the smallest order in the mvpListLX.

- If the number of elements NumMVPCand( LX ) within the mvpListLX is equal to 1, mvpIdx is set equal to 0, otherwise, mvpIdx is set equal to mvp_idx_lX[ xP, yP ].

- The motion vector of mvListLX[ mvpIdx ] is assigned to mvpLX.