| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F170 |
| --- | --- |

| Title: | CE8 Subtest2: Cross-verification on Sony's Parametric Adaptive Loop Filter (JCTVC-F285) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Akira Minezawa Kazuo Sugimoto Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results on CE8 subtest2 that evaluates a low-complexity ALF design proposed in JCTVC-E320. The cross-verification work was performed using software that implements E320 technique into HM3.0 code base provided by the E320 proponent. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. It has been confirmed that objective R-D performance produced in this work perfectly matches with CE report from the proponent. Similar tendency on encoding and decoding time was also observed.

# Introduction

Performance verification on CE8[1] subtest2 approved at the last Geneva meeting was conducted. The verified method is Parametric ALF (PALF) technique proposed by Sony in JCTVC-E320[2]. This work was conducted using modified HM3.0 software provided by the proponent.

The proposed scheme is a combination of QC_ALF and PALF, and the ALF mode for each CU is decided using the following procedures:

- Design 5 tap filters for QC_ALF

- CU-based RD-decision between QC_ALF, PALF and non-filtering

- If QC_ALF is selected, re-design 5 tap filters using the samples included in CUs for QC_ALF mode.

- Design 7 tap and 9 tap filters for QC_ALF

- Final number of filter tap for QC_ALF is decided using RD-decision

By doing this, encoding complexity is reduced even though PALF is added to the conventional QC_ALF.

In the PALF mode, 32 different pre-designed filters composed of 8 directions and 4 levels of strength are used. The directions and filter taps are shown as below:

# Simulation results

Table 1 shows test conditions used in our experiment, which is to test configuration defined in the CE8 document [1]. We built executables from source code provided by Sony and ran it onto 64bit Linux operating system (more specifically Fedora 10 x86_64). Encoding and decoding time for both anchor and the test case were measured using “time” command in the Linux platform.

Table 1  Test Condition

| Codec software | Implementation of the tool proposed in JCTVC- E320[2] based on HM3.0 software |
| --- | --- |
| Test configuration | E700[3] |
| QP setting | QPI=22,37,32,37 |

The objective performance data and simulation time obtained in this work are shown in Table 2 to 4. It has been confirmed that BD rate reduction results perfectly match the results of JCTVC-F285 reported from the proponent [3]. Similar tendency on encoding and decoding time was also observed.

Table 2 Simulation results for All Intra Condition

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | -0.6 | -1.0 | -2.5 | -0.6 | -1.0 |
| Class B | -0.1 | -0.5 | -1.0 | -1.3 | -1.3 | -1.5 |
| Class C | -0.3 | -0.2 | -0.6 | -1.4 | -2.5 | -3.4 |
| Class D | -0.2 | 1.0 | 1.0 | -0.7 | -1.8 | -2.1 |
| Class E | 0.1 | -0.1 | -0.2 | -2.1 | -0.2 | -0.9 |
| Overall | -0.1 | -0.1 | -0.4 | -1.6 | -1.3 | -1.8 |
| Enc Time[%] | 91% |  |  | 114% |  |  |
| Dec Time[%] | 102% |  |  | 128% |  |  |

Table 3 Simulation results for Random Access Condition

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2 | -1.5 | -2.1 | -4.2 | -2.9 | -3.0 |
| Class B | -0.4 | -1.6 | -1.9 | -2.7 | -1.7 | -1.2 |
| Class C | -0.5 | -1.3 | -1.2 | -2.6 | -1.2 | -2.0 |
| Class D | -0.6 | 0.0 | -0.2 | -2.9 | -0.2 | -0.1 |
| Class E |  |  |  |  |  |  |
| Overall | -0.4 | -1.1 | -1.4 | -3.1 | -1.5 | -1.6 |
| Enc Time[%] | 96% |  |  | 104% |  |  |
| Dec Time[%] | 101% |  |  | 117% |  |  |

Table 4 Simulation results for Low Delay Condition

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.7 | -2.7 | -3.4 | -3.6 | -3.5 | -3.4 |
| Class C | -1.0 | -3.0 | -3.7 | -3.6 | -3.0 | -4.5 |
| Class D | -0.9 | -0.6 | -0.4 | -2.8 | -1.0 | -0.9 |
| Class E | -1.2 | -2.4 | -4.1 | -5.5 | -6.4 | -6.5 |
| Overall | -0.9 | -2.2 | -2.8 | -3.7 | -3.3 | -3.6 |
| Enc Time[%] | 98% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 118% |  |  |

# Conclusion

This contribution reports cross-verification results on CE8 subtest2 that evaluates a low-complexity ALF design proposed in JCTVC-E320. It has been confirmed that all the results perfectly match the results from the proponent. Similar tendency on encoding and decoding time was also observed.

# References

- Tomoo Yamakage, et al., “Description of CE8: Non-deblocking loop filtering”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E708, 5th Meeting: Geneva, CH, March 2011.

- Ehsan Maani, et al., “Parametric Adaptive Loop Filter”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E320, 5th Meeting: Geneva, CH, March 2011.

- Ehsan Maani, et al., “CE8: Parametric Adaptive Loop Filter”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F285, 6th Meeting: Torino, IT, July 2011.