| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F172 |
| --- | --- |

| Title: | An improved intra vertical and horizontal prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akira Minezawa Kazuo Sugimoto Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

In this contribution, a modification to intra vertical and horizontal prediction is proposed. In the proposed scheme, difference between two reference samples in left or above prediction direction is scaled and added to corresponding prediction sample in intra vertical and horizontal prediction modes. The proposed scheme is implemented onto HM-3.0 software, and simulations are conducted using common test configurations [1] to evaluate the performance of the proposed scheme. It is reportedly confirmed that the proposed scheme achieves 0.3% BD-rate reduction on average for high efficiency and low complexity intra configuration respectively.

# Introduction

In HM-3, AVC-like intra vertical and horizontal prediction is adopted [2]. These predictions work well for the blocks those contain stable or clearly linear edges, but produce somewhat big prediction errors for the blocks having non-stable or blurred edges.

To improve prediction efficiency in these modes, we propose to add scaled difference between two reference samples (upper-left and conventionally used one) to corresponding prediction sample in left or above prediction direction.

# Proposed scheme

We propose to replace the derivation process for intra vertical and horizontal prediction samples adopted in WD-3 to the following.

Vertical prediction:

S'(x,y)=S(x,−1)+(S(−1,y)−S(−1,−1))2x+1                  (1)

Horizontal prediction:

S'(x,y)=S(−1,y)+(S(x,−1)−S(−1,−1))2y+1                  (2)

where (x,y) indicates the position of prediction sample in a luma prediction block as shown in Figure 1, S'(x,y) indicates the sample to be predicted and S(x,y) indicates the reference sample to be used for conventional prediction.

Figure 1.  The coordinate for vertical and horizontal prediction

We also tested a simplified version of the proposed method as specified below.

Vertical prediction:

S'(x,y)=S(x,−1)+(S(−1,y)−S(−1,−1))2 (x=0)S(x,−1) (x≥1)                  (3)

Horizontal prediction:

S'(x,y)=S(−1,y)+(S(x,−1)−S(−1,−1))2 (y=0)S(−1,y) (y≥1)                  (4)

Second equation in equation 3 and 4 indicates AVC-like conventional intra vertical and horizontal prediction, respectively. Therefore, the proposed prediction process modifies prediction samples in only one column or one line compared to HM-3.

# Simulations & Results

The proposed scheme has been implemented on top of the HM-3.0 software for luma component, and the simulations have been conducted using the common test configurations [1]. Table 1 shows the results with a configuration that applies the proposed scheme to 4x4 and 8x8 luma prediction blocks.

Table 1. BD Bitrate savings of the proposed scheme (application to 4x4 and 8x8)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4 | -0.3 | -0.2 | -0.3 | -0.3 | -0.3 |
| Class B | -0.3 | -0.2 | -0.1 | -0.3 | -0.2 | -0.2 |
| Class C | -0.3 | -0.3 | -0.2 | -0.3 | -0.3 | -0.3 |
| Class D | -0.2 | -0.3 | -0.3 | -0.3 | -0.2 | -0.2 |
| Class E | -0.4 | -0.3 | -0.4 | -0.4 | -0.2 | -0.4 |
| Overall | -0.3 | -0.3 | -0.2 | -0.3 | -0.3 | -0.3 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 2 shows the performance of the proposed scheme when configuring to apply it to 4x4, 8x8 and 16x16.

Table 2. BD Bitrate savings of the proposed scheme (application to 4x4, 8x8 and 16x16)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4 | -0.4 | -0.4 | -0.4 | -0.5 | -0.5 |
| Class B | -0.3 | -0.3 | -0.2 | -0.3 | -0.3 | -0.3 |
| Class C | -0.3 | -0.3 | -0.3 | -0.3 | -0.4 | -0.4 |
| Class D | -0.3 | -0.3 | -0.4 | -0.3 | -0.3 | -0.3 |
| Class E | -0.4 | -0.5 | -0.5 | -0.4 | -0.4 | -0.5 |
| Overall | -0.3 | -0.4 | -0.4 | -0.3 | -0.4 | -0.4 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

We also verified the performance of the simplified version. The results for the case when it is applied to 4x4 and 8x8 and the case when it is additionally applied to 16x16 are shown in Table 3 and 4, respectively.

Table 3. BD Bitrate savings of the simplified version of the proposal (application to 4x4 and 8x8)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3 | -0.2 | -0.1 | -0.2 | -0.2 | -0.2 |
| Class B | -0.2 | -0.1 | -0.1 | -0.3 | -0.2 | -0.2 |
| Class C | -0.3 | -0.3 | -0.2 | -0.3 | -0.3 | -0.3 |
| Class D | -0.3 | -0.2 | -0.3 | -0.3 | -0.2 | -0.2 |
| Class E | -0.4 | -0.3 | -0.3 | -0.4 | -0.2 | -0.3 |
| Overall | -0.3 | -0.2 | -0.2 | -0.3 | -0.2 | -0.2 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 4. BD Bitrate savings of the simplified version of the proposal (application to 4x4, 8x8 and 16x16)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3 | -0.2 | -0.2 | -0.3 | -0.3 | -0.3 |
| Class B | -0.2 | -0.2 | -0.2 | -0.3 | -0.2 | -0.2 |
| Class C | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 |
| Class D | -0.3 | -0.3 | -0.3 | -0.3 | -0.2 | -0.2 |
| Class E | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 | -0.4 |
| Overall | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Conclusion

In this contribution, a modified intra vertical and horizontal prediction scheme has been proposed. It has been confirmed that the proposed scheme achieves 0.3% BD-rate reduction on average for both high efficiency and low complexity intra configurations. It has also been observed that the encoding and decoding time do not essentially increase relative to HM-3.0 default setting. We propose this intra vertical and horizontal prediction scheme to be adopted to HM-4.

# Reference

- Frank Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E700, 5th Meeting: Geneva, CH, March 2011.

- “WD3: Working Draft 3 of High-Efficiency Video Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E603, 5th Meeting: Geneva, CH, March 2011.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix: Normative descriptions to be reflected to WD text

A description of the proposal is shown as follows.

##### 8.3.3.1.3  Specification of Intra_Vertical prediction mode

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction size,

–	a variable cIdx specifying the chroma component of the current block.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is equal to 0.

- If cIdx is equal to 0 and nS is smaller than a pre-defined value LOWER_LIMIT_OF_DISABLE_BL_SIZE, the prediction samples predSamples[ x, y ] are derived as

predSamples[ x, y ] = clip( p[ x, -1 ] + ( p[ -1, y ] - p[ -1, -1 ] ) / 2x+1 ), with x, y = 0..nS-1	(8-1)

- Otherwise, the prediction samples predSamples[ x, y ] are derived as

predSamples[ x, y ] = p[ x, -1 ], with x, y = 0..nS-1		(8-2)

##### 8.3.3.1.4  Specification of Intra_Horizontal prediction mode

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

- a variable nS specifying the prediction size,

–	a variable cIdx specifying the chroma component of the current block.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is equal to 1.

- If cIdx is equal to 0 and nS is smaller than a pre-defined value LOWER_LIMIT_OF_DISABLE_BL_SIZE, the prediction samples predSamples[ x, y ] are derived as

predSamples[ x, y ] = clip( p[ -1, y ] + ( p[ x, -1 ] - p [ -1, -1 ] ) / 2y+1 ), with x, y = 0..nS-1	(8-3)

- Otherwise, the prediction samples predSamples[ x, y ] are derived as

predSamples[ x, y ] = p[ -1, y ], with x, y = 0..nS-1		(8-4)

A description of the simplified proposal is shown as follows.

##### 8.3.3.1.3  Specification of Intra_Vertical prediction mode

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

- a variable nS specifying the prediction size.

–	a variable cIdx specifying the chroma component of the current block.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is equal to 0.

- If cIdx is equal to 0 and nS is smaller than a pre-defined value LOWER_LIMIT_OF_DISABLE_BL_SIZE, the prediction samples predSamples[ x, y ] are derived as

predSamples[ x, y ] = p[ x, -1 ], with x = 1..nS-1, y = 0..nS-1	(8-5)

predSamples[ x, y ] = clip( p[ x, -1 ] + (p[ -1, y ] - p[ -1, -1 ] ) / 2 ) , with x = 0, y = 0..nS-1	(8-6)

- Otherwise, the prediction samples predSamples[ x, y ] are derived as

predSamples[ x, y ] = p[ x, -1 ], with x, y = 0..nS-1		(8-7)

##### 8.3.3.1.4  Specification of Intra_Horizontal prediction mode

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

- a variable nS specifying the prediction size.

–	a variable cIdx specifying the chroma component of the current block.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is equal to 1.

- If cIdx is equal to 0 and nS is smaller than a pre-defined value LOWER_LIMIT_OF_DISABLE_BL_SIZE, the prediction samples predSamples[ x, y ] are derived as

predSamples[ x, y ] = p[ -1, y ], with x = 0..nS-1, y = 1..nS-1	(8-8)

predSamples[ x, y ] = clip( p[ -1, y ] + ( p[ x, -1 ] - p [ -1, -1 ] ) / 2 ), with x = 0..nS-1, y = 0	(8-9)

- Otherwise, the prediction samples predSamples[ x, y ] are derived as

predSamples[ x, y ] = p[ -1, y ], with x, y = 0..nS-1		(8-10)