| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F174 |
| --- | --- |

| Title: | Signaling of Max and Min QP in slice |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution proposes signaling of maximum and minimum QP values (MaxQP, MinQP) in a slice at the slice header. By signaling these two values, the range of delta QP (dQPrange) within a slice is also specified. A modified indexing of delta QP according to MaxQP, MinQP and dQPrange is used with a modified binarization table of CABAC or a modified VLC table of CAVLC. The proposed scheme is implemented onto modified version of HM-3.0 software used as the base software for CE4, and simulations are conducted using common test configurations of CE4[1] with MaxCuDQPDepth=3 to evaluate the performance of the proposed scheme. It is reportedly confirmed that the proposed scheme achieves delta QP bit reduction of 2.6, 3.5 and 4.4% for AI-HE/RA-HE/LDB-HE configurations and 8.2%, 10.3% and 11.0% for AI-LC/RA-LC/LDB-LC respectively.

# Introduction

In HM-3, CU level delta QP syntax is adopted [2]. Several methods of predicting QP value have been proposed, and those proposals have been evaluated in CE4, while keeping the entropy coding for delta QP as it was. In this contribution we propose to signal maximum and minimum QP values (MaxQP, MinQP) in a slice at the slice header to improve the coding efficiency of delta QP syntax. The proposed scheme can be applied with any prediction schemes for CU level QP.

# Proposed scheme

The encoding process of the proposed scheme is composed of 3 steps as follows:

- Signaling the MaxQP and the MinQP at the slice header

- Delta QP indexing

- Entropy coding using a modified binarization table for CABAC or modified VLC table for CAVLC

## Signaling of MaxQP and the MinQP

The maximum QP value and minimum QP value within a slice is detected and signaled at the slice header as the difference from the slice QP value, which is also signaled at the slice header.

## Delta QP indexing

Delta QP (dQP) is the difference between the current QP (curQP) and the predicted QP (predQP). As MaxQP and MinQP are signaled, the possible dQP values are limited from MinQP-predQP to MaxQP-predQP. In this contribution, the index of dQP is specified so that the smaller index number is assigned to the smaller absolute value of dQP. For example, if MaxQP=32, MinQP=24 and predQP=30, the indexing is done as follows:

| Index | dQP value |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | -1 |
| 3 | 2 |
| 4 | -2 |
| 5 | -3 |
| 6 | -4 |
| 7 | -5 |
| 8 | -6 |

## Entropy coding of dQP

### CABAC

Binarization of dQP index specified in the previous section is performed using a truncated unary binarization with maximum value = dQPrange instead of unary binarization.

### CAVLC

Concatenated Truncated Golomb code (CTG code) is used to code the dQP index. Truncated Golomb code is a modified version of the Golomb code where maximum length of the code is specified according to the maximum index value, where the last code of the prefix is removed from the code with the maximum length.

| index | Golomb code | Truncated Golomn code (max length=2) | Truncated Golomn code (max length=4) | Truncated Golomn code (max length=6) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 010 | 0 0 | 010 | 010 |
| 2 | 011 | 0 1 | 011 | 011 |
| 3 | 00100 |  | 00 00 | 00100 |
| 4 | 00101 |  | 00 01 | 00101 |
| 5 | 00110 |  | 00 10 | 00110 |
| 6 | 00111 |  | 00 11 | 00111 |
| 7 | 0001000 |  |  | 000 000 |
| 8 | 0001001 |  |  | 000 001 |
| 9 | 0001010 |  |  | 000 010 |
| 10 | 0001011 |  |  | 000 011 |
| 11 | 0001100 |  |  | 000 100 |
| 12 | 0001101 |  |  | 000 101 |
| 13 | 0001110 |  |  | 000 110 |
| 14 | 0001111 |  |  | 000 111 |
| 15 | 000010000 |  |  |  |
| 16 | 000010001 |  |  |  |
| 17 | : |  |  |  |

CTG code is a series of Truncated Golomb codes to compose a VLC table with the number of index equal to dQPrange. Truncated Golomb codes with maxmum number of index less than the rest of the number of index are concatenated repeatedly until no index is left.

Some examples of the Concatenated Truncated Golomb code are shown below:

| index | Golomb code | CTG code (Max=2) | CTG code (Max=4) | CTG code (Max=12) | CTG code (Max=18) |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 1 | 1 |
| 1 | 010 | 0 0 | 00 | 010 | 010 |
| 2 | 011 | 0 1 | 01 1 | 011 | 011 |
| 3 | 00100 |  | 01 00 | 0000 | 00100 |
| 4 | 00101 |  | 01 01 | 0001 | 00101 |
| 5 | 00110 |  |  | 0010 | 00110 |
| 6 | 00111 |  |  | 0011 1 | 00111 |
| 7 | 0001000 |  |  | 0011 010 | 000000 |
| 8 | 0001001 |  |  | 0011 011 | 000001 |
| 9 | 0001010 |  |  | 0011 0000 | 000010 |
| 10 | 0001011 |  |  | 0011 0001 | 000011 |
| 11 | 0001100 |  |  | 0011 0010 | 000100 |
| 12 | 0001101 |  |  | 0011 0011 | 000101 |
| 13 | 0001110 |  |  |  | 000110 |
| 14 | 0001111 |  |  |  | 000111 1 |
| 15 | 000010000 |  |  |  | 000111 00 |
| 16 | 000010001 |  |  |  | 000111 01 1 |
| 17 | 000010010 |  |  |  | 000111 01 00 |
| 18 | 000010011 |  |  |  | 000111 01 01 |
| 19 | 000010100 |  |  |  |  |
| : |  |  |  |  |  |

# Simulations & Results

The proposed scheme has been implemented on top of the modified HM-3.0 software for CE4, and the simulations have been conducted using the common test configurations for CE4. Table 1 shows the results with a configuration that applies the proposed scheme.

Table 1. Bitrate savings of the proposed scheme

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.1 | -0.1 | -0.1 | -2.7% | -0.2 | -0.2 | -0.2 | -8.4% |
| Class B | 0.0 | -0.1 | 0.0 | -1.5% | -0.2 | -0.2 | -0.2 | -6.8% |
| Class C | -0.1 | -0.1 | -0.1 | -2.3% | -0.3 | -0.3 | -0.3 | -8.2% |
| Class D | -0.1 | -0.1 | -0.1 | -3.9% | -0.3 | -0.3 | -0.3 | -10.3% |
| Class E | -0.1 | -0.2 | -0.2 | -2.9% | -0.3 | -0.3 | -0.3 | -7.7% |
| All | -0.1 | -0.1 | -0.1 | -2.6% | -0.3 | -0.3 | -0.3 | -8.2% |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.2 | -0.2 | 0.3 | -3.3% | -0.3 | -0.3 | -0.3 | -9.8% |
| Class B | -0.1 | 0.0 | -0.1 | -2.4% | -0.3 | -0.3 | -0.3 | -8.7% |
| Class C | -0.1 | 0.1 | 0.0 | -3.6% | -0.4 | -0.4 | -0.4 | -10.6% |
| Class D | 0.1 | 0.1 | 0.3 | -5.0% | -0.3 | -0.3 | -0.3 | -12.7% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.1 | 0.0 | 0.1 | -3.5% | -0.3 | -0.3 | -0.3 | -10.3% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1 | -0.2 | -0.3 | -2.7% | -0.4 | -0.4 | -0.4 | -9.5% |
| Class C | -0.1 | -0.1 | -0.2 | -4.2% | -0.5 | -0.5 | -0.5 | -11.3% |
| Class D | 0.0 | -0.2 | 0.0 | -5.5% | -0.4 | -0.4 | -0.4 | -13.4% |
| Class E | -0.1 | 0.6 | 0.2 | -6.7% | -0.5 | -0.5 | -0.5 | -13.0% |
| All | -0.1 | 0.0 | -0.1 | -4.5% | -0.4 | -0.4 | -0.4 | -11.6% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |

# Conclusion

In this contribution, a modified intra vertical and horizontal prediction scheme has been proposed. It is confirmed that the proposed scheme achieves delta QP bit reduction of 2.6, 3.5 and 4.4% for AI-HE/RA-HE/LDB-HE configurations and 8.2%, 10.3% and 11.0% for AI-LC/RA-LC/LDB-LC respectively. We propose this scheme to be adopted in HM-4.

# Reference

- K.Sato, M.Budagavi, M.Coban, H.Aoki and X.Li, “Description of Core Experiment 4: Quantization”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E704, 5th Meeting: Geneva, CH, March 2011.

- “WD3: Working Draft 3 of High-Efficiency Video Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E603, 5th Meeting: Geneva, CH, March 2011.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix: Normative descriptions to be reflected to WD text