| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F180 |
| --- | --- |

| Title: | Cross-check of HHI's proposal (JCTVC-F268) on Unified PIPE-based Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Akira Minezawa Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution reports performance cross-check results of HHI’s proposal on a unified solution of entropy coders for high efficiency and low complexity settings using PIPE-based coder[1]. The verification tests have been conducted using common test conditions[2]. It is confirmed that the results perfectly match with the ones reported by the proponent.

# Verified scheme

The verified proposal contains two operation modes: Low complexity PIPE (LC-PIPE) and High Efficiency PIPE (HE-PIPE).

LC-PIPE is operating under the following constraints:

- All probability models are non-adaptive, i.e., all models will be initialized at the beginning of each slice with appropriate model probabilities (depending on the choice of slice type and slice QP) and will be kept fixed throughout processing of the slice.

- All context-based selection processes for deriving conditional probabilities based on prior encoded/decoded symbols are completely removed.

- Only 8 different model probabilities corresponding to 8 different PIPE/V2V codes are supported.

- Some specific syntax elements for residual coding, like significant_coeff_flag and coeff_abs_level_greaterX_flag (with X=1,2), are using multiple (fixed) probability models such that the selection simply depends on the number of prior encoded/decoded bins.

HE-PIPE is conceptually similar to CABAC with the following differences:

- Binary arithmetic coding (BAC) is replaced by PIPE coding (same as in the LC-PIPE case).

- Each probability model, i.e., each ctxIdx is represented by a valMps, pipeIdx and a refineIdx, where pipeIdx with values in the range from 0...7 represents the model probability of the 8 different PIPE/V2V codes. This change affects only the internal representation of states, not the behavior of the state machine (i.e., probability estimation) itself.

- Initialization of probability models use 8 bit initialization values as stated above.

- Backward scanning of syntax elements coeff_abs_level_greaterX_flag (with X = 1, 2), coeff_abs_level_minus3, and coeff_sign_flag is performed along the same scanning path as the forward scan. Context derivation for coding of coeff_abs_level_greaterX_flag (with X = 1, 2) is also simplified.

# Simulation Results

This cross-checking was conducted using modified HM3.2 software provided by the proponent. It is confirmed that the results presented in this section perfectly match the results provided by the proponent.

Table 1. BD-rate performance against the anchor

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.5 | -0.2 | -0.4 | 1.5 | 9.7 | 9.6 |
| Class B | 0.3 | -0.5 | -0.6 | 0.0 | 8.9 | 9.1 |
| Class C | 0.2 | -0.4 | -0.5 | -0.6 | 6.9 | 7.1 |
| Class D | 0.1 | -0.3 | -0.3 | -0.1 | 7.1 | 6.9 |
| Class E | 0.5 | -0.7 | -0.7 | -0.8 | 11.8 | 11.0 |
| Overall | 0.3 | -0.4 | -0.5 | 0.0 | 8.7 | 8.6 |
| Enc Time[%] | 105% |  |  | 95% |  |  |
| Dec Time[%] | 97% |  |  | 95% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.6 | 0.0 | 0.6 | 1.1 | 20.6 | 17.9 |
| Class B | 0.3 | -0.5 | -0.5 | -0.2 | 16.1 | 14.8 |
| Class C | 0.2 | -0.4 | -0.5 | 0.4 | 11.6 | 12.0 |
| Class D | 0.1 | -0.3 | -0.6 | 0.8 | 12.3 | 11.7 |
| Class E |  |  |  |  |  |  |
| Overall | 0.3 | -0.3 | -0.3 | 0.5 | 15.2 | 14.1 |
| Enc Time[%] | 111% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 95% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.4 | -0.7 | -0.8 | 0.8 | 18.0 | 19.3 |
| Class C | 0.3 | -0.6 | -1.1 | 0.5 | 13.3 | 14.7 |
| Class D | 0.0 | -1.7 | -1.7 | 0.8 | 17.6 | 17.7 |
| Class E | -0.2 | -2.4 | -1.9 | 3.0 | 15.1 | 13.0 |
| Overall | 0.2 | -1.2 | -1.3 | 1.2 | 16.2 | 16.6 |
| Enc Time[%] | 110% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 93% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.5 | -0.4 | -0.4 | 0.6 | 19.8 | 20.3 |
| Class C | 0.5 | -0.2 | -0.1 | 0.6 | 14.5 | 15.9 |
| Class D | 0.2 | -0.4 | 0.0 | 0.7 | 17.2 | 17.3 |
| Class E | 0.3 | -1.2 | -1.4 | 2.8 | 16.8 | 14.7 |
| Overall | 0.4 | -0.5 | -0.4 | 1.0 | 17.3 | 17.4 |
| Enc Time[%] | 103% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 96% |  |  |

# Conclusion

We confirmed that the implementation of HHI’s proposal on top of HM-3.2 is consistent with the descriptions, and all test results perfectly matched the results provided by the proponent.

References:

- D. Marpe, H. Kirchhoffer, B. Bross, V. George, T. Nguyen, M. Preiß, M. Siekmann, J. Stegemann, T. Wiegand, “Unified PIPE-Based Entropy Coding for HEVC,” JCTVC-F268, Jul. 2011.

- Frank Bossen, “Common test conditions and software reference configurations,” JCTVC- E700, Mar. 2011.