| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F183 |
| --- | --- |

| Title: | Cross-verification report on Fujitsu's proposal (JCTVC-F142) on AMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Akira Minezawa Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution reports performance cross-check results on Fujitsu’s proposal on a modification of AMVP[1]. The verification tests have been conducted using common test conditions[2]. It is confirmed that the results perfectly match with the ones reported by the proponent.

# Verified scheme

The verified proposal contains two modifications on AMVP as follows:

- modification on the position of colPu

In WD3, the colPu is derived as the Right-bottom PU in the colPic. In the case if colPu is coded in a CU with an intra prediction mode or colPu is unavailable, the colPu is derived as the center position of the current prediction unit in the colPic.

The proposed scheme uses four different combinations of candidates PuCandA and PuCandB for colPu depending on the location of the CenterPu within 16x16 CU. Final colPu is decided one of the candidates which gives shorter distance from the CenterPu to the position where the collocated motion vector points.

- modification on the scaling method for mvLXCol

In WD3, mvLXA, mvLXB and mvLXCol are derived as scaling version of the motion vector mvCol as specified below:

mvLXA = ClipMv((DistScaleFactor*mvLXA+128) >> 8)

mvLXB = ClipMv((DistScaleFactor*mvLXB+128) >> 8)

mvLXCol = ClipMv((DistScaleFactor*mvCol+128) >> 8)

In the proposed scheme, they are derived as follows:

mvLXA = ClipMv(Sign(DistScaleFactor*mvLXA)*((Abs(DistScaleFactor*mvLXA)+127) >> 8))

mvLXB = ClipMv(Sign(DistScaleFactor*mvLXB)*((Abs(DistScaleFactor*mvLXB)+127) >> 8))

mvLXCol = ClipMv(Sign(DistScaleFactor*mvCol)*((Abs(DistScaleFactor*mvCol)+127) >> 8))

# Simulation Results

The cross-checking was conducted using modified HM3.0 software provided by the proponent. It is confirmed that the results presented in this section perfectly match the results provided by the proponent. On average, the performance with the technique shows 0.5% and 0.1% BD-rate improvements on Random Access conditions and Low Delay conditions respectively as shown in the table below.

Table 1. BD-rate performance against the anchor

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1 | -0.2 | 0.2 | -0.1 | 0.0 | 0.1 |
| Class B | -0.4 | -0.2 | -0.1 | -0.4 | -0.1 | -0.1 |
| Class C | -0.6 | -0.3 | -0.3 | -0.5 | -0.3 | -0.4 |
| Class D | -0.9 | -0.7 | -0.8 | -0.9 | -0.6 | -0.5 |
| Class E |  |  |  |  |  |  |
| Overall | -0.5 | -0.3 | -0.2 | -0.5 | -0.3 | -0.2 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | 0.0 | 0.2 | 0.0 | -0.1 | -0.4 |
| Class C | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 | -0.2 |
| Class D | -0.1 | -0.1 | 0.6 | 0.0 | 0.0 | -0.1 |
| Class E | -0.1 | 0.3 | -0.5 | -0.3 | -0.6 | -0.8 |
| Overall | -0.1 | 0.0 | 0.1 | -0.1 | -0.2 | -0.4 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

# Conclusion

We confirmed that the implementation of Fujitsu’s proposal on HM-3.0 is consistent with the descriptions, and all test results perfectly matched the results provided by the proponent.

References:

- K. Kazui, S.Shimada, J.Koyama and A.Nakagawa, “Improvement on deviation process for luma motion vector prediction,” JCTVC-F142, Jul. 2011.

- Frank Bossen, “Common test conditions and software reference configurations,” JCTVC- E700, Mar. 2011.