| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F187 WG11 Number: m20606 |
| --- | --- |

| Title: | Comments on Slice Common Information Sharing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ming LI Ping WU | Tel: Email: | +86-25-88014641 li.ming42@zte.com.cn ping.wu@zte.com.cn |
| Source: | ZTE Corporation |  |  |

_____________________________

# Abstract

In this document, a slice common information sharing approach is proposed. A new parameter set, namely “slice_common_info_set”, is employed to carry the ALF parameters among the slices in a picture instead of PPS. With slice_common_info_set, when ALF parameters are changed, the encoder does not need to send a new PPS so that the bits for repeatedly coding common information in PPS are saved.

# Introduction

To facilitate parallel slice decoding, a slice common information sharing approach was proposed in JCTVC-E045 [1] and adopted in HEVC WD document JCTVC-E603 [2]. This method moves the ALF parameters from the header of the first slice in a picture to PPS, and leaves the ALF CU flags in slice headers. Thereby, when a picture is partitioned into multiple slices, the decoder does not need to separately parse the ALF parameters in the first slice header before decoding the other slices, and can process all the slice headers in parallel.

However, if something changes in ALF parameters, the encoder needs to send a new PPS as its implementation in HM3.2rc1. Since the ALF filters are always of different types and coefficients among pictures, the encoder will have to generate PPS for almost every picture filtered by ALF to take full advantage of the filter’s adaptiveness feature to contribute to coding performance improvement, which leads to waste bits for repeatedly coding common syntax elements (e.g. seq_parameter_set_id, entropy_coding_mode_flag, num_temporal_layer_switching_point_flags, num_ref_idx_l0_default_active_minus1, num_ref_idx_l1_default_active_minus1, and etc) in PPS.

To tackle the above mentioned problem, it is necessary to create a new holder to convey common information in the slices in one picture but varying among different pictures. Here, a new parameter set named as “slice_common_info_set” is designed and placed between PPS and slice header. The ALF related syntax elements are moved from PPS to this parameter set to implement common information sharing concept naturally and minimize cost bits for duplicated information.

Similar to PPS, a parameter set ID is assigned to every slice_common_info_set, which is used to identify which set is referred to by the current decoding slice. Additionally, pic_parameter_set_id also exists in slice_common_info_set to indicate which PPS is set active.

With slice_common_info_set, when the reconstructed pictures are processed by the filters adaptively determined according to picture features to suppress the quantization noise, the encoder only needs to generate a new slice_common_info_set for the slices in each picture, while keeping a PPS active for the pictures sharing the same coding parameters. For the applications where the filters are fixed for the pictures within a certain periods, e.g. one GOP, to bring down complexity, only one slice_common_info_set needs to be sent along with the very first picture (in decoding order) in this period, and the encoder simply makes all the related slices refer to this parameter set. Therefore, both adaptive loop filter and (locally) fixed loop filter applications can be well supported, while sharing ALF filter information among slices to facilitate parallel slice decoding.

# Syntax Description

The syntax elements in slice_common_info_set are listed in the table below:

Table 1.  Proposed slice_common_info_set Syntax

| slice_common_info_set_rbsp ( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| slice_common_info_set_id | ue(v) |
| if( adaptive_loop_filter_enabled_flag = = 1 ) |  |
| alf_param( ) |  |
| } |  |

where

pic_parameter_set_id specifies the picture parameter set in use. The value of pic_parameter_set_id shall be in the range of 0 to 255, inclusive.

slice_common_info_set_id identifies the slice common information set that is referred to in the slice header. The value of slice_common_info_set_id shall be in the range of 0 to 1023, inclusive.

To implement the proposed parameter set on JCTVC-E603-v8, some modifications on PPS and slice header are made as shown in Table 2 and 3.

Table 2.  Modified PPS Syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| shared_pps_info_enabled_flag | u(1) |
| if( shared_pps_info_enabled_flag ) |  |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_param( ) |  |
| if( cu_qp_delta_enabled_flag ) |  |
| max_cu_qp_delta_depth | u(4) |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 3.  Modified Slice Header Syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( ! lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| slice_common_info_set_id | ue(v) |
| …… |  |
| } |  |
| …… |  |
| if( !lightweight_slice_flag ) { |  |
| …… |  |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| } |  |
| } |  |

where

slice_common_info_set_id specifies the slice common information set in use. The value of slice_common_info_set_id shall be in the range of 0 to 1023, inclusive.

# Conclusions

A new parameter set namely “slice_common_info_set” for ALF parameters is proposed and placed between PPS and slice header. With slice_common_info_set, it is beneficial for the codec to carry out the slice common information sharing concept while costing bits as few as possible for coding duplicated information. It is suggested to adopt this sharing implementation into WD.

# References

[1] Chia-Yang Tsai, Chih-Wei Hsu, Yu-Wen Huang, Shawmin Lei. "CE4 Subset3: Slice Common Information Sharing", JCT-VC Document, JCTVC-E045-v3, 5th Meeting, Geneva, CH, March 2011.

[2] Thomas Wiegand, Woo-Jin Han, Benjamin Bross, Jens-Rainer Ohm, Gary J. Sullivan. "WD3: Working Draft 3 of High-Efficiency Video Coding", JCT-VC Document, JCTVC-E603-v8, 5th Meeting, Geneva, CH, March 2011.

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).