| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F189_r2 WG11 Number: m20608 |
| --- | --- |

| Title: | CE12: Verification results of Ericsson/MediaTek’s deblocking filter JCTVC-F118 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage Saori Asaka | Email: | tomoo.yamakage@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

Verification results of JCTVC-F118 (CE12: Ericsson’s and MediaTek’s deblocking filter) are reported.  Coding efficiency results are exactly matched with the proponent’s results for intra only, random access and low delay B cases.  As for low delay P case, the cross-checker is waiting for the proponents’ results.  No mismatch between encoder and decoder is observed so far.  Relative encoding and decoding time are similar to those of proponents’.  Source code was reviewed and confirmed the code was consistent with the description in JCTVC-F118.  Since the tc offset value adjustment for intra blocks is not included in original contributions (JCTVC-E079 and JCTVC-E276), the spirit of JCTVC-E193 and JCTVC-F143 can be considered to incorporate JCTVC-F118.

# Introduction

This contribution is a report of verification results of JCTVC-F118, which is a contribution to CE12.  JCTVC-F118 is the harmonization of deblocking proposals in CE12 based on both JCTVC-E079 and JCTVC-E276.  In addition, a tc offset value for intra blocks and chroma filtering processes are modified.

Verification is performed under the CE12 test conditions that refer to JCTVC-E700 and JCTVC-E712.  Coding efficiency results are exactly matched with the proponent’s results so far.  Relative encoding and decoding time are similar to those of proponent’s.  Verification was performed on several PCs that were identically configured and Turbo Boost and Hyper Threading were disabled so that the clock frequency is kept when the number of threads is smaller than the number of CPU cores.  As for decoding time measurement, each PC run only one decoding in order to avoid HDD access conflict that may affect decoding time measurement.

Source code was reviewed and confirmed that it was consistent with the description in JCTVC-F118.

# Experimental Results

Detailed results in Excel sheet are attached.  The following table is the summary of experimental results:

| Test case | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IO | RA | LDB | LDP | IO | RA | LDB | LDP |
| Class A | -1.6 | -1.4 |  |  | -1.3 | -1.4 |  |  |
| Class B | -1.2 | -1.2 | -1.4 | -1.3 | -1.0 | -1.3 | -1.4 | -1.2 |
| Class C | -0.9 | -1.0 | -1.1 | -1.0 | -0.9 | -1.0 | -1.0 | -0.9 |
| Class D | -0.9 | -0.8 | -0.9 | -0.9 | -0.8 | -0.8 | -0.8 | -0.7 |
| Class E | -1.3 |  | -2.8 | -2.7 | -1.1 |  | -2.4 | -2.0 |
| Average | -1.2 | -1.1 | -1.5 | -1.4 | -1.0 | -1.1 | -1.3 | -1.2 |
| Enc time | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Dec time (original) | 101% | 101% | 101% | 101% | 103% | 102% | 103% | 104% |
| Dec time (improved) | 101% | 100% | 100% | 100% | 102% | 102% | 102% | 102% |

Dec time (original) is the relative decoding time of originally provided source code.  Dec time (improved) is the relative decoding time of improved source code.

There is no mismatch between encoder and both decoders.  Relative encoding and decoding time are similar to those of proponent’s.

# Conclusion

This contribution shows verification results of JCTVC-F118. Coding efficiency results are confirmed so far.  No mismatch between encoder and decoder is observed.  Relative encoding and decoding time are also confirmed.  Source code was reviewed.  Since the tc offset value adjustment for intra blocks is not included in JCTVC-E079 or JCTVC-E276, the spirit of our contributions (JCTVC-E193 and JCTVC-F143) can be considered to incorporate JCTVC-F118.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).