| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F190 |
| --- | --- |

| Title: | Planar Mode Mapping for Intra Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feng Zou | Emails: | fengzou@ust.hk |
|  | Oscar C. Au |  | eeau@ust.hk |
|  | Chao Pang |  | pcece@ust.hk |
|  | Jingjing Dai Xingyu Zhang |  | jjdai@ust.hk eexyzhang@ust.hk |
|  | Xing Wen |  | wxxab@ust.hk |
|  | HKUST, Clear Water Bay |  |  |
|  | KLN, Hong Kong |  |  |
| Source: | The Hong Kong University of Science and Technology |  |  |

_____________________________

# Abstract

This contribution proposes a codeword mapping scheme for Planar mode in CABAC setting. In HM3.0, when the syntax prev_intra_luma_pred_flag (MPM_flag) is 0 and the two MPM candidates are different, there is always one redundant codeword in the remaining mode coding. The proposed scheme removes the codeword redundancy by mapping Planar mode to the mode who corresponds to the redundant codeword. In this case, the DCPlanar flag is also saved.

In particular, the proposed scheme allows Planar mode to be mapped to Mode 2 first, and then decides whether the most probable modes can predict it correctly. If most probable modes cannot predict the current mode correctly (prev_intra_luma_pred_flag=0), and the two most probable modes are different, the Planar mode is remapped to the mode who corresponds to the max allowable codeword used in remaining mode coding. Therefore there is no need to transmit the DCPlanar flag in this case. Otherwise, the original HM3.0 scheme is applied. Doing so not only saves DCPlanar flag but also reduces codeword design redundancy in HM3.0. The proposed scheme is implemented in HM3.0, achieving average 0.1% bitrate reduction for intra HE with no effect on encoding and decoding time. The results are verified by Motorola Mobility in JCTVC-F572[1].

- Introduction

In the Geneva meeting in March, 2011, there are two techniques adopted simultaneously, which are intra mode coding related. One is Intra Mode Coding from both Mediatek in JCTVC-E088[2] and NTT docomo in JCTVC-E131[3]. This technique allows up to 2 most probable modes, and uses one additional bit flag to indicate the most probable mode decision.

The other adopted technique is Intra Planar Mode from Docomo USA in JCTVC-E321[4]. In this technique, Planar mode shares the same mode index and codeword with DC mode during the most probable mode derivation and the mode coding. During the mode coding, an additional flag DCPlanar Flag is appended to indicate whether the current mode is DC or Planar.

In HM3.0, there are 35 luma intra prediction modes, including 33 angular prediction modes, DC and Planar for 8x8,16x16 and 32x32 PUs; 18 modes, including 16 angular modes, DC and Planar for 4x4 PU; and 4 modes, including 2 angular modes, DC and Planar for 64x64 PU. In this scheme, when none of the MPM candidates is the same as the current mode and the two MPM candidates are different, leaving 32(34-2=32) other possible mode indices with the remaining modes ranging from 0~31, or 15(17-2=15) other possible mode indices with the remaining modes ranging from 0~14, or 1(3-2=1) other possible mode index with the remaining mode 0. However, in CABAC intra mode coding, 5 or 6 bit escape code, which can represent 33 possible candidates, is used to represent 32 possible candidates only. 4 bit fixed length binary code, which can represent 16 possible candidates, is used to represent 15 candidates only. 1 bit fixed length binary code, which can represent 2 possible candidates, is used to represent 1 candidate only. Therefore, there is codeword redundancy for the remaining mode coding.

# Proposed methods

In order to reduce the codeword redundancy, the proposed method maps Planar mode to the mode who corresponds to the max allowable codeword in the remaining mode coding, when none of the MPM candidates is the same as the current mode and the two MPM candidates are different. Doing so not only reduces codeword redundancy but also saves DCPlanar flag in HM3.0. The proposed scheme is describe in Section 2.1 and 2.2 in detail.

## Most probable mode derivation

This part is the same as the existing method in HM3.0. If either of the two neighboring blocks is not available or not intra coded, its corresponding candIntraPredMode_N is set to be 2(N represents A or B).  A represents the neighboring block on the left, while B represents the neighboring block on the top, which is shown in Fig. 1. candIntraPredModeN are assigned to the candidate modes list with the smaller of the two candidates at candModeList[ 0 ] and the larger at candModeList[ 1 ]. If the neighboring block’s intra prediction mode is Planar mode, its corresponding candIntraPredMode_N is set to be 2.

Fig. 1 Most probable mode derivation

## Intra mode coding

The proposed scheme allows Planar mode to be mapped to Mode 2 first, and then decides whether the MPM candidates can predict it correctly.

- If one of the MPM candidates is equal to the current mode, then set prev_intra_luma_pred_flag =1. And then indicate the MPM candidate index if NumMPMC (number of candidate modes) is equal to 2. And then add one additional DCPlanar flag if the current mode is DC or Planar. This branch is the same as the one in HM3.0

- Otherwise, set prev_intra_luma_pred_flag =0. And then code the remaining mode rem_intra_pred_mode

-   If NumMPMC is equal to 1, the same method as HM3.0 is used in the remaining mode codeing. And then add one additional DCPlanar flag if the current mode is DC or Planar

-  Otherwise, the planar mode is remapped to the mode which corresponds to the max allowable codeword in the remaining mode coding. The following procedure is applied

-  If the current mode is Planar, then

-  set IntraPredMode =3, if the current PU is 64x64

-  set IntraPredMode=17, if the current PU is 4x4

-  set IntraPredMode=34, if the current PU is 8x8, 16x16 or 32x32

-  rem_intra_pred_mode = IntraPredMode

-  for (cIdx = NumMPMCand-1; cIdx>-1 ; cIdx --)

If (rem_intra_pred_mode > candModeList[cIdx]) rem_intra_pred_mode –

-  code the rem_intra_pred_mode using the same binarization method defined in HM3.0. Note that if the current mode is Planar, rem_intra_pred_mode has become 1, or 15, or 32, which is the vacant codeword in the remaining mode coding as discussed in Section 1. Also note that there is no need to code the DCPlanar flag if the current mode is DC or planar, since Planar and DC are using separate codewords.

In summary, the Planar mode mapping change is applied when the following two conditions are satisfied

- prev_intra_luma_pred_flag =0

- NumMPMC =2

Therefore, the modification upon HM3.0 is small, and simulation results indicate there is no encoding and decoding running time increase.

# Simulation results

Simulations were conducted following common test conditions defined in JCTVC-E700[5]. The anchor is HM3.0. Results are shown in Table 1 including the performance and running time of the proposed method.

Table 1. Simulation Results for All Intra HE

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.0 | 0.0 |
| Class B | -0.1 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 |
| Class D | -0.1 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 |
| All | -0.1 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusions

This technique maps Planar to the mode which corresponds to the max allowable codeword in the remaining mode coding, when none of the MPM candidates is the same as the current mode and the two MPM candidates are different. Doing so not only saves DCPlanar flag but also reduces codeword design redundancy in HM3.0. The proposed scheme achieves average 0.1% bitrate reduction in Intra HE case with no encoding and decoding time increase.

# Patent rights declaration(s)

The Hong Kong University of Science and Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Jian Lou, Limin Wang , “Cross-check report for HKUST's test JCTVC-F190 by Motorola Mobility”, JCTVC-F572, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Torino, Italy, July. 2011.

- Mei Guo, Xun Guo, Shawmin Lei, “CE14 Subtest 1: The Most Probable Mode Signaling for Luma”, JCTVC-E088, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, March. 2011.

- TK Tan, “CE14.1: Results for DOCOMO’s proposal and cross verification of MediaTek’s implementation for the most probable mode signaling for luma”, JCTVC-E131, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, March. 2011.

- Sandeep Kanumuri, Frank Bossen, “CE6.e/f: Planar mode experiments and results”, JCTVC-E321, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, March. 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, March. 2011.

Appendix A: Proposed modification of HEVC Working Draft 3

The modified or added portions are highlighted with yellow color.

7.3.7 Prediction unit syntax

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx , InferredMergeFlag ) { | Descriptor |
| --- | --- |
| ...... |  |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| if( NumMPMCand > 1 ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| if( IntraPredMode[ x0 ][ y0 ] == 2 && (prev_intra_luma_pred_flag[ x0 ][ y0 ] ¦¦ (!prev_intra_luma_pred_flag[ x0 ][ y0 ] && NumMPMCand==1 )) ) |  |
| planar_flag_luma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| if( IntraPredMode[ x0 ][ y0 ] != 2 && IntraPredMode[ x0 ][ y0 ]!=34 && SignaledAsChromaDC ) |  |
| planar_flag_chroma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } |  |

    - Derivation process for luma intra prediction mode

6.   IntraPredMode[ xB ][ yB ] is derived by applying the following procedure.

- If prev_intra_pred_flag[ xB ][ yB ] is true, the IntraPredMode[ xB ][ yB ] is set equal to  candModeList[mpm_idx[ xB ][ yB ]]

- Otherwise IntraPredMode[ xB ][ yB ] is derived by applying the following procedure

- IntraPredMode[ xB ][ yB ] = rem_intra_pred_mode

- for (cIdx = 0; cIdx < NumMPMCand; cIdx++)

if (IntraPredMode[ xB ][ yB ]>= candModeList[cIdx])     IntraPredMode[ xB ][ yB ]++

- IntraPredMode[ xB ][ yB ] is further modified by the following procedure.

- If  prev_intra_pred_flag[ xB ][ yB ] is false and NumMPMCand is equal to 2,

- If intraPredModeNum is equal to 4, and IntraPredMode[ xB ][ yB ] is equal to 3, IntraPredMode[ xB ][ yB ] is further modified to be equal to Intra_Planar

- Otherwise,if intraPredModeNum is equal to 18, and IntraPredMode[ xB ][ yB ] is equal to 17, IntraPredMode[ xB ][ yB ] is further modified to be equal to Intra_Planar

- Otherwise, if intraPredModeNum is equal to 35, and IntraPredMode[ xB ][ yB ] is equal to 34, IntraPredMode[ xB ][ yB ] is further modified to be equal to Intra_Planar

- Otherwise,  if IntraPredMode[ xB ][ yB ] is equal to Intra_DC and planar_flag_luma is equal to 1, IntraPredMode[ xB ][ yB ] is further modified to be equal to Intra_Planar.