| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F191 |
| --- | --- |

| Title: | CE12: Results for decisions for deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matthias Narroschke Thomas Wedi Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +49 6103 766 204 Matthias.Narroschke@eu.panasonic.com +49 6103 766 1194 Thomas.Wedi@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is part of CE12. It presents the results for Decisions for deblocking which was presented first in JCTVC-E251. In HM3.0, one single decision for enabling the deblocking is performed for each edge segment of eight lines/columns using two decision values, each of these representing the second derivation within a block neighboring to the edge. In contrast to the HM3.0, H.264/MPEG-4 AVC calculates decision values and performs decisions for each individual line/column. Changing the decisions of HM3.0 to individual ones similar as in H.264/MPEG-4 AVC reduces the bit rate at the same quality by 0.2% in average over all test cases. However, the calculation of additional decision values is associated with additional computational expense. In this proposal, individual decisions are performed for each line/column but decision values are calculated for only 4 lines/columns. The remaining 4 ones are interpolated. At a 47% lower additional computational expense than the H.264/MPEG-4 AVC like decisions, the same average bit rate reduction of 0.2% at the same quality is achieved compared to HM3.0 (I-HE: 0.1%, I-LC: 0.1%, RA-HE: 0.2%, RA-LC: 0.2%, LD(B)-HE: 0.3%, LD(B)-LC: 0.3%, LD(P)-HE: 0.3%, LD(P)-LC: 0.2%). Encoder/decoder run times stay approximately unchanged. For the low delay high efficiency configuration, an average bit rate reduction of more than 0.5% in achieved for the Class E sequences. Combined with an evaluation of the change of the signal gradient at the block boundary in the decision process, the average bit rate reduction is increased to 0.5% (I-HE: 0.4%, I-LC: 0.2%, RA-HE: 0.5%, RA-LC: 0.3%, LD(B)-HE: 1.0%, LD(B)-LC: 0.6%, LD(P)-HE: 0.9%, LD(P)-LC: 0.2%). It is proposed to adopt both proposed techniques into the next version of the HM.

# Introduction

The improved Decisions for deblocking were first presented in the proposal JCTVC-E251 [6] at the last meeting in Geneva. At that meeting, an expert viewing was performed. The result was that the subjective quality of the proposal was slightly better than the one of the reference [4]. Consequently, it had been decided to evaluate the improved Decisions for deblocking within the core experiment CE12 [5].

In the following Section Error: Reference source not found, a brief technical review is given. Further details can be found in JCTVC-E251 [6]. Section 4 explains the performed experiments and shows the results. Section 5 concludes this contribution.

# Proposed individual decisions for each line/column

The current HM 3.0 [1][2] applies hybrid coding. In a first step, the input signal to be coded is predicted block-wise by either motion compensated prediction or Intra prediction. The resulting prediction error is block-wise transform coded by applying an approximation of the discrete cosine transform, also known as Integer DCT, followed by a quantization of the coefficients. Due to the block wise motion compensated prediction and a block wise prediction error coding, so called blocking artifacts often become visible in the decoded images. These blocking artifacts tend to be annoying for human observers. In order to reduce these annoying blocking artifacts, an adaptive deblocking filter is applied. The deblocked signal is further filtered by the use of an adaptive loop filter before being output and stored for further predictions. Figure 1 illustrates the signal before and after the deblocking filter for a region of the example test sequence Kimono.

Figure 1: Illustration of the signal before and after the deblocking filter for a region of the example test sequence Kimono.

The deblocking of images is performed based on edge segments of 8 consecutive lines or columns. The respective sides of a segment are denoted as A and B, see Figure 2. The segment A corresponds to the left neighboring partition to B for vertical edges and to the above neighboring partition to B for horizontal edges. The samples on the respective sides of an edge segment are denoted as also shown in Figure 2.

Figure 2: Notation of a segment of a vertical edge for deblocking (from [3]).

For each segment of 8 lines/colums, HM3.0 performs the decision and filtering operations as explained in the following section 2.1.

## Decisions according to HM3.0

In a first step, two vales d2 and d5 are calculated by the use of the samples of two lines/columns as illustrated in Figure 3:

Figure 3: Calculation of the values d2 and d7 required to decide whether to filter or not if according to HM3.0 [1].

Hereby, the term p22−2⋅p1+p02 can be written as p22−p1−p12−p0. It can be seen that it represents the change of the gradient of the signal within the Block A in the line/column with the number 2. The change of the gradient is also known as the second derivation.

Therefore, the absolute values p22−2⋅p1+p02, and p25−2⋅p1+p05 each represent the change of the gradient of the signal within the Block A and the absolute values q22−2⋅q1+q02, and q25−2⋅q1+q05 each represent the change of the gradient of the signal within the Block B.

By the use of d2 and d5, it is decided by the threshold operation d2+d5<β if all 8 lines/columns of the corresponding segment are filtered or not.

In order to perform the decision for a segment of 8 lines/columns, 20 operations are required.

## Individual decisions similar as in H.264/MPEG-4 AVC

In contrast to the HM3.0, H.264/MPEG-4 AVC applies individual decisions for each line/column. In order to investigate decisions similar as in H.264/MPEG-4 AVC in the HM3.0 framework, an individual value di is calculated for each of the 8 lines/columns as illustrated in Figure 4.

Figure 4: Calculation of the values d0,d1,…,d7 required to decide whether to filter or not according to decisions similar as in H.264/MPEG-4 AVC.

By the use of the individual values di, it is decided for each individual line/column by the threshold operation 2⋅di<β if a line/column of the corresponding segment is filtered or not.

In order to perform the decision for a segment of 8 lines/columns, 88 operations are required.

## Proposed individual decisions

In this contribution, individual decisions are proposed which are associated with a lower computational expense than the ones similar as in H.264/MPEG-4 AVC, measured by the number of required operations. For this purpose, only four values d0', d2', d5', and d7' are calculated for each segment of 8 lines/columns as illustrated in Figure 5.

Figure 5: Calculation of the values d0', d2', d5', and d7' required to decide whether to filter or not according to the proposal.

These values are normalized and interpolated resulting in decision values for each of the 8 individual lines or columns:

di=2⋅di'                        for i=0,2,5,7

di=di−1'+di+1'                for i=1,6

d3=4⋅d2'+2⋅d5'

d4=4⋅d5'+2⋅d2'

By the use of the individual values di, it is decided for each individual line/column by the threshold operations

di<β      for i=0,1,2,5,6,7

di<3⋅β  for i=3,4

if a line/column of the corresponding segment is filtered or not.

In order to perform the decision for a segment of 8 lines/columns, 56 operations are required. Hereby, it is assumed that the value 3⋅β is pre-calculated and stored in a table in a same way as β.

# Evaluation of the change of the gradient at the block boundary

As explained in Section 2.1, the only the change of the gradients within the blocks are evaluated for the decision whether to apply the deblocking filter or not. The change of the gradient at the block boundary is not considered. This leads to the problems summarized in the following table. Especially in situations, in which a large signal change at the block boundary is caused by a real signal change, these decisions leads to undesired deblocking and thus to undesired smoothing of the signal as illustrated in Figure 6.

| Size of signal change at the block boundary | Main reason for signal change at the block boundary | Problems of HM3.0 decisions |
| --- | --- | --- |
| Small | Quantization error | Since the signal change at the block boundary is small, blocking artifacts may not be visible (No problem) |
| Small | Real signal | Since the signal change at the block boundary is small, blocking artifacts may not be visible (No problem) |
| Large | Quantization error | If not deblocked, blocking artifacts may become visible. However, no deblocking is only decided in the case that the signal changes within the blocks neighboring to the edge are also large. (Small problem) |
| Large | Real signal | If deblocking is decided, it leads to undesired smoothing (Serious problem) |

Figure 6: Illustration of a situation, in which a large offset at the block boundary caused by a real signal change causes undesired deblocking in HM3.0.

To overcome this mentioned problem, also the second derivation at the block boundary is evaluated in order to detect large signal changes at block boundaries. This evaluation comprises a comparison against a QP-dependent threshold value as well as a comparison against the second derivation within the blocks. In total, deblocking is only applied if

- The second derivation within the blocks is small

- The second derivation at the block boundary is small

- The second derivation at the block boundary is larger than the second derivation within the blocks

In order to calculate the second derivation at the block boundaries, the four values b0', b2', b5', and b7' are calculated for each segment of 8 lines/columns as illustrated in Figure 7.

Figure 7: Illustration of the samples used to calculate the second derivations at the block boundary.

The values b0', b2', b5', and b7' are normalized and interpolated resulting in decision values for each individual line or column:

bi=2⋅bi'                        for i=0,2,5,7

bi=bi−1'+bi+1'                for i=1,6

b3=4⋅b2'+2⋅b5'

b4=4⋅b5'+2⋅b2'

By the use of the second derivations within the blocks di and at the block boundaries bi, it is decided for each individual line/column by the threshold operations

di<β        ∧      bi<β<<2         ∧      2⋅bi>3⋅di      for i=0,1,2,5,6,7

di<3⋅β    ∧      bi<3⋅β<<2      ∧      2⋅bi>3⋅di      for i=3,4

if a line/column of the corresponding segment is filtered or not.

# Experiments and results

The individual decisions similar as in H.264/MPEG-4 AVC, see Section 2.2, the proposed individual decisions, see Section 2.3, as well as the combination of the proposed individual decisions with the evaluation of the gradient change at the block boundary, see Section 3, have been integrated into the reference software of HM3.0 [1].

## BD-bit rate and run time ratios

Following the common conditions [7], the performance is evaluated for all six mandatory as well as for the two optional test cases, which is Intra, Random access, Low delay B, and Low delay P, each in high efficiency and low complexity operation mode. For all run time measurements, computers of the same configuration are used.

The BD-rates as well as the encoder-/decoder run time ratios compared to the reference HM3.0 [1] are shown in Figure 8 for the individual decisions similar as in H.264/MPEG-4 AVC, in Figure 9 for the proposed individual decisions, and in Figure 10 for the proposed individual decisions in combination with the evaluation of the second derivation at block boundaries. Negative BD-rate numbers show a gain compared to the reference. Run-time ratios less than 100% show reflect that the run time is lower than the one of the reference. The following results can be observed:

- Proposed individual decisions

  - BD-bit rate reduction

    - 0.2% in average of over all test sequences and configurations

    - 0.6% in average for LD-LC(P), Class E

    - Approximately the same as achieved by the individual decisions performed similarly as in H.264/MPEG-4 AVC

  - Approximately no encoder-/decoder run time increase

- Proposed individual decisions combined with the evaluation of the gradient change at the block boundary

  - BD-bit rate reduction

    - 0.5% in average of over all test sequences and configurations

    - 2.1% in average for LD-LC(B), Class E

  - Approximately no encoder-/decoder run time increase

Figure 8: BD-bit rates and run time ratios of the individual decisions similar as in H.264/MPEG-4 AVC compared to the reference HM3.0 [1].

Figure 9: BD-bit rates and run time ratios of the proposed individual decisions compared to the reference HM3.0 [1].

Figure 10: BD-bit rates and run time ratios of the proposed individual decisions combined with the evaluation of the gradient change at the block boundary compared to the reference HM3.0 [1].

## Coding efficiency versus complexity for the individual decisions

In Figure 11, the achieved BD-bit rate reduction averaged over all test cases and test sequences is shown versus the additional number of required operations per edge segment of 8 lines/colums, both compared to the reference HM3.0. It can be observed that the proposed individual achieve the same average bit rate reduction of 0.2% compared to the HM3.0 reference but with 47% less additional operations than the individual decisions performed similar as in H.264/MPEG-4 AVC.

Figure 11: BD-bit rate reduction averaged over all test cases versus additional number of required operations per edge segment of 8 lines/columns compared to the reference HM3.0 [1].

# Conclusion

It is proposal to use individual decisions for each line/column for enabling and disabling the deblocking filter. Decision values are calculated for 4 lines/columns. The remaining 4 ones are interpolated. By this modification, an average BD-bit rate reduction of 0.2% (I-HE: 0.1%, I-LC: 0.1%, RA-HE: 0.2%, RA-LC: 0.2%, LD-HE(B): 0.3%, LD-LC(B): 0.3%, LD-HE(P): 0.3%, LD-LC(P): 0.2%) is achieved compared to HM3.0 with approximately no encoder/decoder run time increases. For the low delay high efficiency configuration, an average BD-bit rate reduction of 0.5% in achieved for the Class E sequences. Combined with the proposed evaluation of the gradient change at the block boundary in the decision process, the average bit rate reduction is increased to 0.5% (I-HE: 0.4%, I-LC: 0.2%, RA-HE: 0.5%, RA-LC: 0.3%, LD(B)-HE: 1.0%, LD(B)-LC: 0.6%, LD(P)-HE: 0.9%, LD(P)-LC: 0.2%). It is proposed to adopt both techniques into the next version of the HM.

# References

- HM3.0 software: http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-3.0.

- T. Wiegand, W.-J. Han, J.-R. Ohm, G. J. Sullivan, WD3: Working Draft 3 of High-Efficiency Video Coding, JCTVC-E603, Geneva, CH, March 2011.

- JCT-VC, Test Model under Consideration, JCTVC-B205_draft007, Geneva, Switzerland, 21-28 July 2010.

- A. Norkin, K. Andersson, K. Chono, I. S. Chong, M. Narroschke, B. Jeon, D. Flynn, T. Suzuki, BoG report on subjective viewing test for deblocking filter proposals, JCTVC-E501, Geneva, CH, March 2011

- A. Norkin, Xun Guo, B. Jeon, M. Narroschke, Description of Core Experiment 12 (CE12): Deblocking filtering, JCTVC-E712, Geneva, CH, March 2011.

- M. Narroschke, T. Wedi, Decisions for deblocking, JCTVC-E251, Geneva, CH, March 2011.

- F. Bossen, Common test conditions and software reference configurations, JCTVC-E700, Geneva, CH, March 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).