| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F192_r1 |
| --- | --- |

| Title: | A study on addition of 64x64 transform to HM 3.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yasuko Sugito Atsuro Ichigaya Shin-ichi Sakaida NHK Science & Technology Research Laboratories 1-10-11, Kinuta, Setagaya-ku, Tokyo, JAPAN Kazuo Sugimoto Akira Minezawa Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-3-5494-3353 sugitou.y-gy@nhk.or.jp |
| Source: | NHK (Japan Broadcasting Corporation) and Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution proposes addition of 64x64 transform to HM 3.0. 64x64 transform improves coding efficiency of higher resolution images and provides an average gain around 0.5% for sequences of Class A and B (peak is 2.6%). The results of encoding and decoding time are up to 3% increase compared with HM 3.0.

# Introduction

In TMuC software, transform size up to 64x64 was proposed and a performance verification on large block size transform was reported [1]. In the Dresden meeting, 64x64 transform was removed because of the balance between coding efficiency and hardware cost. In current HM software, transform size up to 32x32 is implemented and 64x64 transform is not implemented. However, TMuC was developed into HM. The complexity has been reduced significantly and the coding efficiency has been improved much. Moreover two Class A sequences cropped highest resolution sequences, 8Kx4K, were added to test sequences. Therefore, to verify coding efficiency and complexity of latest software, we added 64x64 transform to HM 3.0.

# Test conditions

2.1 basis

The new transform which was proposed by Cisco and TI[2] is adopted since HM 3.0. By using accuracy improvement method[3], we designed new 64x64 transform basis based on HM 3.0. The word "accuracy" means orthogonality and isometry of basis vectors. The 64x64 transform basis is attached as a text file to this contribution.

We tested in two conditions: Simulation A and B, and their bases are different as described below. Accuracy improved 16x16 and 32x32 transform bases are described in another contribution[3].

|  | 4x4 | 8x8 | 16x16 | 32x32 | 64x64 |
| --- | --- | --- | --- | --- | --- |
| Simulation A | O | O | O | O | * |
| Simulation B | O | O | * | * | * |

O: HM 3.0 original basis[2], *: Accuracy improved basis[3]

2.2 transform calculation

To calculate 64x64 transform and inverse transform, we used "partial butterfly"[1] in the same way as HM3.0's 4x4 to 32x32 transform and inverse transform.

2.3 TU structure

In current HM, maximum size of LCU is 64x64 and maximum size of TU is 32x32. Then if a size of LCU is 64x64, it would be separated into more than 4 TUs. In contrast, by using proposed method, we can treat an area of 64x64 LCU as an area of 64x64 TU.

In each simulations, "QuadtreeTULog2MaxSize" is configured in 6 for 64x64 transform and both "QuadtreeTUMaxDepthInter" and "QuadtreeTUMaxDepthIntra" are configured in 3 (current setting).

2.4 other conditions

Other conditions are configured in common test conditions [4].

# Results

3.1 Simulation A

Table 1 shows BD-rate results for Simulation A using HM 3.0 as the anchor for common test conditions[4]. 64x64 transform improves coding efficiency of higher resolution images. Although 64x64 transform applied only to luma, BD-rate results of chroma have also improved. The results of encoding and decoding time are up to 2% increase compared with HM 3.0.

Table 2 shows BD-rate Y results by sequences of Class A and B for Simulation A. 64x64 transform provides an average gain around 0.5% for sequences of Class A and B (peak is 2.6%).

Table 1: BD-rate results for simulation A (Accuracy improved 64x64 transform basis)

Table 2: BD-rate Y results by sequences of Class A and B for Simulation A

3.2 Simulation B

Table 3 shows BD-rate results for Simulation B using HM 3.0 as the anchor for common test conditions[4]. Since execution environment is different, Encoding/Decoding Time is slightly different from Simulation A. Some results show larger gains than Simulation A's.

Table 4 shows BD-rate Y results by sequences of Class A and B for Simulation B.

Table 3: BD-rate results for simulation B (Accuracy improved 16x16 to 64x64 transform bases)

Table 4: BD-rate Y results by sequences of Class A and B for Simulation B

# Conclusion

We added 64x64 transform to HM 3.0. It provides an average gain 0.0 to 0.4%. Especially, it improves coding efficiency of higher resolution sequences and provides gain up to 2.6%. The results of encoding and decoding time are up to 3% increase compared with HM 3.0.

# References

[1]Minhua Zhou, "Coding efficiency test on large block size transforms in HEVC," JCTVC-B028, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, July 2010

[2]A. Fuldseth, G. Bjontegaard, M. Sadafale, M. Budagavi, "Transform design for HEVC with 16 bit intermediate data representation," JCTVC-E243, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, March 2011

[3]Yasuko Sugito, Atsuro Ichigaya, Shin-ichi Sakaida, "Accuracy improvement of Cisco and TI's transform (JCTVC-E243)," JCTVC-F193, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July 2011

[4]Frank Bossen, "Common test conditions and software reference configurations," JCTVC-E700, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, March 2011

# Patent rights declaration(s)

NHK (Japan Broadcasting Corporation) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).