| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F193_r1 |
| --- | --- |

| Title: | Accuracy improvement of Cisco and TI's transform (JCTVC-E243) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yasuko Sugito Atsuro Ichigaya Shin-ichi Sakaida NHK Science & Technology Research Laboratories 1-10-11, Kinuta, Setagaya-ku, Tokyo, JAPAN | Tel: Email: | +81-3-5494-3353 sugitou.y-gy@nhk.or.jp |
| Source: | NHK (Japan Broadcasting Corporation) |  |  |

_____________________________

# Abstract

This contribution proposes high accuracy 16x16 and 32x32 transform bases based on HM 3.0's transform proposed by Cisco and TI[1]. The word "accuracy" means orthogonality and isometry (normality of transform) of basis vectors. Proposed bases are verified according to CE10's coding efficiency [3]. The results for normal QP range (QP=22, 27, 32, 37) and high QP range (QP=36, 42, 47, 51) show around 0% of average gains for luma. Almost all of the results of sequences for low QP range (QP=1, 5, 9, 13) show gains for luma, peak gain is 1.2% without any process time increasing.

# Introduction

The core transform of HM 3.0[1] is improved the balance between orthogonality and normality than HM 2.0. The transform is designed based on DCT and its features are similar to DCT's.

1. Basis vectors are almost orthogonal.

2. Norms of basis vectors are almost same.

Redesigning bases, we focused attention on orthogonality of basis vectors as well as their norms. To improve above features 1 and 2, we introduced the cost function for calculating elements of bases.

# Description

Hereinafter, we take the derivation of 16x16 transform basis for example.

Figure 1 shows the matrix product of HM 3.0's 16x16 transform basis and its transpose. The element of the matrix in the i th row and j th column represents the inner product of the i th basis vector and the j th basis vector. If there is the ideal 16x16 transform basis based on HM 3.0's transform, the matrix product of it and its transpose will be the result of multiplying identity matrix by 65536 (= 642x16), as indicated by Figure 2.

In view of the difference between Figure 1's matrix and Figure 2's matrix, we defined the cost function as below.

C=ai|(xi,xi)−b|+i,j|(xi,xj)|.    (1)

(a>0, xi represents the i th basis vector, i,j=1,2,3,...,16, i≠j, bis the ideal size of norm: 65536)

Figure 1: Matrix product of HM 3.0's 16x16 transform basis and its transpose

Figure 2: Matrix product of the ideal 16x16 transform basis and its transpose

Figure 3: Matrix product of the proposed basis and its transpose

In formula (1), the variable Crepresents cost and would be 0 if xis are the ideal 16x16 transform basis vectors. We would like to obtain xis which minimize C at any given range.

Here, some 16x16 transform basis to be obtained has the following features.

- Odd rows of 16x16 transform basis vectors are already determined by 8x8 transform basis vectors.

- Basis vectors have symmetry as DCT's basis vectors.

- Correspondence relations of elements at even rows of basis vectors are determined in common with DCT.

Therefore, we can rewrite formula (1) as bellow.

C'=16|(x2,x2)−65536|+j=even,414|(x2,xj)|.    (2)

In formula (2), 16 indicates size of basis. In consideration of quantization, we focused on isometry of basis vectors than orthogonality of them.

As we obtained minimum ofC', we first calculated 256 times the 2nd 16x16 DCT basis vector and rounded it to integer. The vector obtained is r2=(90,87,80,70,57,43,26,9,−9,−26,−43,−57,−70,−80,−87,−90).

Secondly, in formula (2), by setting elements of x2 in the range of r2±1 which is same as HM 3.0's transform, we calculated basis vectors x4,x6,x8,...,x14 corresponding to x2and minimum of C'. The basis vector x2 which minimize C' is x2=(91,87,79,70,56,43,27,8,−8,−27,−43,−56,−70,−79,−87,−91). To maintain the shape of a cosine wave, we selected elements of x2in descending order.

The proposed 16x16 transform basis is shown in Figure 4. Colored elements show difference from current HM     3.0's.

Figure 3 shows the matrix product of the proposed 16x16 transform basis and its transpose, colored hatching means improved elements. Figure 3's matrix is closer to Figure 2's matrix than Figure 1's matrix. This means accuracy of the proposed basis is improved than HM 3.0's.

The proposed 32x32 transform basis was obtained by the same method as the proposed 16x16 transform basis. The proposed 32x32 transform basis is shown in Figure 5. Colored elements show difference from current HM 3.0's. We also designed 64x64 transform basis and it is used in another contribution[2].

Figure 4: Proposed 16x16 transform basis

Figure 5: Proposed 32x32 transform basis

# Results

We verified proposed bases according to conditions of CE10's coding efficiency[3].

Table 1 shows BD-rate results for the proposed bases using HM 3.0 as the anchor for common conditions (QP=22, 27, 32, 37)[4]. Averages of BD-rate show some losses, but these are less than 0.1%.

Table 2 shows BD-rate results for low QP range (QP=1, 5, 9, 13) and 2 sec of video.

Table 3 shows BD-rate Y results by sequences for low QP range, colored hatching means gain. It shows accuracy improvement provides gains to almost all of sequences. Analysis results of TU structure at low QP range is described below.

Table 4 shows BD-rate results for high QP range (QP=36, 42, 47, 51) and 2 sec of video. There are losses for luma, but their average are less than 0.1%. The results for chroma vary significantly. We carry the reason forward.

All of the results show approximately-same complexity as HM 3.0.

Table 1: BD-rate results for normal QP range (QP=22, 27, 32, 37)

Table 2: BD-rate results for low QP range (QP=1, 5, 9, 13)

Table 3: BD-rate Y results by sequences, low QP range (QP=1, 5, 9, 13)

Table 4: BD-rate results for high QP range (QP=36, 42, 47, 51)

# Analysis of the results

We analyzed the results of low QP range and high QP range.

4.1 low QP range

Figure 6 shows comparison of TU structure in Kimono, AI_HE, QP=5, POC=4. As shown in Table 5, by using proposed transform bases, both image capacity and PSNR Y of this picture are improved.

Figure 7 shows comparison of top-left area of Figure6 and Table 6 shows count of TUs in Figure 6. By using proposed bases, the number of larger TUs is increased. This is believed to have improved image capacity and PSNR Y.

Figure 6: Comparison of TU structure, Pink line: HM 3.0, Aqua line: Proposed

(Kimono, AI_HE, QP=5, POC=4)

Table 5: Comparison of Image capacity and PSNR Y(Kimono, AI_HE, QP=5, POC=4)

|  | Image capacity [bit] | PSNR Y [dB] |
| --- | --- | --- |
| HM 3.0 | 8,470,008 | 57.3731 |
| Proposed | 8,452,848 | 57.3852 |

Figure 7: Top-left area of Figure 6 (Left: HM 3.0, Right: Proposed)

Table 6: Count of TUs in Figure 6 (luma and chroma)

|  | 4x4 | 8x8 | 16x16 | 32x32 | Total |
| --- | --- | --- | --- | --- | --- |
| HM 3.0 | 25120 | 17272 | 5034 | 307 | 47733 |
| Proposed | 21184 | 15372 | 5171 | 453 | 42180 |

4.2 high QP range

At high QP range, RA_HE, Class D, BD-rate U losses over 3%. This is because of shapes of RD curves. For example, Figure 8 shows shapes of RD curve U, high QP range, BQSquare, RA_HE. In this case, BD- rate may not suite for verifying coding efficiency. Same kind of problems are also shown in high QP range, RA_LC, LB_HE and LB_LC.

Figure 8: BQSquare, RA_HE, high QP range, RD curve for U

# Conclusions

We proposed high accuracy 16x16 and 32x32 transform bases based on Cisco and TI's transform[1]. We verified these bases according to CE10's coding efficiency[3]. All of the results show approximately-same complexity as HM 3.0. The results of low QP range showed accuracy improvement provides gains to almost all of sequences for luma.

# References

[1] A. Fuldseth, G. Bjontegaard, M. Sadafale, M. Budagavi, "Transform design for HEVC with 16 bit intermediate data representation," JCTVC-E243, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, March 2011

[2]Yasuko Sugito, Atsuro Ichigaya, Shin-ichi Sakaida, Kazuo Sugimoto, Akira Minezawa, Shun-ichi Sekiguchi, "A study on addition of 64x64 transform to HM 3.0," JCTVC-F192, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July 2011

[3]Pankaj Topiwala, Madhukar Budagavi, Arild Fuldseth, Rajan Joshi, Il-Koo Kim, "CE10: Core transform design," JCTVC-E710, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, March 2011

[4]Frank Bossen, "Common test conditions and software reference configurations," JCTVC-E700, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, March 2011

# Patent rights declaration(s)

NHK (Japan Broadcasting Corporation) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).