| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F195 |
| --- | --- |

| Title: | CE6.b: Cross-verification results of Microsoft’s LM mode integration on SDIP by LG |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jaehyun Lim Byeongmoon Jeon 221, Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Tel: Email: | jaehyun.lim@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This document is a crosscheck report of JCTVC-F196 on LM mode integration on SDIP submitted by Microsoft. Actually in this verification test, coding efficiency which is achieved by LM mode in SDIP chroma block was measured. The simulation succeeded and produced the same result as the proponent provided.

# Verification

The proponents provide their source codes which are described in JCTVC-F196. Two separate binary files for each encoder and decoder are compiled for Windows 64bit system. The simulation follows the common test conditions of HM 3.0 as below with SDIP option.

- Intra only, high efficiency

- Intra only, low complexity

# Experimental Results

The simulation succeeded and produced as same result as the proponent provided in performance as shown in Table.1. The encoding time and decoding time was also similar to those provided by the proponent. The spreadsheet JCTVC-F195.xls contains the detailed data of the simulation.

Table.1 BD rate (%) reduction of the proposed method

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.3 | 0.4 | 0.0 | 1.0 | 0.5 |
| Class B | 0.0 | 0.5 | 0.5 | 0.1 | 0.8 | 0.7 |
| Class C | 0.1 | 1.0 | 1.0 | 0.2 | 1.3 | 1.3 |
| Class D | 0.1 | 0.6 | 0.5 | 0.1 | 0.8 | 0.7 |
| Class E | 0.0 | 0.5 | 0.5 | 0.1 | 0.9 | 1.1 |
| All | 0.1 | 0.6 | 0.6 | 0.1 | 1.0 | 0.8 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |