| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F196 |
| --- | --- |

| Title: | CE6.b Report of Test 4: Harmonization of LM mode and the chroma prediction in SDIP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiulian Peng Jizheng Xu | Email: {v-xipe, jzxu}@microsoft.com |  |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

This document reports the results of the harmonization of LM mode and the chroma prediction in SDIP for Core Experiment 6.b Test4, JCTVC-E706 [1]. LM and SDIP were presented in JCTVC-E266 [3] and JCTVC-E278 [2], respectively. CE6.b Test4 evaluates how much coding efficiency will be lost when LM is disabled only for SDIP partitions.

# Introduction

In HM3.0, the LM mode is added for chroma prediction. The chroma prediction mode is coded as shown in Table 1, where “DM” denotes the luma prediction mode of the first PU in the corresponding luma block. The DM mode is coded with the shortest codeword, followed by LM mode, Ver, Hor, DC and Planar.

However, in the SDIP modes, the chroma block uses not the luma prediction mode of the first PU in the corresponding luma block but all four luma prediction modes of the four PUs in the luma block. The four luma modes are coded with the shortest codewords, followed by Ver, Hor and DC. When DM is also used for SDIP chroma prediction, the codewords for chroma modes should be redesigned.

In HM-3.0-SDIP software, the chroma prediction mode coding for SDIP partitions with LM mode has been implemented. CE6.b Test4 is to test the coding efficiency of DM mode in SDIP partitions.

Table 1 Mode Numbers and Codewords for intra chroma prediction mode

| Intra chroma prediction mode | Mode number | Codewords |
| --- | --- | --- |
| DM | 4 | 0 |
| LM | 3 | 10 |
| Ver | 0 | 110 |
| Hor | 1 | 1111 |
| DC | 2 | 11100 |
| Planar | 34 | 11101 |

# Algorithm description

For SDIP partitions, the luma block is always partitioned into four PUs with four prediction modes in total. Therefore, the four luma prediction modes can all be the candidate modes for the corresponding chroma block. Let DM1~DM4 denote the four prediction modes in increasing order and their corresponding mode numbers are 4~7 (Table 2). Two modes the same as each other will be combined into one. In chroma mode coding, modes with number 4~7 are mapped to codewords with the shortest length.

Except the four luma prediction modes, other candidate chroma prediction modes include LM, Ver, Hor, DC and Planar. They are added one by one after the four luma modes in the SDIP chroma prediction mode list. One mode which is already in the chroma prediction mode list will not be added to the list again. The first 6 modes in the list will be used as the candidate modes for SDIP chroma prediction. In chroma mode coding, the corresponding index in the list will be binarized using truncated unary code. Therefore, LM mode is mapped to a longer codeword than the four luma prediction modes, followed by Ver, Hor, DC and Planar.

Table 2 Mode Numbers and Codewords for intra chroma prediction mode in SDIP partitions

| Intra chroma prediction mode | Mode number | Codewords |
| --- | --- | --- |
| DM1 | 4 | 0 |
| DM2 | 5 | 10 |
| DM3 | 6 | 110 |
| DM4 | 7 | 1110 |
| LM | 3 | 11110 |
| Ver | 0 | 11111 |
| Hor | 1 |  |
| DC | 2 |  |
| Planar | 34 |  |

# Test conditions and Results

The LM mode has already been implemented in HM-3.0-SDIP software in SDIP partitions. To evaluate its efficiency, the LM for SDIP partitions is disabled in HM-3.0-SDIP software by setting SDIP=1, LM=1, SDIP_LM=0. The results are to be compared with that of the HM-3.0-SDIP software with setting =1, LM=1, SDIP_LM=1. All results were tested in Intra and Intra LoCo cases under the recommended test conditions of CE6 [4].

The platform and results are shown in Table 3 and Table 4, respectively. When LM for SDIP partitions is disabled, there is a small loss in coding efficiency. The results validate the coding efficiency of LM for SDIP partitions.

Table 3 Platform configuration

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2008 R2, 64 bits | 2xIntel Xeon L5420@2.5Ghz | 16GB | VS2008 |

Table 4 Results of Intra and Intra LoCo configuration

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.01 | 0.34 | 0.38 | 0.03 | 1.02 | 0.54 |
| Class B | 0.03 | 0.52 | 0.52 | 0.07 | 0.76 | 0.68 |
| Class C | 0.13 | 1.03 | 0.99 | 0.21 | 1.27 | 1.28 |
| Class D | 0.09 | 0.64 | 0.54 | 0.13 | 0.85 | 0.69 |
| Class E | 0.02 | 0.50 | 0.49 | 0.06 | 0.93 | 1.07 |
| All | 0.06 | 0.61 | 0.59 | 0.10 | 0.96 | 0.83 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# References

- Ali Tabatabai, Madhukar Budagavi, Keiichi Chono, Mohammad Coban, Andrew Segall, Haoping Yu, “Core Experiment 6: Intra Prediction Improvement” Doc. JCTVC-E706, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, March 2011.

- Xiaoran Cao, Xiulian Peng, Changcai Lai, Yunfei Wang, Yongbing Lin, Jizheng Xu, Lingzhi Liu, Jianhua Zheng, Yun He, Haoping Yu, Feng Wu,, “CE6.b1 Report on Short Distance Intra Prediction Method”, Doc. JCTVC-E278, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, March 2011.

- Jianle Chen, Vadim Seregin, Woo-Jin Han, Jungsun Kim, Byeongmoon Jeon, “CE6.a.4: Chroma intra prediction by reconstructed luma samples”, Doc. JCTVC-E266, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, March 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-E700 (m20235), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, March 2011.

# Patent rights declaration(s)

University of Science and Technology of China, and Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix: text for working draft

### 7.3.7 Prediction unit syntax

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx , InferredMergeFlag ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( NumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && log2PUWidth >= Log2IPCMCUSize ) |  |
| pcm_flag | u(1) ¦ ae(v) |
| if( pcm_flag ) { |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| if( NumMPMCand > 1 ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| if( IntraPredMode[ x0 ][ y0 ] == 2 ) |  |
| planar_flag_luma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| if( PartMode != PART_2NxhN && PartMode != PART_hNx2N && IntraPredMode[ x0 ][ y0 ] != 2 && IntraPredMode[ x0 ][ y0 ]!=34 && SignaledAsChromaDC ) |  |
| planar_flag_chroma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } |  |
| } else { /* MODE_INTER */ |  |
| if( !InferredMergeFlag ) |  |
| if( entropy_coding_mode_flag ¦¦ PartMode != PART_2Nx2N ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] && NumMergeCand > 1 ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| if( slice_type = = B ) { |  |
| if(!entropy_coding_mode_flag && use_combined_inter_pred_ref( x0, y0 ) ) { |  |
| combined_inter_pred_ref_idx | ue(v) |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) |
| } else |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag && use_combined_inter_pred_ref( x0, y0 ) ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_lc_minus4[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_lc[ x0 ][ y0 ] | ae(v) |
| } |  |
| mvd_lc[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( LcToLx ) > 1 ) |  |
| mvp_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag && use_combined_inter_pred_ref( x0, y0 ) ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag && use_combined_inter_pred_ref( x0, y0 ) ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l1_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

### 8.3.2 Derivation process for chroma intra prediction mode

[Ed.: (WJ) this subclause may be moved to the semantics of intra_chroma_pred_mode syntax]

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–    a variable log2TrafoSize specifying the size of the current coding unit.

Output of this process is the variable IntraPredModeC.

Depending on PartMode, the chroma intra prediction mode IntraPredModeC is derived as follows:

- If PartMode is equal to PART_2Nx2N or PART_NxN, the chroma intra prediction mode IntraPredModeC is derived as specifed in Table 8-1 or Table 8-2 with intra_chroma_pred_mode, IntraPredMode[ xB ][ yB ] and chroma_pred_from_luma_enabled_flag as inputs.

IntraPredModeC is further modified to be equal to Intra_Planar when IntraPredModeC is equal to Intra_DC and planar_flag_chroma is equal to 1.

Table 8-1 – Specification of IntraPredModeC according to the values of intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ] when chroma_pred_from_luma_enabled_flag is equal to 1

| intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | X ( 4 <= X < 35 ) |
| 0 | 35 | 35 | 35 | 35 | 35 |
| 1 | n/a | 0 | 0 | 0 | 0 |
| 2 | 1 | n/a | 1 | 1 | 1 |
| 3 | 2 | 2 | 2 | 2 | 2 |
| 4 | 0 | 1 | 2 | 3 | X |

Table 8-2 – Specification of IntraPredModeC according to the values of intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ] when chroma_pred_from_luma_enabled_flag is equal to 0

| intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | X ( 4 <= X < 35 ) |
| 0 | n/a | 0 | 0 | 0 | 0 |
| 1 | 1 | n/a | 1 | 1 | 1 |
| 2 | 2 | 2 | 2 | 2 | 2 |
| 3 | 0 | 1 | 2 | 3 | X |

[Ed. (WJ): is it possible that intra_chroma_pred_mode = 2 and IntraPredMode[ xB ][ yB ] = 2? Since it is actualy overlapped with the case that intra_chroma_pred_mode = 3 (derivation from luma) and IntraPredMode[ xB ][ yB ] = 2]

- Otherwise (PartMode is equal to PART_2NxhN or PART_hNx2N), IntraPredModeC is derived as specifed in Table 8-3 or Table 8-4 with intra_chroma_pred_mode and the four intra prediction modes of the current coding unit as inputs.

If intra_chroma_pred_mode is equal to 4, 5, 6 or 7, IntraPredModeC is derived as follows:

1.   The variables xOff, yOff are set as follows:

if(PartMode == PART_2NxhN)

{

xOff = 0;

yOff = (1<< log2TrafoSize)>>2;

}

else if(PartMode == PART_ hNx2N)

{

xOff = (1<< log2TrafoSize)>>2;

yOff = 0;

}

2.	     The mapping table mapIntraPredModeC with a size of 4 recording the four intra prediction modes in increasing order is derived as follows.

IntraPredModeCnt[35] = {0};

for(blkIdx = 0; blkIdx < 4; blkIdx++)

IntraPredModeCnt[ IntraPredMode[ xB+xOff*blkIdx][yB+yOff*blkIdx] ]++;

j = 0;

for( n = 0; n < 35; n++ )

if( IntraPredModeCnt[n] )

mapIntraPredModeC[j++] = n;

3.	IntraPredModeC is then derived by

IntraPredModeC = mapIntraPredModeC[ intra_chroma_pred_mode-4 ].

Table 8-5 – Specification of IntraPredModeC when chroma_pred_from_luma_enabled_flag is equal to 1 and sdip_flag is equal to 1

| intra_chroma_pred_mode | IntraPredModeC |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 2 |
| 3 | 35 |
| 34 | 34 |
| 4~7 | as specified in this subclause |

Table 8-6 – Specification of IntraPredModeC when chroma_pred_from_luma_enabled_flag is equal to 0 and sdip_flag is equal to 1

| intra_chroma_pred_mode | IntraPredModeC |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |
| 34 | 34 |
| 4~7 | as specified in this subclause |

[ Note that in the HM3.0 and HM-3.0-SDIP software, LM mode number is 3 instead of 0. In Table 8-5, intra_chroma_pred_mode 0, 1, 2, 3 denotes Ver, Hor, DC and LM modes, respectively. It is different from Table 8-1. ]