| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F197 |
| --- | --- |

| Title: | CE6.b Report of Test 3: Interaction between SDIP and MDCS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiulian Peng Jizheng Xu | Email: {v-xipe, jzxu}@microsoft.com |  |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

This document reports the results of the interaction between MDCS and SDIP for Core Experiment 6.b Test3, JCTVC-E706 [1]. MDCS and SDIP were presented in JCTVC-D393 [3] and JCTVC-E278 [2], respectively. CE6.b Test3 evaluates the gains of MDCS in HM-3.0, MDCS in HM-3.0-SDIP, and the harmonization of SDIP and MDCS.

# Introduction

MDCS designs mode dependent scanning order for square TU coding. However, for SDIP partitions, the TUs can be rectangular. Special scanning order needs to be designed for SDIP TUs. In E278 as well as HM-3.0-SDIP software, MDCS has already been implemented in SDIP partitions with similar mapping table as in HM3.0. CE6.b Test3 is to test the interaction between SDIP and MDCS.

# Algorithm description

Similar to MDCS in square TU coding, there are three scan orders for rectangular SDIP TU coding: zigzag (0), horizontal (1) and vertical (2) as shown in Figure 1 for 8x2 TU.

Figure 1: MDCS coefficient scan orders for SDIP partitions, zigzag (0), horizontal (1), vertical (2).

In square TU, the scanning order is determined by the intra prediction mode and the TU size using a fixed look-up table. MDCS for SDIP TUs follows the same look-up tables as in HM3.0 with minor variations. Luma blocks use Table 1 and the chroma blocks use Table 2. For an MxN (M≠N) rectangular TU (must be a luma block), its scanning order is obtained by the look-up table with its intra prediction mode and the corresponding square block size nSxnS as indices, where nS is equal to the square root of M*N. Therefore, the scanning orders for 8x2/2x8/16x4/4x16 are mode dependent. The 32x8/8x32 blocks simply follow the Zigzag scan. Additionally, the 16x1/1x16 TUs are scanned simply from the start to the end since they are already one dimensional. For SDIP chroma blocks, since they are square due to zero TU depth, the scanning order is obtained by the look-up table in Table 2 with the chroma prediction mode and the block size as indices.

| Mode TU | 0 | 1 | 2/Planar | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32x32 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 16x16 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8x8 | 1 | 2 | 0 | 0 | 1 | 1 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 |
| 4x4 | 1 | 2 | 0 | 0 | 1 | 1 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 |

Table 1: LUT for SDIP luma coefficient scan index selection.

| Mode TU | 0 | 1 | 2/Planar | 3 | >=4 |
| --- | --- | --- | --- | --- | --- |
| 16x16 | 0 | 0 | 0 | 0 | Same as corresponding luma block |
| 8x8 | 0 | 0 | 0 | 0 | Same as corresponding luma block |
| 4x4 | 1 | 2 | 0 | 0 | Same as corresponding luma block |

Table 2: LUT for SDIP chroma coefficient scan index selection.

The significance map/significant coefficient coding context support for rectangular SDIP blocks with various scan orders is derived as follows.

First, the MxN (M≠N) rectangular block is scanned into a one-dimensional buffer according to the scanning order obtained above. The one-dimensional coefficients are then reorganized into an nSxnS square block by Zigzag mapping, where nS is equal to the square root of M*N. Figure 2 below shows the mapping for a 8x2 rectangular block with horizontal scan to a 4x4 square block as an example.

Figure 2: Mapping rectangular blocks with horizontal scan to square blocks.

Second, the generated nSxnS square block then uses the same context support of a square block with Zigzag scan for its significance map/significant coefficient coding.

The significance map/significant coefficient coding context support for square SDIP blocks (e.g. SDIP chroma blocks) use the same method as the square block in HM3.0.

# Test conditions and Results

The MDCS has already been implemented in HM-3.0-SDIP software in SDIP partitions. To evaluate the interaction between SDIP and MDCS, three tests are conducted with the configurations as follows.

Case 1: SDIP=0, MDCS=0, Anchor: HM-3.0

Case 2: SDIP=1, MDCS=0, Anchor: HM-3.0-SDIP

Case 3: SDIP=1, MDCS=1, SDIP_MDCS=0, Anchor: HM-3.0-SDIP

All results were tested in Intra and Intra LoCo cases under the recommended test conditions of CE6 [4].

The platform and results are shown in Table 3 and Table 4-6, respectively. The results of case 1 and case 2 show that MDCS performs better in HM-3.0 than HM-3.0-SDIP, i.e. some gains are compensated by SDIP. The results of case 3 show that the MDCS in SDIP partitions can bring 0.08% bit saving.

Table 3 Platform configuration

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2008 R2, 64 bits | 2xIntel Xeon L5420@2.5Ghz | 16GB | VS2008 |

Table 4 Case 1: SDIP=0, MDCS=0, anchor: HM-3.0 ( JCTVC-F197-case1.xls )

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.52 | 0.43 | 0.29 | 0.22 | -0.02 | -0.09 |
| Class B | 0.87 | 0.66 | 0.65 | 1.73 | 0.82 | 0.86 |
| Class C | 1.41 | 1.15 | 1.16 | 1.92 | 1.18 | 1.20 |
| Class D | 1.34 | 1.45 | 1.61 | 1.67 | 1.03 | 1.11 |
| Class E | 1.94 | 0.44 | 0.54 | 2.23 | 0.13 | 0.25 |
| All | 1.16 | 0.83 | 0.86 | 1.53 | 0.66 | 0.70 |
| Enc Time[%] | 98% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 5 Case 2: SDIP=1, MDCS=0, anchor: HM-3.0-SDIP ( JCTVC-F197-case2.xls )

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.31 | 0.14 | 0.17 | 0.00 | 0.13 | 0.06 |
| Class B | 0.47 | 0.32 | 0.35 | 0.45 | 0.53 | 0.49 |
| Class C | 0.88 | 0.80 | 0.78 | 0.88 | 0.78 | 0.79 |
| Class D | 0.93 | 1.15 | 1.27 | 0.79 | 0.65 | 0.74 |
| Class E | 0.87 | -0.18 | -0.21 | 0.61 | 0.09 | 0.21 |
| All | 0.67 | 0.47 | 0.50 | 0.54 | 0.46 | 0.47 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 6 Case 3: SDIP=1, MDCS=1, SDIP_MDCS=0, anchor: HM-3.0-SDIP ( JCTVC-F197-case3.xls )

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.01 | 0.06 | 0.05 | 0.03 | 0.08 | 0.10 |
| Class B | 0.05 | 0.05 | 0.03 | 0.06 | 0.10 | 0.13 |
| Class C | 0.16 | 0.16 | 0.17 | 0.15 | 0.18 | 0.20 |
| Class D | 0.18 | 0.19 | 0.16 | 0.11 | 0.09 | 0.11 |
| Class E | 0.02 | -0.05 | -0.04 | 0.02 | 0.10 | 0.05 |
| All | 0.08 | 0.09 | 0.08 | 0.08 | 0.11 | 0.12 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# References

- Ali Tabatabai, Madhukar Budagavi, Keiichi Chono, Mohammad Coban, Andrew Segall, Haoping Yu  “Core Experiment 6: Intra Prediction Improvement” Doc. JCTVC-E706, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, March 2011.

- Xiaoran Cao, Xiulian Peng, Changcai Lai, Yunfei Wang, Yongbing Lin, Jizheng Xu, Lingzhi Liu, Jianhua Zheng, Yun He, Haoping Yu, Feng Wu,, “CE6.b1 Report on Short Distance Intra Prediction Method”, Doc. JCTVC-E278, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, March 2011.

- Yunfei Zheng, Muhammed Coban, Xianglin Wang, Joel Sole, Rajan Joshi, Marta Karczewicz, “CE11: Mode Dependent Coefficient Scanning”, Doc. JCTVC-D393, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, Korea, January 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-E700 (m20235), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, March 2011.

# Patent rights declaration(s)

University of Science and Technology of China, and Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix: text for working draft

## 6.5 Zig-zag scanning array initialisation process

Input to this process is the block width blkWidth and the block height blkHeight..

Output of this process is the array ZigZag[ pos ][ comp ]. The array index pos specify the scan position ranging from 0 to ( blkWidth * blkHeight ) − 1. The array index comp equal to 0 specifies the horizontal component and the array index comp equal to 1 specifies the vertical component. Depending on the values of blkWidth and blkHeight, the array ZagZag is derived as follows.

- If blkWidth is less than or equal to blkHeight,

ZigZag[ 0 ][ 0 ]  =  0
ZigZag[ 0 ][ 1 ]  =  0
i  =  1
x  =  1
y  =  0
stopLoop  =  ( blk Height = = 1 ) ? true : false
while( !stopLoop ) {
	while( x  >=  0 ) {
		if (x  >=  0  &&  x  <  blkWidth  &&  y  >=  0  &&  y  <  blkHeight ) {
			ZigZag[ i ][ 0 ]  =  x
			ZigZag[ i ][ 1 ]  =  y
			i++
		}
		x− −
		y++
	}
	x  =  0
 	while( y  >=  0 ) {
		if (x  >=  0  &&  x  <  blkWidth  &&  y  >=  0  &&  y  <  blkHeight ) {
			ZigZag[ i ][ 0 ]  =  x
			ZigZag[ i ][ 1 ]  =  y
			i++
		}
		x++
		y− −
	}
	y  =  0
	if ( i  >=  blkWidth  *  blkHeight ) 
		stopLoop  =  true
}

- Otherwise(blkWidth is greater than blkHeight)

ZigZag[ 0 ][ 0 ]  =  0
ZigZag[ 0 ][ 1 ]  =  0
i  =  1
x  =  0
y  =  1
stopLoop  =  false
while( !stopLoop ) {

while( y  >=  0 ) {
		if (x  >=  0  &&  x  <  blkWidth  &&  y  >=  0  &&  y  <  blkHeight ) {
			ZigZag[ i ][ 0 ]  =  x
			ZigZag[ i ][ 1 ]  =  y
			i++
		}
		x++
		y− −
	}
	y  =  0
	while( x  >=  0 ) {
		if (x  >=  0  &&  x  <  blkWidth  &&  y  >=  0  &&  y  <  blkHeight ) {
			ZigZag[ i ][ 0 ]  =  x
			ZigZag[ i ][ 1 ]  =  y
			i++
		}
		x− −
		y++
	}
	x  =  0

	if ( i  >=  blkWidth  *  blkHeight ) 
		stopLoop  =  true
}

## 6.6 Scanning order array initialisation process

Input to this process is the block width blkWidth and the block height blkHeight.

Output of this process is the array ScanOrder[ scanIdx ][ pos ][ comp ]. The array index scanIdx equal to 0 specifies a zig-zag scan as specified in subclause Error: Reference source not found, scanIdx equal to 1 specifies a horizontal scan and scanIdx equal to 2 specifies a vertical scan. The array index pos specifies the scan position ranging from 0 to ( blkSize * blkSize ) − 1. The array index comp equal to 0 specifies the horizontal component and the array index comp equal to 1 specifies the vertical component. The array ScanOrder is derived as follows.

ScanOrder[0] = ZigZag

i  =  0
y  =  0
while( y  < blkHeight ) {
	x  =  0
	while( x  <  blkWidth ) {
		ScanOrder[ 1 ][ i ][ 0 ]  = x
		ScanOrder[ 1 ][ i ][ 1 ]  = y
		x++
		i++
	}
	y++
}

i  =  0
x  =  0
while( x  < blkWidth ) {
	y  =  0
	while( y  <  blkHeight ) {
		ScanOrder[ 2 ][ i ][ 0 ]  = x
		ScanOrder[ 2 ][ i ][ 1 ]  = y
		y++
		i++
	}
	x++
}

### 7.3.9 Transform coefficient syntax

| transform_coeff( x0, y0, log2Trafo Width, log2TrafoHeight, trafoDepth, cIdx ) { | Descriptor |
| --- | --- |
| if( ( ( cIdx = = 0 && cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 1 && cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 2 && cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) ) ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | se(v) ¦ ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| log2TrafoSize = (log2TrafoWidth + log2TrafoHeight) >> 1 |  |
| intraSdipFlag = (PartMode == PART_2NxhN ¦ ¦ PartMode == PART_hNx2N ) |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] && (!intraSdipFlag ¦ ¦ cIdx == 0)) { |  |
| if(PartMode == PART_2NxhN ){ |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) |  |
| transform_coeff( x0, y0, log2TrafoWidth, log2TrafoHeight - 2, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x0, y1, log2TrafoWidth, log2TrafoHeight - 2, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x0, y2, log2TrafoWidth, log2TrafoHeight - 2, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x0, y3, log2TrafoWidth, log2TrafoHeight - 2, trafoDepth + 1, cIdx ) |  |
| }else if (PartMode == PART_hNx2N) { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth) >> 2 ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth) >> 2 ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth) >> 2 ) |  |
| transform_coeff( x0, y0, log2TrafoWidth − 2, log2TrafoHeight, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y0, log2TrafoWidth − 2, log2TrafoHeight, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x2, y0, log2TrafoWidth − 2, log2TrafoHeight, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x3, y0, log2TrafoWidth − 2, log2TrafoHeight, trafoDepth + 1, cIdx ) |  |
| }else{ |  |
| x1 = x0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| transform_coeff( x0, y0, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y0, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x0, y1, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y1, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| } |  |
| } else { |  |
| if ( PredMode = = MODE_INTRA ) { |  |
| if (cIdx = = 0) |  |
| scanIdx = ScanType[ log2TrafoSize -2][ IntraPredMode ] |  |
| else |  |
| scanIdx = ScanTypeC[ log2TrafoSize -2][ IntraPredModeC ] |  |
| } else { |  |
| scanIdx = 0 |  |
| } |  |
| if ( entropy_coding_mode_flag ) |  |
| residual_coding_cabac( x0, y0, log2Trafo Width, log2TrafoHeight, trafoDepth, scanIdx, cIdx ) |  |
| else if ( !entropy_coding_mode_flag ) |  |
| residual_coding_cavlc( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, scanIdx, cIdx ) |  |
| } |  |
| } |  |
| } |  |

### 7.3.10 Residual coding CABAC syntax

| residual_coding_cabac( x0, y0, log2TrafoWidth,log2TrafoHeight, trafoDepth, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| log2TrafoSize = (log2TrafoWidth + log2TrafoHeight) >> 1 |  |
| if( log2TrafoWidth != log2TrafoHeight ) |  |
| scanIdx = 0 |  |
| last_significant_coeff_x | ae(v) |
| last_significant_coeff_y | ae(v) |
| n = 0 |  |
| xC = ScanOrder[ log2TrafoSize ][log2TrafoSize][ scanIdx ][ n ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize ] [log2TrafoSize][ scanIdx ][ n ][ 1 ] |  |
| while( ( xC != last_significant_coeff_x ) ¦¦ ( yC != last_significant_coeff_y ) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| n++ |  |
| xC = ScanOrder[ log2TrafoSize ] [log2TrafoSize] [ scanIdx ][ n ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize ] [log2TrafoSize] [ scanIdx ][ n ][ 1 ] |  |
| } |  |
| numSubsets = max( 1, ( 1 << ( log2TrafoSize << 1 ) ) >> ( cIdx > 0 ? 6 : 4 )) |  |
| for( i = 0; i < numSubsets; i++ ) { |  |
| offset = i << 4 |  |
| xS = ScanOrder[ log2TrafoSize ] [log2TrafoSize] [ 0 ][ i ][ 0 ] << 2 |  |
| yS = ScanOrder[ log2TrafoSize ] [log2TrafoSize] [ 0 ][ i ][ 1 ] << 2 |  |
| for( n = 0; n < 16; n++ ) { |  |
| xOffset = n – (n >> 2 ) << 2 |  |
| yOffset = n >> 2 |  |
| if( significant_coeff_flag[ xS + xOffset ][ yS + yOffset ] ) |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| } |  |
| for( n = 0; n < 16; n++ ) { |  |
| if( coeff_abs_level_greater1_flag[ n ] ) { |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| if( coeff_abs_level_greater2_flag[ n ] ) |  |
| coeff_abs_level_minus3[ n ] | ae(v) |
| } |  |
| } |  |
| for( n = 0; n < 16; n++ ) { |  |
| xOffset = n – (n >> 2 ) << 2 |  |
| yOffset = n >> 2 |  |
| if( significant_coeff_flag[ xS + xOffset ][ yS + yOffset ] ) { |  |
| coeff_sign_flag[ n ] | ae(v) |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = ( coeff_abs_level_minus3[ n ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = 0 |  |
| } |  |
| } |  |
| } |  |

### 7.4.10 Residual coding CABAC semantics

The array ScanOrder[ log2BlockWidth ][ log2BlockHeight ][ scanIdx ][ pos ][ comp ] specifies the mapping of the scan position pos, ranging from 0 to ( ( 1 << log2BlockWidth ) * ( 1 << log2BlockHeight ) ) – 1, to horizontal and vertical components of the scan-order matrix. The array index scanIdx equal to 0 specifies a zig-zag scan, scanIdx equal to 1 specifies a horizontal scan and scanIdx equal to 2 speicifes a vertical scan. The array index comp equal to 0 specifies the horizontal component and the array index comp equal to 1 specifies the vertical component. The array ScanOrder is derived as follows.

The scanning order array initialisation process as specified in  is invoked with 1 << log2TrafoWidth and 1 << log2TrafoHeight as input and the output is assigned to ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ].

### 8.5.2 Inverse scanning process for transform coefficients

Inputs to this process are:

–	two variables nW and nH specifying the width and height of the current transform unit,

–	a list of (nW)x(nH) values transCoeffLevel[ xT ][ yT ][ trafoDepth ][ cIdx ].

–	a variable cIdx specifying the chroma component of the current block,

- a variable scanIdx specifying the scanning order of the current block.

Output of this process is a variable c containing a two-dimensional array of (nW)x(nH) values.

- The variable c[ i ][ j ] which is located in the ( i, j ) position in the array c is derived as follows.

- The variable nS is set equal to 1<<((log2(nW)+log2(nH)>>1). The current transform unit is divided into ( nS >> 2 ) * ( nS >> 2 ) subsets of 4 * 4 samples. The subsets are processed in zig-zag scan order. The position ( k0, l0 ) in the array c, corresponding to the top left position of a subset s, is derived as follows.

k0 = ZigZag[ log2( nS >> 2 ) ][ s ][ 0 ] << 2			
l0 = ZigZag[ log2( nS >> 2 ) ][ s ][ 1 ] << 2, 			
with s = 0.. ( ( nS >> 2 ) * ( nS >> 2 ) ) – 1 )	(8-1)

  - For every subset s, the 16 transform coefficient level at scanning position n are mapped to the position ( i, j ) in the array c using the reverse zig-zag scan.

i = k0 + ZigZag[ 2 ][ n ][ 0 ]			
	j = l0 + ZigZag[ 2 ][ n ][ 1 ]			
	c[ i ][ j ] = transCoeffLevel[ xT ][ yT ][ trafoDepth ][ cIdx ][ n + ( s << 4 ) ],			
	with n = 15..0	(8-2)

- If nW is not equal to nH, the coefficients in the array c are reordered according to the corresponding scanning order scanIdx and derived as follows.

    - The array c0 is the copy of the array c. For scanning position n, a coefficient at position (i0, j0) of c0 is mapped to the new position (i, j) of the reordered array c.

i0 = ZigZag[ log2(nS) ][ n ][ 0]			
	j0 = ZigZag[ log2(nS) ][ n ][ 1]

i = ScanOrder[ log2(nW) ][ log2(nH) ][ scanIdx ][ n ][ 0 ]

j = ScanOrder[ log2(nW) ][ log2(nH) ][ scanIdx ][ n ][ 1 ]			
	c[ i ][ j ] = c0[ i0 ][ j0 ]			
	 with n = 0…( ( nS >> 2 ) * ( nS >> 2 ) ) – 1 )

(8-3)