| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F199 |
| --- | --- |

| Title: | A further improvement of inter prediction direction and reference frame index combined coding in CAVLC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Houqiang Li |  |  |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

This document presents improvement on the combined coding of inter prediction direction and reference frame index in CAVLC. The experimental results show that the BD-Rate difference is negligible under the default test configuration. But when the number of reference frames is reduced, about 0.2% bits saving were obtained on average. Another item proposed in this contribution is that to move the adaptive algorithm to decide using combined coding or not, from CU to PU, to make it more nature. Changing the adaptation algorithm from CU to PU will not have much impact on the performance.

# Introduction

The improvement of [2] was proposed in [1] and was adopted in Geneva meeting. But there are still some small mistakes on the calculation of uiMaxVal (in the software) or MaxPredRef (in the WD). This contribution aims to fix that small bug. Another item proposed in this contribution is that to move the adaptive algorithm to decide using combined coding or not, from CU to PU, to make it more nature. Changing the adaptation algorithm from PU to CU will not have great impact on the performance.

- Solution

The variables NumPredRefLC, NumPredRefL0 and NumPredRefL1 are specified as

NumPredRefLC = Min( 4, num_ref_idx_lc_active_minus1 + 1 ) 
NumPredRefL0 = Min( 2, num_ref_idx_l0_active_minus1 + 1 ) 
NumPredRefL1 = Min( 2, num_ref_idx_l1_active_minus1 + 1 )

The variable MaxPredRef is specified as

MaxPredRef = NumPredRefLC + NumPredRefL0 * NumPredRefL1

The maximum value (MaxPredRef) represents the exception. But if the number of the reference frame index does not exceed the range, the exception is not needed to be represented. In other words, if
      num_ref_idx_lc_active_minus1 + 1 <= 4 && 
      num_ref_idx_l0_active_minus1 + 1 <= 2 &&
      num_ref_idx_l1_active_minus1 + 1 <= 2

the exception is not needed, and the MaxPredRef can be decreased by 1. When it is decreased by 1, one bit can be saved when encoding the last element.

- Experimental results

The proposed algorithm was implemented in HM-3.0. RA_LC and LD_LC specified in [3] were tested. Under the default test configuration, the performance is almost the same as anchor. And when setting the number of reference frames to 1, 0.2% bits saving were obtained on average. The summary results are shown in Table 1 and 2. The detailed results are in JCTVC-F199_Fix.xls and JCTVC-F199_Fix_1Ref.xls.

Table 1 Summary result of default test configuration (Y BD-rate)

|  | RA_LC | LD_LC |
| --- | --- | --- |
| Class A | 0.0 |  |
| Class B | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 |
| Class D | 0.0 | -0.1 |
| Class E |  | 0.0 |
| All | 0.0 | 0.0 |
| Enc Time | 100% | 101% |
| Dec Time | 101% | 101% |

Table 2 Summary result of one reference frame (Y BD-rate)

|  | RA_LC | LD_LC |
| --- | --- | --- |
| Class A | -0.2 |  |
| Class B | -0.2 | -0.2 |
| Class C | -0.3 | -0.1 |
| Class D | -0.3 | -0.2 |
| Class E |  | -0.1 |
| All | -0.2 | -0.2 |
| Enc Time | 101% | 101% |
| Dec Time | 101% | 101% |

The summary result of moving the adaptation algorithm from PU to CU is shown in Table 3. And Table 4 shows the summary result of the combination of two proposed items. The detailed results can be found in JCTVC-F199_PU.xls and JCTVC-F199_Fix+PU.xls.

Table 3 Summary result of PU adaptation algorithm (Y BD-rate)

|  | RA_LC | LD_LC |
| --- | --- | --- |
| Class A | 0.0 |  |
| Class B | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 |
| Class D | 0.0 | -0.1 |
| Class E |  | 0.1 |
| All | 0.0 | 0.0 |
| Enc Time | 102% | 100% |
| Dec Time | 99% | 99% |

Table 4 Summary result of the combination (Y BD-rate)

|  | RA_LC | LD_LC |
| --- | --- | --- |
| Class A | 0.0 |  |
| Class B | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 |
| Class E |  | 0.0 |
| All | 0.0 | 0.0 |
| Enc Time | 102% | 100% |
| Dec Time | 98% | 98% |

- Text Modification

The text modification is based on JCTVC-E603_d8, and should be further improved.

In 7.4.7

combined_inter_pred_ref_idx specifies the values of inter_pred_flag and ref_idx_lX with X being 0, 1 or c. When it is not present, it shall be inferred to be equal to 0. [Ed. (WJ): needs to be improved]

The variables NumPredRefLC, NumPredRefL0 and NumPredRefL1 are specified as

NumPredRefLC = Min( 4, num_ref_idx_lc_active_minus1 + 1 )	(7-1)
NumPredRefL0 = Min( 2, num_ref_idx_l0_active_minus1 + 1 )	(7-2)
NumPredRefL1 = Min( 2, num_ref_idx_l1_active_minus1 + 1 )	(7-3)

The variable MaxPredRef is specified as

- If num_ref_idx_lc_active_minus1 is greater than 0, the following applies.

MaxPredRef = NumPredRefLC + NumPredRefL0 * NumPredRefL1	(7-4)

- Otherwise (combined list is not used), the following applies.

MaxPredRef = NumPredRefL0 + NumPredRefL0 * NumPredRefL1	(7-5)

If ( num_ref_idx_lc_active_minus1 + 1 > 4 ||

num_ref_idx_l0_active_minus1 + 1 > 2 ||

num_ref_idx_l1_active_minus1 + 1 > 2 )
			NeedCodeRefException = true;

Otherwise,
NeedCodeRefException = false;
MaxPredRef--;

……

ref_idx_l0_minusX[ x0 ][ y0 ] + X specifies the value of ref_idx_l0[ x0 ][ y0 ] when combined_inter_pred_ref_idx is present and it is equal to MaxPredRef and NeedCodeRefException is true.

The constant X is derived as follows:

- If inter_pred_flag[ x0 ][ y0 ] is Pred_L0, X is set equal to 2,

Otherwise, X is set equal to 0.

ref_idx_l1_minusX[ x0 ][ y0 ] + X specifies the value of ref_idx_l1[ x0 ][ y0 ] when combined_inter_pred_ref_idx is present and it is equal to MaxPredRef and NeedCodeRefException is true.

The constant X is derived as follows:

- If ref_idx_l0[ x0 ][ y0 ] is less than 2, X is set equal to 2,

Otherwise, X is set equal to 0.

ref_idx_lc_minus4[ x0 ][ y0 ] + 4 specifies the value of ref_idx_lc [ x0 ][ y0 ] when combined_inter_pred_ref_idx is present and it is equal to MaxPredRef and NeedCodeRefException is true.

In 7.3.7

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx , InferredMergeFlag ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( NumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && log2PUWidth >= Log2IPCMCUSize ) |  |
| pcm_flag | u(1) ¦ ae(v) |
| if( pcm_flag ) { |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| if( NumMPMCand > 1 ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| if( IntraPredMode[ x0 ][ y0 ] == 2 ) |  |
| planar_flag_luma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| if( IntraPredMode[ x0 ][ y0 ] != 2 && IntraPredMode[ x0 ][ y0 ]!=34 && SignaledAsChromaDC ) |  |
| planar_flag_chroma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } |  |
| } else { /* MODE_INTER */ |  |
| if( !InferredMergeFlag ) |  |
| if( entropy_coding_mode_flag ¦¦ PartMode != PART_2Nx2N ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] && NumMergeCand > 1 ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| if( slice_type = = B ) { |  |
| if(!entropy_coding_mode_flag && use_combined_inter_pred_ref( x0, y0 ) ) { |  |
| combined_inter_pred_ref_idx | ue(v) |
| if( combined_inter_pred_ref_idx == MaxPredRef && NeedCodeRefException ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) |
| } else |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag && use_combined_inter_pred_ref( x0, y0 ) ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef && NeedCodeRefException) |  |
| ref_idx_lc_minus4[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_lc[ x0 ][ y0 ] | ae(v) |
| } |  |
| mvd_lc[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( LcToLx ) > 1 ) |  |
| mvp_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag && use_combined_inter_pred_ref( x0, y0 ) ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef && NeedCodeRefException) |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag && use_combined_inter_pred_ref( x0, y0 ) ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef && NeedCodeRefException) |  |
| ref_idx_l1_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Moving the adaptation algorithm from PU to CU will not change the current WD, as in WD3_d8, the adaptation algorithm is PU based.

- References

- JVT-VC, “Improvement of inter prediction direction and reference frame index combined coding in LCEC”, JCTVC-E381, JCT-VC Meeting, Geneva, CH, March, 2011.

- JCT-VC, “Improvement and extension of inter prediction direction and reference frame index combined coding in LCEC”, JCTVC-D141, JCT-VC Meeting, Daegu, KR, January, 2011

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-E700, JCT-VC Meeting, Geneva, CH, March, 2011.

# Patent rights declaration(s)

University of Science and Technology of China, and Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).