| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F200 |
| --- | --- |

| Title: | Improvements of the BCIM mode for screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cuiling Lan Jizheng Xu Gary J. Sullivan Feng Wu |  |  |
| Source: | Xidian University Microsoft |  |  |

_____________________________

# Abstract

This document presents improvements of BCIM (Base Color and Index Map) mode to compress screen contents. It further improves the coding efficiency for screen contents. Moreover, the improved mode is an alternative method to provide an upper bound for the coded bits, avoiding the unexpected large bit number which is prohibitively greater than that of raw data. Experimental results of implementing BCIM in HM3.0 for screen content sequences under the high efficiency intra coding configuration are shown in this document. For the typical screen content sequences, the proposed BCIM mode achieves 38% bit saving on average. For all the tested screen sequences, it achieves 21.6 % bit saving on average. Experimental results over the synthesized noise sequence demonstrate that BCIM can provide an upper bund for the output bits.

# Introduction

Screen contents refer to images/videos generated/rendered by computers or some other electronic devices, e.g., mobile phone. As these contents are widely used, it becomes important to record, store and transmit them. To handle well these contents can also help many applications and even enable new application scenarios. For example, some application scenarios are listed below.

- Remote desktop

- Desktop sharing

- Video conferencing

- Game video

- Demo recording

- Remote education

- Gaming on demand, e.g., onlive.com

- …

Thus far in the HEVC project, the focus has been on video content that was generated by video cameras, although it is clear that screen content is also an important video source. We would like to point out that that the HEVC requirements [1][2] apply to video coding in general for a wide range of applications – not just video from camera sources. We suggest a greater focus on screen content in the further work on the HEVC project.

Unlike natural images/video, screen contents may not be very smooth. They usually have totally different statistics. For text or graphics contents, it is much sharper with high contrast. Because of the high contrast, any little artifact may be perceived by users. Thus, coding for screen contents usually require a very high fidelity of the decoded video. Besides, transform based coding approaches are usually inefficiency on compression these contents. It’s imperative to add new tools to HEVC to handle screen contents.

In JVTVC-B084 [3] and JVTVC-C276 [4], we have showed that by integrating two new modes into AVC, the coding performance for screen content can be much improved. In JVTVC-E145 [5], besides those two modes, a lossless dictionary based compression mode using LZMA is implemented into HM framework. In addition, three inter coding tools are proposed to improve the coding efficiency of screen videos, which can be seen as an extension of the two modes in [4]. In this document, we propose to use BCIM mode to compress screen contents. This mode is implemented to both luma and chroma planes in HM3.0. For its compact representation manner without transform, it can serve as a tool that confines the output bit number. The coding tool will be introduced in Section 2 and the experimental results are shown in Section 3.

# Base Color and Index Map (BCIM) coding tool

## Coding method

Having limited colors but complicated shapes is a property of the text and graphics parts for screen contents. Such text/graphics blocks can be expressed concisely by several base colors together with an index map. It is somewhat like color quantization that is a process of choosing a representative set of colors to approximate all the colors of an image [6]. In the BCIM mode, we ﬁrst get the base colors of a block using a clustering algorithm. All the base colors constitute a base color table. Then, each sample in the block will be quantized to its nearest base color. The index map indicates which base color is used by each sample. Different from color quantization, each text/graphics block, but not an entire image, has its own base colors and an index map for representation in our scheme. The maximum allowable base color number K is predefined, e.g. K=16. The best base color number for the coding block is adaptively decided by rate-distortion optimization for the possible number from 1 to 16. Thus, it is content adaptive for each block.

Fig. 1. The neighboring indexes as contexts of the current index.

In this mode, there are several key problems to solve. First, base colors should be chosen to well represent a block. Second, after base colors are selected, there should be an efficient entropy coding method to code the base colors and index map. If the coding method has been decided and is good enough, the problem is then simpliﬁed as the searching of the best base colors for that block. In this mode, we use dynamic programming to not only achieve the global optimal partition but also avoid intensive computation caused by clustering of multiple times. The best base color number is decided in the rate-distortion sense. To represent a block, we need to encode the base colors and index map into the bitstream. Instead of using the binarization schemes in AVC, such as Exp-Golomb, the base color number, values and remapped indexes are all simply expressed by their binary format. The context setting for an index coding is shown in Fig. 1 where the neighboring four indexes are taken as the context. In such a case, there are 15 basic patterns of the neighboring indexes, as {AAAA, AAAB, AABA, ABAA, BAAA, AABB, ABAB, ABBA, AABC, BAAC, BCAA, ABAC, ABCA, BACA, ABCD}, where A, B, C and D stand for indexes that have been sorted by their occurrences. Thus, there will be 15 contexts for coding the current index. We also perform a remapping of the current index based on the neighboring indexes before the entropy encoding. More details can be found in [7].

## Upper bound of the output bit number

Fig. 2. The relationship between base color number (=) and average bit number

For a coding block, the total output bits in BCIM consist of the bits for representing base color number, base color values and the index of each sample. Assuming that the bit depth of each sample is 8, the total bits for a block withsamples is given as

(1)

Here,represents the bits to represent each index without entropy coding.indicates the actual used base color number, where <. Each base color is represented by 8 bits. Then the average bit number for each sample is given as

(2)

To facilitate the analysis, we assume. The relationship betweenandis illustrated in Fig. 2. Firstly, we can see that if the base color number is small enough, even without entropy coding, the required average bit number will be small than the bit depth of the original sample, achieving compression. Secondly, if the base color number is equal to the color number in a block, we can get the lossless representation of the coding block and the maximum average bit number will not exceed a certain value. For example, over a 4x4 block, when the base color numberis less than ≈ 9.1217, BCIM mode can take fewer bits than the raw data to represent the coding block. At the worst case when the color number is 16 for a 4x4 block and we give a lossless representation using 16 base colors (4 bits for each index), the average bit number is then 12.25. This is viewed as an upper bund of bit number. Similarly, for an 8x8 block, at the worst case when the color number is 64 and we give a lossless representation using 64 base colors (6 bits for each index), the upper bound for the average bit number is about 14.

We can see that BCIM mode plays similar role as PCM (Pulse Code Modulation) described in [8] even though the bund is higher than that given by PCM. However, for a block having the color number smaller than some value, the average bit will be smaller than 8. Moreover, the adaptive determination of base color number usually brings better rate-distortion performance, yielding bits much less than the given upper bund value. This will be indicated by the experimental results over the synthesized sequence in next Section.

# Experimental results

We have implemented the BCIM mode into HM3.0 software to evaluate the compression performance. We add this mode on the PU for luma and chroma color planes separately, with blocks sizes ranging from 32x32 to 4x4 for the luma plane. For CU with size ranging from 8x8 to 32x32, this mode works on PART_2Nx2N partition PU with the transform depth being zero. For the 8x8 CU, to enable this mode work on 4x4 block, this mode is also performed on PART_NxN partition with transform depth being zero. It reuses the DC prediction direction. A flag is coded to distinguish it from the original mode in HM3.0 when necessary.

The maximum allowable base color number K is set to be 4 in previous proposals [3][4][5]. However, at high bit rates, for screen content having more flexible colors, 4 base colors will not be enough to provide a close approximation. Thus, we extend the maximum base color number to 8, 16 and 32 respectively to test their coding performance. Over five screen images, we evaluate its coding performance under the intra configuration. We set the Qp from 47 to 7 with step of 10 and disable the deblocking filter. We show their coding performance in Fig. 3 - Fig. 7. The curve marked by HM shows the coding performance of HM3.0. The coding results after implementing BCIM mode with the maximum allowable base color number K being 4, 8 and 16 are marked by HM+BCIM4c, HM+BCIM8c, HM+BCIM16c respectively. We can see that the coding performance with larger allowable maximum base color number is significantly better than that with maximum base color number of 4 especially at middle and high bit rate for some screen images. But for images with pure text/graphics colors, similar performance is achieved since a very few colors are sufficient to well represent the coding content. It is noted that larger maximum base color number than 16 gives comparable performance over the test images and we have not shown them in the curves. Therefore, we set the maximum base color number as 16 hereafter. The best base color number for each coding block is adaptively decided by rate-distortion optimization.

To evaluate the coding performance of BCIM over various screen contents, we take the screen content sequences submitted in 5th JCTVC meeting as the test sequences. We take three groups of sequence for the investigation. Three sequences (Doc, Web and Slide of the version 2) from BJUT show the document editing, web browsing and slide showing respectively. Three sequences from HKUST (from the folder of new1_SCC_SEQUENCES_HKUST) show the document browsing and editing, web browsing, and circuit board editing. Two sequences from HUAWEI (from the folder JCTVC-EXXX-HUAWEI-SCC-SEQUENCES) show the doc editing and window switching which simultaneously render natural video contents. Another seven sequences form HUAWEI (from the folder new1_JCTVC-EXXX-HUAWEI-SCC-SEQUENCES-PART-1A and new1_CactusText_1920x1080_50_yuv_for_review_by_RAI) show the HEVC sequences with subtitles padded at the bottom of the screen.

We test these sequences under the high efficiency intra coding configuration. Qp is set to be 22, 27, 32 and 37. Deblocking filter is disabled. The BD-Rate results are shown in Table I. We can see that HM + BCIM16c achieves 38% bit saving on average for the first eight test sequences and 21.6% bit saving on average for all the test sequences. For the last seven sequences from HUAWEI, the gain is small since there is only a small region of text content over each frame. The decoding speed (being 86.4% of HM3.0 for the first eight sequences) is a little fast than HM since no transform is required for BCIM mode and the reconstruction is just to look up decoded base color table. The encoding time is about 1.77 times that of HM3.0 without taking any optimization and fast algorithm. The maximum base color number is set to be 16 in the experiments. Large allowable base color number means more times of checking the rate-distortion costs at different base color numbers, resulting in increased computational complexity. Besides, for a block having many colors, most likely, it is not a text/graphics block. Then, the checking of this mode can be skipped to save the encoding time. This will greatly reduce the complexity especially for video camera generated sequences. By considering these, the encoding complexity can be reduced in the further work.

Table I:  BD-rate results for HM+BCIM16c

| Intra |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-psnr | U BD-psnr | V BD-psnr |
| doc_1280x720_500_10 | -36.62 | -30.43 | -30.49 | 7.42 | 3.71 | 3.61 |
| web_1280x720_500_10 | -48.02 | -38.73 | -39.99 | 11.27 | 5.36 | 5.27 |
| slide_1280x720_500_20 | -16.88 | -13.49 | -13.80 | 1.64 | 1.07 | 1.06 |
| new1_HKUST1_1280x720_30_420 | -51.99 | -45.79 | -45.99 | 10.82 | 6.69 | 6.74 |
| new1_HKUST2_1280x720_30_420 | -46.40 | -38.57 | -38.83 | 8.00 | 4.88 | 5.11 |
| new1_HKUST3_1280x720_30_420 | -50.47 | -35.66 | -37.56 | 10.37 | 4.68 | 5.54 |
| ScreenCaptureDocEditing_1280x720_60 | -27.62 | -24.44 | -24.81 | 3.27 | 1.85 | 1.88 |
| ScreenCaptureWinSwitching_1280x720_60 | -25.20 | -21.67 | -22.13 | 3.10 | 1.81 | 1.80 |
| new1_BQTerraceText_1920x1080_60 | -1.73 | -2.81 | -4.18 | 0.10 | 0.09 | 0.12 |
| new1_BasketballDriveText_1920x1080_50 | -3.93 | -4.61 | -4.76 | 0.13 | 0.15 | 0.19 |
| new1_CactusText_1920x1080_50 | -2.87 | -3.18 | -3.63 | 0.11 | 0.09 | 0.14 |
| new1_BasketballDrillText_832x480_50 | -3.92 | -3.81 | -4.05 | 0.22 | 0.19 | 0.22 |
| new1_BQMallText832x480_60 | -3.73 | -3.64 | -4.24 | 0.24 | 0.20 | 0.25 |
| new1_PartySceneText_832x480_50 | -1.96 | -2.05 | -2.38 | 0.15 | 0.11 | 0.13 |
| new1_RaceHorsesText_832x480_30 | -3.06 | -2.06 | -2.64 | 0.23 | 0.13 | 0.18 |
| BJUT(Vertion2) (1th-3th sequences) | -33.84 | -27.55 | -28.09 | 6.78 | 3.38 | 3.31 |
| HKUST(new1) (4th-6th sequences) | -49.62 | -40.01 | -40.79 | 9.73 | 5.42 | 5.80 |
| HUAWEI (7th-8th sequences) | -26.41 | -23.05 | -23.47 | 3.18 | 1.83 | 1.84 |
| HUAWEI (9th-15th sequences) | -3.03 | -3.17 | -3.62 | 0.17 | 0.14 | 0.19 |
| 1th-8th sequences | -37.90 | -31.10 | -31.70 | 6.99 | 3.76 | 3.88 |
| All | -21.63 | -18.06 | -18.63 | 3.81 | 2.07 | 2.15 |
| Enc Time[%] | 1.77 |  |  |  |  |  |
| Dec Time[%] | 0.94 |  |  |  |  |  |

To demonstrate the role of providing a bit number bound of BCIM, a synthesized sequence Sandstorms which is Gaussian noise that generated by JCTVC-E057-NEC-Sandstorms-Software [8] is tested. Its first frame is shown in Fig. 8. Table II shows the intra coding results for Sandstorms at Qp 0, 2, 7, 12, 17, 22, 27, 32 and 37. “Com. Ratio” is the ratio between the output bitstream size and the data size of input video (smaller value means better compression performance). The rate distortion curve is shown in Fig. 8. We can see that for this sequence, at Qp 0, the bit rate of HM + BCIM16c is 7.97bpp, much less than 12 bpp. And it is far fewer than the upper bound of 18.375 bpp (12.25x1.5).

If no entropy coding is adopted in the BCIM mode, as shown by the curve marked HM+BCIM16c (no entropy), the bit rate is increased about 1.3bpp in comparison with HM + BCIM16c for the test sequence Sandstorms. With the purpose of evaluation, we use the equal probability arithmetic coding to simulate the case of no entropy for simplicity. In the HM + BCIM15c (no entropy, sameflag) method, if the coding index is the same with previous one, a flag is coded without coding the index value. It gives similar performance to HM+BCIM16c (no entropy). Without entropy coding, the upper bound for output bits is the result analyzed in Section 2.2. In general, taking entropy coding can achieve a better compression giving average bit number lower than the bound.

The coding performance of BCIM without entropy coding is also evaluated on several screen content images. The rate-distortion curves are shown in Fig. 9. We can see that the coding performance is obviously dropped without using entropy coding. However, it still outperforms HM3.0.

Table II: BCIM results on synthesized noisy sequence at the high efficiency intra configuration

| Sandstorms | HM3.0 |  |  |  |  | HM3.0 + BCIM16c |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qp | kbps | PSNR Y | Enc T[s] | Dec T[s] | Com. Ratio | kbps | PSNR Y | Enc T[s] | Dec T[s] | Com. Ratio |
| 0 | 88360 | 58.8 | 159.9 | 5.242 | 2.4211 | 24235 | 74.324 | 231.92 | 2.153 | 0.66405 |
| 2 | 80793 | 58.253 | 151.7 | 4.899 | 2.2138 | 24155 | 71.962 | 231.22 | 2.091 | 0.66187 |
| 7 | 62572 | 56.233 | 134.96 | 4.072 | 1.7145 | 23884 | 67.113 | 219.64 | 2.137 | 0.65444 |
| 12 | 45882 | 53.565 | 119.56 | 3.385 | 1.2572 | 23476 | 60.836 | 202.06 | 2.074 | 0.64325 |
| 17 | 34747 | 48.289 | 111.84 | 2.777 | 0.9521 | 21651 | 51.873 | 164.56 | 1.965 | 0.59326 |
| 22 | 29142 | 44.12 | 105.15 | 2.512 | 0.7985 | 14501 | 40.679 | 158.7 | 1.388 | 0.39733 |
| 27 | 25054 | 39.458 | 100.86 | 2.246 | 0.6865 | 14462 | 40.636 | 156.18 | 1.389 | 0.39626 |
| 32 | 20931 | 34.978 | 91.653 | 1.996 | 0.5735 | 11920 | 33.635 | 143.93 | 1.201 | 0.32662 |
| 37 | 17070 | 29.833 | 86.614 | 1.81 | 0.4677 | 9091.4 | 26.824 | 139.17 | 1.03 | 0.24911 |

(a )				     	                   (b)

Fig. 3. (a) screen image Files, and (b) coding performance comparison.

(a )				     	        (b)

Fig. 4. (a) screen image Mix, and (b) coding performance comparison.

(a)					        (b)

Fig. 5. (a) screen image Webpage, and (b) coding performance comparison.

(a)				                	        (b)

Fig. 6. (a) screen image Slide, and (b) coding performance comparison.

(a)					             (b)

Fig. 7. (a) screen image Word, and (b) coding performance comparison.

(a )				     	        (b)

Fig. 8. (a) Synthesized sequence, Sandstorms, (CIF 30 Hz 30 frames) [8], and (b) coding performance comparison.

Fig. 9. Evaluation of BCIM without entropy coding.

# Conclusions

In this document, we have shown results of applying the BCIM intra coding tool to HM3.0 software over screen content sequences. The coding performance is significantly improved for these sequences, achieving 21.6% bit saving on average. Moreover, BCIM can serve as an efficient tool to confine the output bit number.

# References

[1] “Draft Requirements for next-generation video coding project,” ITU-T SG16/Q6/VCEG-AL96, London, UK/Geneva, CH, July 2009.

[2] “Vision, Applications and Requirements for High-Performance Video Coding (HVC),” ISO/IEC JTC1/SC29/WG11/N11096, Kyoto, JP, Jan. 2010.

[3] C. Lan, J. Xu, F. Wu and G. Sullivan, “Screen content coding,”  JCT-VC Doc JCT-B084, Geneva, CH, July, 2010.

[4] C. Lan, J. Xu and F. Wu, “Screen content coding results using TMuC,” JCT-VC Doc JCTVC-C276, Guangzhou, CN, Oct. 2010.

[5] C. Lan, X. Peng, J. Xu and F. Wu, “Screen content coding using HM,” JCT-VC Doc JCT-E145, Geneva, CH, March, 2011.

[6] X. Wu, “Color quantization by dynamic programming and principal analysis,” ACM Trans. Graph., vol. 11, no. 4, pp. 348–372, Oct. 1992.

[7] C. Lan, G. Shi and F. Wu, “Compress compound images in H.264/MPEG-4 AVC by exploiting spatial correlation,” IEEE Trans. on Image Processing, Apr., 2010.

[8] K. Chono, H. Aoki and Y. Senda, “Pluse code modulation mode for HEVC,” JCT-VC Doc JCT-E057, Geneva, CH, March, 2011.

# Patent rights declaration(s)

Xidian University, and Microsoft Corp. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

### Appendix: Proposed syntax and semantics

### (The modified portion is highlighted with red colour.)

### Change in transform tree syntax

| transform_tree( x0, y0, log2TrafoSize, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && trafoDepth = = 0 && IntraSplitFlag = = 0) { |  |
| if( PredMode != MODE_INTRA ) |  |
| no_residual_data_flag | u(1) ¦ ae(v) |
| residualDataPresentFlag = !no_residual_data_flag |  |
| } else { |  |
| residualDataPresentFlag = TRUE |  |
| } |  |
| if( residualDataPresentFlag) { |  |
| intraSplitFlag = ( IntraSplitFlag && trafoDepth = = 0 ? 1 : 0 ) |  |
| maxDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| if( !entropy_coding_mode_flag ) { |  |
| xB = x0 − ( x0 & ( ( 1 << log2TrafoSize ) − 1 ) ) |  |
| yB = y0 − ( y0 & ( ( 1 << log2TrafoSize ) − 1 ) ) |  |
| if( trafoDepth != 0 && !cbf_luma[ xB ][ yB ][ trafoDepth − 1 ] ) { |  |
| if( cbf_cb[ xB ][ yB ][ trafoDepth − 1 ] && log2TrafoSize > Log2MinTrafoSize ) |  |
| cbf_cb[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| if( cbf_cr[ xB ][ yB ][ trafoDepth − 1 ] && log2TrafoSize > Log2MinTrafoSize ) |  |
| cbf_cr[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| if(log2TrafoSize <= Log2MaxTrafoSize && !intraSplitFlag && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth ) |  |
| split_transform_flag[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| } else |  |
| cbp_and_split_transform | vlc(n,v) |
| } |  |
| if( log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth && !intraSplitFlag && entropy_coding_mode_flag) |  |
| split_transform_flag[ x0 ][ y0 ][ trafoDepth ] | u(1) ¦ ae(v) |
| if( PredMode != MODE_INTRA && log2TrafoSize <= Log2MaxTrafoSize && entropy_coding_mode_flag ) { |  |
| firstChromaCbf = ( log2TrafoSize = = Log2MaxTrafoSize ¦ ¦ trafoDepth = = 0 ? 1 : 0 ) |  |
| if( firstChromaCbf ¦ ¦ log2TrafoSize > Log2MinTrafoSize ) { |  |
| xBase = x0 − ( x0 & ( ( 1 << log2TrafoSize ) − 1 ) ) |  |
| yBase = y0 − ( y0 & ( ( 1 << log2TrafoSize ) − 1 ) ) |  |
| if( firstChromaCbf ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | u(1) ¦ ae(v) |
| if( firstChromaCbf ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | u(1) ¦ ae(v) |
| } |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| transform_tree( x0, y0, log2TrafoSize − 1, trafoDepth + 1, 0 ) |  |
| transform_tree( x1, y0, log2TrafoSize − 1, trafoDepth + 1, 1 ) |  |
| transform_tree( x0, y1, log2TrafoSize − 1, trafoDepth + 1, 2 ) |  |
| transform_tree( x1, y1, log2TrafoSize − 1, trafoDepth + 1, 3 ) |  |
| } else if( entropy_coding_mode_flag ){ |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] | u(1) ¦ ae(v) |
| if( PredMode = = MODE_INTRA ) |  |
| if( log2TrafoSize > Log2MinTrafoSize ) { |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | u(1) ¦ ae(v) |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | u(1) ¦ ae(v) |
| } else if( blkIdx = = 0 ) { |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth − 1 ] | u(1) ¦ ae(v) |
| cbf_cr[ x0 ][ y0 ][ trafoDepth − 1 ] | u(1) ¦ ae(v) |
| } |  |
| if(BCIM_enabled_flag && PredMode = = MODE_INTRA){ |  |
| if(IntraPredMode[ x0 ][ y0 ] == 2 && cbf_luma[ x0 ][ y0 ][ trafoDepth ]){ |  |
| trDepth = trafoDepth – cuDepth |  |
| checkLumaBCIM = log2CUSize == Log2MinCUSize ? ((PartMode == PART_2Nx2N && trDepth == 0) ¦¦ (PartMode == PART_NxN && trDepth == 1)): (trDepth == 0) |  |
| if(checkLumaBCIM){ |  |
| BCIM_flag_luma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } |  |
| if((log2TrafoSize > Log2MinTrafoSize ¦¦ blkIdx == 0) && cbf_cb[ x0 ][ y0 ][ trafoDepth ] && cbf_cr[ x0 ][ y0 ][ trafoDepth ]){ |  |
| checkChromaBCIM = intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ¦¦ (intra_chroma_pred_mode[ x0 ][ y0 ] == 4 && IntraPredMode[ x0 ][ y0 ] == 2) |  |
| If(checkChromaBCIM){ |  |
| BCIM_flag_chroma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

BCIM_flag_luma[x0][y0] specifies whether the luma block shall be coded by BCIM method when IntraPredMode[x0][y0] is equal to 2 and planar_flag_luma is equal to 0.

BCIM_flag_chroma[x0][y0] specifies whether the chorma block shall be coded by BCIM method when intra_chroma_pred_mode[x0][y0] is equal to 2 and planar_flag_chroma is equal to 0.

### Change in transform coefficient syntax

| transform_coeff( x0, y0, log2TrafoSize, trafoDepth, cIdx ) { | Descriptor |
| --- | --- |
| if( ( ( cIdx = = 0 && cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 1 && cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 2 && cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) ) ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | se(v) ¦ ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| transform_coeff( x0, y0, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y0, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x0, y1, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y1, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| } else { |  |
| if ( PredMode = = MODE_INTRA ) { |  |
| if (cIdx = = 0) |  |
| scanIdx = ScanType[ log2TrafoSize -2][ IntraPredMode ] |  |
| else |  |
| scanIdx = ScanTypeC[ log2TrafoSize -2][ IntraPredModeC ] |  |
| } else { |  |
| scanIdx = 0 |  |
| } |  |
| if ( entropy_coding_mode_flag ){ |  |
| if(BCIM_enabled_flag){ |  |
| if((cIdx = = 0 && BCIM_flag_luma [ x0 ][ y0 ]) ¦¦ (cIdx = = 1 && BCIM_flag_chroma[x0][y0])){ |  |
| block_coding_BCIM_cabac(x0, y0, log2TrafoSize, trafoDepth, cIdx) |  |
| }else{ |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |
| } |  |
| }else{ |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |
| } |  |
| } |  |
| else if ( !entropy_coding_mode_flag ) |  |
| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |
| } |  |
| } |  |
| } |  |

### Block coding using BCIM CABAC syntax

| block_coding_BCIM_cabac(x0, y0, log2TrafoSize, trafoDepth, cIdx) { | Descriptor |
| --- | --- |
| base_color_number_minus1 | ae(v) |
| base_color_number = base_color_number_minus1 + 1 |  |
| for(n = 0; n < base_color_number; n ++){ |  |
| base_color_value[n] | ae(v) |
| base_color_value[n] <<= (BitDepth – 8) |  |
| } |  |
| if(base_color_number > 1){ |  |
| index_map (x0, y0, log2TrafoSize,cIdx) |  |
| }else{ |  |
| fill_index_zeros(x0, y0, log2TrafoSize,cIdx) |  |
| } |  |
| } |  |

### Index map coding CABAC syntax

| index_map (x0, y0, log2TrafoSize, cIdx) { | Descriptor |
| --- | --- |
| blkSize = (1 << ( log2TrafoSize )) >> ( cIdx > 0 ? 1 : 0 ) | ae(v) |
| for (yB = 0; yB < blkSize; yB ++){ |  |
| for (xB = 0; xB < blkSize; xB ++){ | ae(v) |
| left_index = index_value[yB][xB-1] |  |
| leftup_index = index_value[yB-1][xB-1] |  |
| up_index = index_value[yB-1][xB] |  |
| rightup_index = index_value[yB-1][xB+1] |  |
| context = get_index_context(left_index, leftup_index, up_index, rightup_index) |  |
| remapped_index_value[yB][xB] |  |
| Index_value[yB][xB] = reverse_remap_index(remapped_index_value[yB][xB], context) |  |
| } |  |
| } |  |
| } |  |

base_color_number_minus1 is the number of base colors minus 1 for the BCIM coding block.

- The value of base_color_number_minus1 is constrained by the predefined maximum allowabled base color number K (base_color_number_minus1 <= K-1 && base_color_number_minus1 <= (1<< (log2TrafoSize << 1) - 1))

base_color_value specifies the values of base colors.

remapped_index_value specifies the remapped index for a sample. It is an already remapped value of the index value based on neighboring indexes with the purpose of reducing contexts.

- It is a value in the range from 0 to K-1, where K is the maximum allowable base color number.

get_index_context specifies the process to get the context for the coding of index value.

reverse_remap_index specifies the real index value deduced from the context and remapped index.

Note: For the neighboring index which is not available, we fill it by K, where K is the maximum allowable base color number.

#### Reconstruction in BCIM by looking up table

Inputs of this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current block,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable nS specifying the size of the current block,

Output of this process is a modified (nS)x(nS) array resSample specifying the samples of the current reconstructed block

The sample location ( xBN, yBN ) is specified by

xBN = xB + x                                                             (8-1)

yBN = yB +y                                                              (8-2)

The modified array resample is derived as follows.

resSample[xB+i, yB+j] = base_color_value[translated_index[i][j]], with i, j = 0,..,nS-1  (8-68)