| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F201 |
| --- | --- |

| Title: | High-level Syntax: Temporal Information Decoding Refresh |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Houqiang Li. |  |  |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

Temporal MVP brings bits saving to HEVC as well as breaks the parsing robustness. The parsing problem has been analyzed in many proposals. This contribution presents an algorithm to provide correct MV at some recovery point as a tradeoff between coding efficiency and MV accessibility.

# Introduction

From the age of TMuC, TMVP brings performance gain as well as parsing problem [1]. This contribution presents a tradeoff method between the coding efficiency and MV accessibility.

- Solution

The propagation of wrong MV will not be stopped until the next IDR or CDR frame. The wrong MV will lead to the failure of entropy decoding. The parsing problem can be solved by

- Disabling TMVP in AMVP and merge mode, which will lead to significant performance loss.

- Code full index, the maximum number of MVP index and merge index does not depend on the value of MV. Although this method can solve the parsing problem for all the frames, the wrong MV will lead to terrible reconstruction result as shown in [1].

The second method can only guarantee the parsing process not fails and the MVd is correct. But when MVp is not correct, the MV will be wrong too. In many cases, correct MV is useful for error concealment when packet loss exists. Although the second method can solve the parsing issue, the MV is still wrong. This contribution proposes to add a new type of access unit (or slice) to truncate the propagation of wrong MV caused by TMVP. We call it temporal information decoding refresh (TIDR). If this kind of slices were inserted into the bit-stream, the error propagation will be truncated at these slices. Thus, the entropy decoding process will not fail after that slice. TIDR is quite similar to IDR or CDR in the current HM. IDR and CDR are used to recover from the wrong texture and to random access the texture. And TIDR is used to recover from the wrong temporal information (such as temporal MVP) and to random access the correct MV. The definition of TIDR picture in defined in the next section.

The entropy decoding after TIDR picture will be successful, and the motion information after TIDR picture will be correct.

It should also be noticed that only inserting frames without TMVP periodically can also provide correct MV after the frames without TMVP. But inserting frames without TMVP is only suitable for the simple coding structure. For example in figure 1, one no TMVP frame is inserted every eight frames. If one frame, frame l, before the frame 8n is lost, the frames between l and 8n-1 (inclusive) will not be successfully entropy decoded because of the frame loss. In figure (a), where regular prediction structure is adopted, frame 8n will be successfully entropy decoded because it is no TMVP frame. The following frame, frame 8n+1, will also be successfully entropy, because the temporal MVP for frame 8n+1 only comes from frame 8n, in which all the motion information is correct. In this regular prediction case, if one frame before frame 8n is lost, all the frames starts from frame 8n will not be affected. So frame 8n, which is no TMVP frame, can be considered as entropy decoding recovery point in the bit-stream. But for the coding structure in figure (b), some different happened. Also, frame l is lost, and the frames before frame 8n will not be successfully entropy decoded. The frame 8n which is no TMVP frame can be successfully decoded. But the next frame, frame 8n+1, which uses frame 8n-1 as reference in the special coding structure, will also fail in the entropy decoding due to the incorrect or unavailable motion information in frame 8n-1. So in this case, the frame 8n, which cannot guarantee the successful decoding of the following frames, should not be considered as a recovery point from entropy decoding.

(a) regular prediction structure                           (b) special prediction structure

Figure 1 inter prediction structure

So we propose TIDR picture, not only not using TMVP itself, but also marking all the picture may have incorrect MV as unused for temporal information prediction.

- Text Modification

Temporal information decoding refresh (TIDR) access unit: An access unit in which the primary coded picture is a TIDR picture.

Temporal information decoding refresh (TIDR) picture: A coded picture not using temporal information prediction that causes the decoding process to mark all reference pictures except the current TIDR picture as “unused for temporal information prediction” immediately before the decoding of the first picture following the current TIDR with an output order greater than the current TIDR picture. All coded pictures follow a TIDR picture in output order can be successfully entropy decoded without temporal information prediction from any pictures that precedes the TIDR picture in output order. All coded pictures with output order smaller than the current TIDR are not affected by the deferred marking process.

In 8.4.2.1.6,

- If colPu is coded in an intra prediction mode or colPu is unavailable or the picture of colPu is marked as “unused for temporal information prediction” or the current picture is TIDR picture, the following applies.

The variables mvLXCol and availableFlagLXCol are derived as follows.

If colPu is coded in an intra prediction mode or colPu is unavailable or the picture of colPu is marked as “unused for temporal information prediction” or the current picture is TIDR picture, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- References

- JVT-VC, “An investigation on robust parsing”, JCTVC-E148, JCT-VC Meeting, Geneva, CH, March, 2011.

# Patent rights declaration(s)

University of Science and Technology of China, and Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).