| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F203 |
| --- | --- |

| Title: | CE4.3.a Cross-check of Adaptive De-Quantization Offset |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bin Li Gary J. Sullivan Jizheng Xu |  |  |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

This document is a crosscheck report of CE 4.3.a [1], adaptive de-quantization offset. It is reported that the proposed adaptive de-quantization offset algorithm brings about 1.3%, 1.6%, 0.7%, and 0.9% for RA_HE, RA_LC, LD_HE, and LD_LC respectively, and that these measurements match the results reported by the proponent. However, it is reported that it is difficult to determine whether the measured benefit is actually due to the normative modification. Bit allocation effects unrelated to the proposal are reportedly providing some of the measured benefit, and problems with the BD measurement method may also be affecting the results. A non-normative modification was reportedly able to provide about half of the total benefit measured relative to the anchor, and it is remarked that the bit allocation effects with the tested non-normative modification are not as extreme as when the normative modification is included.

# Introduction

This document reports the crosscheck result of CE 4.3.a, adaptive de-quantization offset. The executable file was compiled by Microsoft Visual Studio 2010, 64 bit. The tests were run with the following running environment.

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2008 R2 64 bit | 2xIntel Xeon L5420@2.5Ghz | 16GB | VS2010 |

# Result

Three tests were performed. The anchor is HM-3.0 default anchor.

AQO_10: only the adaptive offset is enabled, relative to the anchor (without Lambda modification).

AQO_11: the adaptive offset is used, and the lambda is adjusted as proposed by the proponent.

AQO_12: only the lambda is adjusted (no normative change is made).

Our results match with those from the MediaTek [2]. The summary result is as follows, and the detailed results can be found in JCTVC-F203_ AQO_10.xls, JCTVC-F203_ AQO_11.xls, and JCTVC-F203_ AQO_12.xls. The decoding time decreasing is due to the decreasing of the bit rate.

Table 1 Summary result of AQO_10 (Y BD-rate)

|  | RA_HE | RA_LC | LD_HE | LD_LC |
| --- | --- | --- | --- | --- |
| Class A | -1.7 | -1.7 |  |  |
| Class B | -0.3 | -0.5 | -0.8 | -1.1 |
| Class C | -0.2 | -0.6 | -0.3 | -1.0 |
| Class D | 0.2 | -0.2 | 0.0 | -0.7 |
| Class E |  |  | -0.1 | -0.4 |
| All | -0.5 | -0.7 | -0.4 | -0.8 |
| Enc Time | 100% | 101% | 100% | 101% |
| Dec Time | 98% | 99% | 99% | 99% |

Table 2 Summary result of AQO_11 (Y BD-rate)

|  | RA_HE | RA_LC | LD_HE | LD_LC |
| --- | --- | --- | --- | --- |
| Class A | -2.7 | -3.7 |  |  |
| Class B | -0.8 | -0.9 | -0.9 | -1.0 |
| Class C | -1.1 | -1.3 | -0.4 | -0.8 |
| Class D | -0.6 | -0.7 | -0.2 | -0.6 |
| Class E |  |  | -1.2 | -1.1 |
| All | -1.3 | -1.6 | -0.7 | -0.9 |
| Enc Time | 99% | 100% | 99% | 100% |
| Dec Time | 96% | 97% | 97% | 97% |

Table 3 Summary result of AQO_12 (Y BD-rate)

|  | RA_HE | RA_LC | LD_HE | LD_LC |
| --- | --- | --- | --- | --- |
| Class A | -1.0 | -2.4 |  |  |
| Class B | -0.4 | -0.2 | 0.1 | 0.6 |
| Class C | -0.7 | -0.5 | 0.2 | 0.6 |
| Class D | -0.4 | 0.0 | 0.2 | 0.7 |
| Class E |  |  | -0.8 | -0.3 |
| All | -0.6 | -0.7 | 0.0 | 0.5 |
| Enc Time | 99% | 100% | 99% | 100% |
| Dec Time | 96% | 97% | 96% | 97% |

# Discussion

Although the results match with those from MediaTek, it is also very hard for us to say that the adaptive de-quantization offset technique provides an actual benefit.

- There are some concerns about the validity of the BD-Rate calculation for some test sequences in CE4, as has been discussed on the JCT-VC email discussion list. The Nebuta sequence provides the largest gain, and this sequence has already been discussed as a primary example of the BD-Rate calculation problem.

- The bit allocation among the frames at different levels of the temporal hierarchy has been changed by the use of the technique. In AQO_11, the quality of the I frames are identical to HM-3.0 anchor, while the average quality of the B frames drops by about 0.3 dB. This seems to explain why the gain of the RA test cases is larger than for LD. We cannot conclude the proposed algorithm is providing a benefit when the bit allocation has changed in this way. The decrease of decoding time is also caused by the decrease of bit rate. Also, as reported in AQO_12, changing only lambda without modifying the de-quantization offsets will also change the bit allocation and also provides some measured gain. The bit-rate changes are shown in Tables 4 and 5. The PSNR of intra frames are the same, the difference of bit rate of I frames are caused by the overhead in the slice header to signal the offset and byte alignment. It can be seen that the bit rate changes so much among different methods. As the bit allocation changes, it is hard to say where the gain comes from. It seems possible that the differences in the allocation of bit rate to frame types may alone lead to the different gains.

We particularly note that the bit rate allocation of the non-normative modification case AQO_12, while being different than the anchor, is also not the same as in the AQO_11 case that includes the normative modification. Both of these cases shift the bit allocation in the same direction, but the shift in the AQO_11 case is substantially larger.

It seems hypothetically possible that this explains why the gain observed with the normative modification is larger. Further adjustment to the selection of the Lambda or quantization parameters as a non-normative modification could potentially provide the same amount of gain that is observed with the proposed technique.

Table 4 Average bit-rate difference between proposal and anchor (kbps) for RA cases

| RA_HE | all | I | B | T0 | T1 | T2 | T3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AQO_10 | -459.67 | 0.09 | -469.27 | -1483.79 | -1018.06 | -608.88 | 11.98 |
| AQO_11 | -845.71 | 0.09 | -864.23 | -2294.92 | -1778.96 | -1150.57 | -95.91 |
| AQO_12 | -495.80 | 0.09 | -507.08 | -934.17 | -1028.68 | -769.16 | -116.23 |
| RA_LC |  |  |  |  |  |  |  |
| AQO_10 | -448.97 | 0.09 | -458.50 | -1496.70 | -935.66 | -534.14 | -21.36 |
| AQO_11 | -951.44 | 0.09 | -971.81 | -2392.77 | -1708.86 | -1076.57 | -337.29 |
| AQO_12 | -682.81 | 0.09 | -697.18 | -1023.35 | -1058.12 | -745.94 | -472.37 |

Table 5 Average bit-rate difference between proposal and anchor (kbps) for LD cases

| LD_HE | all | I | B | T0 | T1 | T2 |
| --- | --- | --- | --- | --- | --- | --- |
| AQO_10 | -241.35 | 0.03 | -241.86 | -601.17 | -206.11 | -79.07 |
| AQO_11 | -504.22 | 0.03 | -505.27 | -971.82 | -501.98 | -271.55 |
| AQO_12 | -318.14 | 0.03 | -318.79 | -390.58 | -402.42 | -239.78 |
| LD_LC |  |  |  |  |  |  |
| AQO_10 | -255.34 | 0.11 | -255.88 | -698.27 | -177.86 | -72.62 |
| AQO_11 | -528.86 | 0.11 | -529.94 | -1111.52 | -477.64 | -263.13 |
| AQO_12 | -339.08 | 0.11 | -339.77 | -498.61 | -392.92 | -232.39 |

- The offsets between different frames, QPs, and source test sequences do not really change much. JCTVC-F203_LDHE_YOffset.xls is the Y offset of LD_HE temporal level 0 (POC=4, 8, 12, … ). Almost always the offsets are equal to 20, which does not really seem very adaptive. If the offset was just set to 20 always instead of being adaptively computed, the results would probably be about the same.

- As proposed in JCTVC-E091, c=c'∗Q+b where c' is the absolute value of the quantized coefficient, c is the absolute value of the reconstruction, Q is the de-quantization step size, and b is the proposed adaptively-computed offset. But b is always positive, which will lead to the absolute value of reconstructed transformed coefficient for a given quantized index value always being larger than when the adaptive de-quantization offset is not used. We can alternatively say that, on average, this tends to results in a larger quantization "dead zone" and smaller quantized index magnitudes for the transform coefficients, thus using fewer bits on average; however, the average distortion will also (always) be increased for a given step size. This causes the change of the average bit allocation – always lowering the bit rate used by the frames affected by the technique. We note that the technique was not applied to the I frames, which largely causes the discussed shift in the allocation of bits among types of frames.

# References

- JVT-VC, “Description of Core Experiment 4: Quantization”, JCTVC-E704, JCT-VC Meeting, Geneva, CH, March, 2011.

- JVT-VC, “CE4 Subtest3: Adaptive De-Quantization Offset”, JCTVC-F119, JCT-VC Meeting, Torino, IT, July, 2011.