| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F206 |
| --- | --- |

| Title: | CE12: Cross-check of harmonized deblocking filter with additional chroma processing (JCTVC-F262) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Zhongbo Shi Jizheng Xu |  |  |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

This document reports cross-verification results for SKKU’s proposal JCTVC-F262, including objective results. Detailed results are summarized in the attached excel sheet.

# Introduction

In JCTVC-F262, SKKU proposes to consider more cases to decide boundary strength values (for both luma and chroma signals) and introduces a modified filtering decision (strong/week filtering) to improve the decoded quality of HEVC coding system. We got the source code from SKKU and independently compiled and ran the code with the following running environment.

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2008 R2 64 bits | 2xIntel Xeon L5420@2.5Ghz | 16GB | VS2010 |

The objective results we got match SKKU’s. The encoding/decoding times compared with the anchors’ are also similar to what SKKU reported. No mismatch between encoder and decoder is found.

# Objective Results

The summaries of the results are shown in the following tables. The detail results can be found in the file JCTVC-F206.xls. The results match those reported by the proponent.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.7 | -1.5 | -1.2 | -1.1 | -2.3 | -2.1 |
| Class B | -1.3 | -1.4 | -1.6 | -1.1 | -2.6 | -2.7 |
| Class C | -1.0 | -2.5 | -2.5 | -1.0 | -3.4 | -3.9 |
| Class D | -0.9 | -2.2 | -2.4 | -0.9 | -3.5 | -3.8 |
| Class E | -1.3 | -0.4 | -0.6 | -1.0 | -2.2 | -3.0 |
| All | -1.2 | -1.7 | -1.7 | -1.1 | -2.8 | -3.1 |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.6 | -2.3 | -2.1 | -0.9 | -2.6 | -2.0 |
| Class B | -1.3 | -2.6 | -2.3 | -1.0 | -2.7 | -2.3 |
| Class C | -1.0 | -3.4 | -3.2 | -0.9 | -3.3 | -3.7 |
| Class D | -0.8 | -2.9 | -3.4 | -0.7 | -2.9 | -3.1 |
| Class E |  |  |  |  |  |  |
| All | -1.2 | -2.8 | -2.7 | -0.9 | -2.8 | -2.7 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.7 | -2.7 | -2.4 | -1.0 | -2.4 | -2.4 |
| Class C | -1.3 | -2.8 | -2.8 | -0.9 | -2.3 | -3.1 |
| Class D | -1.1 | -1.9 | -2.1 | -0.8 | -1.7 | -2.1 |
| Class E | -2.9 | -2.9 | -3.6 | -1.5 | -6.6 | -6.1 |
| All | -1.7 | -2.6 | -2.7 | -1.0 | -3.0 | -3.2 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |