| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F208 |
| --- | --- |

| Title: | Cross-check of parallelized merge/skip mode (JCTVC-F069) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu |  |  |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

This is a cross-check report of JCTVC-F069, Parallelized merge/skip mode for HEVC.

# Introduction

This document reports the crosscheck result of JCTVC-F069, Parallelized merge/skip mode for HEVC [1]. The executable file was compiled by Microsoft Visual Studio 2010, 64-bit. The tests were run with the following running environment.

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2008 R2 64 bits | 2xIntel Xeon L5420@2.5Ghz | 16GB | VS2010 |

Three macros were defined in the software.

#define TI_PARALLEL_ME 0

#define PARALLEL_ME_ANCHOR 1

#define MER_SIZE 64

MER_SIZE defines the parallel ME level, for example, MER_SIZE = 64 assume all the motion vectors and skip/merge are searched in parallel for 64x64 regions. MER stands for motion estimation region which is square. MER can have value of 4, 8, 16, 32 or 64. MER size should not be larger than the LCU size. MER_SIZE = 4 means sequential ME, just like the committee code is doing in which merge/skip is possible for all block sizes.

For PME Anchor, only the top-left CU or PU in the MER_SIZE x MER_SIZE region can use merge mode. No normative changes in PME anchor.

For PME TI (proposed algorithm), the CU or PU can only use the merge candidates outside the region of MER_SIZE x MER_SIZE, as shown in Figure 1. And temporal merge candidate can also be used. There are normative changes in the proposed algorithm.

Figure 1 Illustration of motion estimation region (MER) partitioning and availability rules of spatial MVPs for PUs at different locations

# Result

The summary experimental result is shown in Table 1-4. More detailed result can be found in the excel files attached. The anchor is HM-3.0+r828. Our results match those from TI [1].

Table 1 Summary result of PME Anchor 8 and TI 8 (Y BD-rate)

|  | RA_HE |  | RA_LC |  | LD_HE |  | LD_LC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Anchor | TI | Anchor | TI | Anchor | TI | Anchor | TI |
| Class A | 0.2 | 0.1 | 0.3 | 0.0 |  |  |  |  |
| Class B | 0.2 | 0.0 | 0.3 | 0.0 | 0.2 | -0.2 | 0.3 | 0.0 |
| Class C | 0.5 | 0.1 | 0.6 | 0.1 | 0.6 | 0.0 | 0.7 | 0.0 |
| Class D | 0.7 | 0.1 | 0.8 | 0.1 | 0.7 | -0.1 | 0.9 | 0.0 |
| Class E |  |  |  |  | 0.2 | -0.4 | 0.4 | 0.0 |
| All | 0.4 | 0.1 | 0.5 | 0.0 | 0.4 | -0.1 | 0.6 | 0.0 |
| Enc T | 98% | 99% | 98% | 99% | 99% | 100% | 99% | 100% |
| Dec T | 100% | 100% | 100% | 101% | 100% | 101% | 100% | 101% |

Table 2 Summary result of PME Anchor 16 and TI 16 (Y BD-rate)

|  | RA_HE |  | RA_LC |  | LD_HE |  | LD_LC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Anchor | TI | Anchor | TI | Anchor | TI | Anchor | TI |
| Class A | 1.7 | 0.4 | 1.8 | 0.4 |  |  |  |  |
| Class B | 2.8 | 0.5 | 2.8 | 0.5 | 3.2 | 0.3 | 3.4 | 0.5 |
| Class C | 3.0 | 1.0 | 3.0 | 0.9 | 3.6 | 0.8 | 3.7 | 0.8 |
| Class D | 3.3 | 1.1 | 3.3 | 0.9 | 3.7 | 0.9 | 4.2 | 1.1 |
| Class E |  |  |  |  | 3.4 | 0.0 | 3.5 | 0.3 |
| All | 2.7 | 0.8 | 2.7 | 0.7 | 3.5 | 0.5 | 3.7 | 0.7 |
| Enc T | 95% | 99% | 96% | 99% | 96% | 99% | 98% | 99% |
| Dec T | 98% | 100% | 98% | 101% | 98% | 100% | 97% | 101% |

Table 3 Summary result of PME Anchor 32 and TI 32 (Y BD-rate)

|  | RA_HE |  | RA_LC |  | LD_HE |  | LD_LC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Anchor | TI | Anchor | TI | Anchor | TI | Anchor | TI |
| Class A | 4.1 | 1.3 | 4.4 | 1.5 |  |  |  |  |
| Class B | 6.3 | 2.0 | 6.8 | 1.8 | 6.6 | 1.9 | 8.0 | 2.1 |
| Class C | 6.5 | 2.6 | 6.6 | 2.5 | 6.9 | 2.4 | 7.3 | 2.4 |
| Class D | 7.1 | 2.6 | 7.2 | 2.4 | 7.2 | 2.8 | 8.2 | 2.9 |
| Class E |  |  |  |  | 11.4 | 1.6 | 12.9 | 2.1 |
| All | 6.0 | 2.1 | 6.3 | 2.1 | 7.7 | 2.2 | 8.8 | 2.4 |
| Enc T | 86% | 97% | 88% | 98% | 87% | 98% | 89% | 98% |
| Dec T | 98% | 100% | 97% | 100% | 97% | 100% | 96% | 100% |

Table 4 Summary result of PME Anchor 64 and TI 64 (Y BD-rate)

|  | RA_HE |  | RA_LC |  | LD_HE |  | LD_LC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Anchor | TI | Anchor | TI | Anchor | TI | Anchor | TI |
| Class A | 6.1 | 2.5 | 7.0 | 2.8 |  |  |  |  |
| Class B | 9.3 | 3.7 | 10.4 | 3.7 | 9.2 | 4.0 | 11.8 | 4.3 |
| Class C | 9.0 | 4.2 | 9.4 | 4.1 | 8.5 | 3.9 | 9.6 | 3.9 |
| Class D | 9.4 | 4.2 | 10.0 | 4.0 | 8.6 | 4.6 | 10.4 | 4.7 |
| Class E |  |  |  |  | 15.5 | 4.0 | 19.7 | 4.9 |
| All | 8.5 | 3.7 | 9.3 | 3.6 | 10.0 | 4.1 | 12.4 | 4.4 |
| Enc T | 76% | 95% | 78% | 96% | 78% | 96% | 81% | 97% |
| Dec T | 98% | 100% | 97% | 102% | 97% | 102% | 95% | 102% |

# References

- JVT-VC, “Parallelized merge/skip mode for HEVC”, JCTVC-F069, JCT-VC Meeting, Torino, IT, July, 2011.