| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F209 |
| --- | --- |

| Title: | Cross-check of complexity scalable ALF (JCTVC-F161) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu |  |  |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

This document is a crosscheck report of JCTVC-F161, Complexity Scalable ALF. It is reported turning on the ALF periodically in some frames for low complexity configurations will bring 0.3%, 2.1%, 2.7%, and 3.2% bit saving for AI_LC, RA_LC, LD_B_LC, and LD_P_LC respectively. And these measurements match the results reported by the proponent.

# Introduction

This document reports crosscheck result of JCTVC-F161 [1], complexity scalable ALF. The executable file was compiled by Microsoft Visual Studio 2010, 64 bit. The tests were run with the following running environment.

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2008 R2 64 bit | 2xIntel Xeon L5420@2.5Ghz | 16GB | VS2010 |

# Result

Two tests were performed. In the first test, the period of the frames turning on ALF is 8; and in the second one, the period is 16. The anchor is HM-3.0 r930, low complexity configuration, with all ALF off. The results match those from the proponent. The summary results are shown in Table 1 and 2.

Table 1 Summary result of Period of 8 (Y BD-Rate)

|  | AI_LC | RA_LC | LD_B_LC | LD_P_LC |
| --- | --- | --- | --- | --- |
| Class A | -0.5 | -3.4 |  |  |
| Class B | -0.3 | -2.6 | -2.6 | -3.6 |
| Class C | -0.3 | -1.7 | -2.3 | -2.1 |
| Class D | -0.1 | -0.6 | -1.1 | -0.7 |
| Class E | -0.4 |  | -5.3 | -7.3 |
| All | -0.3 | -2.1 | -2.7 | -3.2 |
| Enc Time | 102% | 101% | 101% | 101% |
| Dec Time | 107% | 110% | 110% | 111% |

Table 2 Summary result of Period of 16 (Y BD-Rate)

|  | AI_LC | RA_LC | LD_B_LC | LD_P_LC |
| --- | --- | --- | --- | --- |
| Class A | -0.3 | -2.4 |  |  |
| Class B | -0.1 | -1.9 | -1.9 | -2.5 |
| Class C | -0.1 | -1.2 | -1.7 | -1.6 |
| Class D | -0.1 | -0.4 | -0.7 | -0.5 |
| Class E | -0.2 |  | -4.7 | -6.4 |
| All | -0.2 | -1.5 | -2.1 | -2.5 |
| Enc Time | 100% | 100% | 100% | 100% |
| Dec Time | 103% | 104% | 104% | 104% |

The decoding time increased because of ALF was turned on at some frames.

# References

- JCT-VC, “Complexity Scalable ALF”, JCTVC-F161, JCT-VC Meeting, Torino, IT, July, 2011.