| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F212 |
| --- | --- |

| Title: | Cross-check of intra mode parsing (JCTVC-F378) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiulian Peng Jizheng Xu |  |  |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

This document reports the cross-check results of Samsung’s proposal JCTVC-F378 on most probable mode.

# Code description

In the software provided by Samsung, there are four macros added:  INTRA_MODE_CODING, FORCE_2MPM, PLANAR_DC_MPM_SWAP and FORCE_2MPM_RESTRICT_SEARCH. In terms of most probable mode, there are four modifications. One is to code planar intra mode as one of the angular modes. The other three unifies the most probable modes for intra prediction to be always two.

# Test conditions and Results

The settings for the four test cases crosschecked are as follows.

1)

#define INTRA_MODE_CODING                1

#define FORCE_2MPM                       0

#define PLANAR_DC_MPM_SWAP                0

#define FORCE_2MPM_RESTRICT_SEARCH       0

2)

#define INTRA_MODE_CODING                1

#define FORCE_2MPM                       1

#define PLANAR_DC_MPM_SWAP                0

#define FORCE_2MPM_RESTRICT_SEARCH       0

3)

#define INTRA_MODE_CODING                1

#define FORCE_2MPM                       1

#define PLANAR_DC_MPM_SWAP                0

#define FORCE_2MPM_RESTRICT_SEARCH       1

4)

#define INTRA_MODE_CODING                1

#define FORCE_2MPM                       1

#define PLANAR_DC_MPM_SWAP                1

#define FORCE_2MPM_RESTRICT_SEARCH       0

All results were tested in Intra and Intra LoCo cases under the recommended test conditions of CE6 [1]. The platform and results are shown in Table 1 and Table 2-5, respectively. The anchor is HM3.0. All results exactly match that provided by Samsung.

Table 1. Platform configuration

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2008 R2, 64 bits | 2xIntel Xeon L5420@2.5Ghz | 16GB | VS2010 |

Table 2. Test 1 Results (JCTVC-F212_ss1000.xls)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.4 | -0.3 | -0.2 | -0.2 | -0.2 |
| Class B | 0.0 | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 |
| Class C | -0.1 | -0.3 | -0.3 | -0.1 | -0.1 | -0.2 |
| Class D | -0.1 | -0.2 | -0.3 | -0.1 | -0.1 | -0.1 |
| Class E | 0.0 | -0.1 | -0.1 | -0.1 | -0.1 | 0.0 |
| All | 0.0 | -0.2 | -0.3 | -0.1 | -0.1 | -0.1 |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

Table 3. Test 2 Results (JCTVC-F212_ss1100.xls)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.2 | -0.1 | -0.1 | -0.1 | 0.0 |
| Class B | 0.0 | -0.1 | 0.0 | -0.1 | 0.1 | 0.1 |
| Class C | 0.0 | -0.2 | -0.1 | 0.0 | 0.1 | 0.1 |
| Class D | -0.1 | -0.1 | -0.2 | 0.0 | 0.2 | 0.1 |
| Class E | 0.2 | 0.3 | 0.2 | 0.2 | 0.4 | 0.4 |
| All | 0.0 | -0.1 | 0.0 | 0.0 | 0.1 | 0.1 |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

Table 4. Test 3 Results (JCTVC-F212_ss1101.xls)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | -0.1 | -0.1 | 0.0 | 0.1 |
| Class B | 0.1 | 0.0 | 0.1 | 0.0 | 0.1 | 0.2 |
| Class C | 0.0 | -0.1 | -0.1 | 0.0 | 0.1 | 0.1 |
| Class D | -0.1 | 0.0 | -0.2 | 0.0 | 0.2 | 0.1 |
| Class E | 0.2 | 0.3 | 0.2 | 0.2 | 0.5 | 0.4 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.2 | 0.1 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

Table 5. Test 4 Results (JCTVC-F212_ss1110.xls)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | -0.1 | 0.1 | 0.1 |
| Class C | -0.1 | -0.1 | -0.1 | 0.0 | 0.1 | 0.1 |
| Class D | -0.1 | -0.1 | -0.1 | -0.1 | 0.1 | 0.1 |
| Class E | 0.0 | 0.3 | 0.3 | 0.1 | 0.5 | 0.4 |
| All | 0.0 | 0.0 | 0.0 | -0.1 | 0.1 | 0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

# References

- Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-E700 (m20235), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, March 2011.