| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F213 WG11 Number: m20632 |
| --- | --- |

| Title: | CE9 4.4: Results and verifications for the inferred merge |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal / Information |  |  |
| Author(s) or Contact(s): | Yoshinori Suzuki T.K. Tan 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239 8536 Japan | Tel: Email: | +81 468 40 3517 suzukiyos@rd.nttdocomo.co.jp +65 9799 3861 tktan.p@rd.nttdocomo.co.jp |
| Source: | NTT DOCOMO, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the simulation results for section 4.4 of Core Experiment CE9 as described in JCTVC-E709r_2. In section 4.4 of CE9, 13 experiments were established. The results of 4 experiments (PART01, PART03, PART05 and PART17) and 6 cross verification (PART02, PART04, PART06, PART12, PART14 and PART15) are reported in this contribution. In addition, the performance-complexity trade-offs of these 10 experiments are compared.

In the current HM specification for the inter block coding, the first 2NxN or Nx2N PU partition in a CU greater than the smallest CU is always merged with a neighboring block. This is referred to as “inferred merge”, as the merge mode of the first PU is inferred without explicit signaling of the merge_flag.

The following experiments were conducted for the four inter coding common conditions. In the PART01 experiment, inferred merge is applied to all CU levels. An overall average luma BD-rate of 0.1% (loss) was achieved with 7.5% encoding time reduction. In the PART03 experiment, the inferred merge with signaling of merge_flag=1 for the first PU is applied to all CU levels. An overall average luma BD-rate of 0.6% (loss) was achieved with 7.8% encoding time reduction.

In the PART05 and PART17 experiments, the combinations of PART01 with other two experiments of 4.4 in CE9 (PART4 and PART14) were tested, respectively. PART05 achieved an overall average luma BD-rates of 0.8% (loss) with 31% encoding time reduction, whereas PART17 achieved an overall average luma BD-rates of 0.3% (loss) with 18% encoding time reduction.

# Introduction

At the 4th JCT-VC meeting in Daegu, Korea, the inferred merge for the first 2NxN or Nx2N PU partition of a CU, based on JCTVC-D233 [2] and JCTVC-D441 [3], was adopted into the HM2.0. In this method, a prediction signal of the first 2NxN or Nx2N PU partition in a CU that is greater than the smallest CU (8x8 in common configuration) is produced utilizing motion data of a neighboring block without signaling of the merge_flag (a flag that selects conventional motion compensated prediction or PU merge).

At the 5th JCT-VC meeting in Geneva, CH, the inferred merge for both first and second 2NxN or Nx2N PU partitions of a CU was proposed in JCTVC-E085, and NTT DOCOMO suggested to test the method in which the inferred merge for the first PU of a CU with 2NxN or Nx2N partitions is applied to all CU size for the simplification of codec design and syntax. Additionally, after the 5th JCT-VC meeting, MediTek proposed to test the inferred merge for all NxN PUs as described in JCTVC-E709_r2 [4].

In section 4.4 of CE9, four items and their combinations were evaluated,

1) applying the inferred merge of the first PU of a CU with 2NxN and Nx2N partitions to all CU levels,

2) enabling the inferred merge of the second PU of a CU with 2NxN and Nx2N partitions,

3) enabling the inferred merge of all PUs of a CU with NxN partitions and

4) disabling the inferred merge.

In this contribution, 4 experiments related to 1) were conducted and 6 experiments were cross verified.

# Experiments of section 4.4 of CE9

## PART01 and PART03: Evaluation of inferred merge for 1st PU of CU with 2NxN/Nx2N partitions

In the HM2.0 and HM3.0, the inferred merge for 1st PU of a CU with 2NxN or Nx2N partitions is applied to a CU that is greater than SCU (8x8 in common test condition). In the CE9, it is applied to all CU size. Two experiments and two verifications were conducted in this item.

- Test

  - PART01: the inferred merge without signaling of merge_flag for all CU levels

  - PART03: the inferred merge with signaling of merge_flag=1 for all CU levels

The proposed specification for the working draft for PART01 can be found in the Annex. The syntax and codec design are simplified by removing the CU size dependency.

## PART04-06: Evaluation of inferred merge for 1st and 2nd PUs of CU with 2NxN/Nx2N partitions

The inferred merge for both 1st and 2nd PUs of a CU with 2NxN or Nx2N partitions described in JCTVC-E085 [4] is evaluated. One experiment and two verifications were conducted in this item.

- Verification

  - PART04: the inferred merge without signaling of merge_flag for CUs greater than 8x8

  - PART06: the inferred merge with signaling of merge_flag=1 for CUs greater than 8x8

- Test

  - PART05: the inferred merge without signaling of merge_flag for all CU levels. The combination of PART04 and PART01 was tested.

The proposed specification for the working draft for PART05 can be found in the Annex. The syntax and codec design are simplified by removing the CU size dependency.

## PART14, PART15 and PART17: Evaluation of inferred merge for all PUs of CU with NxN partitions

The inferred merge for all PUs of a CU with NxN partitions is evaluated. One experiment and two verifications were conducted in this item.

- Verification

  - PART14: the inferred merge without signaling of merge_flag for CUs greater than 8x8

  - PART15: the inferred merge with signaling of merge_flag=1 for CUs greater than 8x8

- Test

  - PART17: the inferred merge without signaling of merge_flag for all CU levels. The combination of PART14 and PART01 was tested.

The proposed specification for the working draft for PART17 can be found in the Annex. The syntax and codec design are simplified by removing the CU size dependency.

## PART02 and PART12: Evaluation of disabling inferred merge

The disabling of inferred merge described in JCTVC-E115 [5] is evaluated. Two verifications were conducted in this item.

- Verification

  - PART02: the inferred merge with signaling of merge_flag=1 for CUs greater than 8x8

  - PART12: disabling the inferred merge

Note that the modification of PART12 was proposed by JCTVC-E115 [5] at the 5th meeting because of encoder flexibility, and the further study was recommended for two reasons (from the tentative meeting notes; JCTVC-E_Notes_d8).

a) Reasonable comparison would be against an encoder which does not use merge at all.

b) Evidence about encoder complexity reduction should be shown.

# Experimental results

The experiments were conducted in accordance with the 4 common test conditions for inter coding described in [6]. The proposed modifications were implemented in HM3.0 r823 of the common software with a bug fix reported in section 5 of JCTVC-E709_r2 [1]. The excel sheet corresponding to the results of PART01, PART02 PART03, PART04, PART05, PART06, PART12, PART14, PART15 and PART17 are attached as part of the contribution. They were cross verified by DOCOMO and MediaTek.

## Inferred merge for 1st PU of CU with 2NxN/Nx2N partitions

The overall average of luma BD-rate, encoding time and decoding time of PART01 and PART03 are shown in Table 1. The results of PART01 are shown in Table 2. They show that PART01 can reduce 7.5% encoding time with no significant loss and it is confirmed that PART01 is better than PART03.

Table 1: Averaged results of inferred merge for 1st PU of a CU with 2NxN/Nx2N partitions

| Experiments | BD-rate [%] | Enc time [%] | Dec time [%] |
| --- | --- | --- | --- |
| PART01 | 0.10 | 92.5 | 100 |
| PART03 | 0.63 | 92.3 | 100 |

Table 2: Results of PART01 (Ref: HM3.0 r823 with bug fix)

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.2 | 0.2 | 0.0 | 0.2 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 | 0.1 |
| Class C | 0.2 | 0.2 | 0.3 | 0.3 | 0.3 | 0.3 |
| Class D | 0.5 | 0.5 | 0.5 | 0.4 | 0.3 | 0.3 |
| Class E |  |  |  |  |  |  |
| All | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 |
| Enc Time[%] | 94% |  |  | 92% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | -0.3 | 0.0 | -0.1 | -0.1 |
| Class C | 0.1 | 0.3 | 0.3 | 0.1 | 0.0 | 0.0 |
| Class D | 0.2 | -0.1 | 0.4 | 0.3 | 0.0 | 0.0 |
| Class E | -0.2 | -0.7 | 0.0 | -0.2 | -0.1 | 0.2 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 93% |  |  | 91% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

## Inferred merge for 1st and 2nd PUs of CU with 2NxN/Nx2N partitions

The overall average of luma BD-rate, encoding time and decoding time of PART04-06 are shown in Table 3. The results of table 3 show that the loss caused by PART04 is not ignorable. The PART05 achieves can reduce encoding time by 31%, though it causes 0.8% BD-rate increase.

Table 3: Averaged results of inferred merge for both 1st and 2nd PUs of a CU with 2NxN/Nx2N partitions

| Experiments | BD-rate [%] | Enc time [%] | Dec time [%] |
| --- | --- | --- | --- |
| PART04 | 0.48 | 85.3 | 100 |
| PART05 (PART04+PART01) | 0.83 | 69.0 | 101 |
| PART06 | 0.75 | 85.3 | 100 |

## Inferred merge for all PUs of CU with NxN partitions

The overall average of luma BD-rate, encoding time and decoding time of PART04-06 are shown in Table 4. The results of PART17 are shown in Table 5. Table 4 indicates that PART14 can reduce encoding time by 10% without a coding loss. PART17 is better than PART15 and PART04. PART01 can reduce encoding time of PART14 by 7.8% with 0.3% BD-rate increase.

Table 4: Averaged results of inferred merge for both all PUs of a CU with NxN partitions

| Experiments | BD-rate [%] | Enc time [%] | Dec time [%] |
| --- | --- | --- | --- |
| PART14 | 0.00 | 89.8 | 100 |
| PART15 | 0.40 | 89.8 | 100 |
| PART17 (PART14+PART01) | 0.33 | 82 | 101 |

Table 5: Results of PART17 (Ref: HM3.0 r823 with bug fix)

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.4 | 0.2 | 0.1 | 0.4 | 0.2 |
| Class B | 0.1 | 0.1 | 0.1 | 0.2 | 0.2 | 0.2 |
| Class C | 0.5 | 0.5 | 0.7 | 0.7 | 0.7 | 0.9 |
| Class D | 0.9 | 1.1 | 1.2 | 1.1 | 1.2 | 1.3 |
| Class E |  |  |  |  |  |  |
| All | 0.3 | 0.5 | 0.5 | 0.5 | 0.6 | 0.6 |
| Enc Time[%] | 85% |  |  | 81% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | -0.1 | 0.0 | -0.1 | -0.2 |
| Class C | 0.3 | 0.3 | 0.3 | 0.4 | 0.4 | 0.4 |
| Class D | 0.5 | 0.8 | 1.1 | 0.8 | 0.6 | 0.6 |
| Class E | -0.2 | -0.2 | 0.0 | -0.2 | -0.3 | 0.3 |
| All | 0.2 | 0.3 | 0.3 | 0.3 | 0.1 | 0.2 |
| Enc Time[%] | 83% |  |  | 79% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

## Disabling inferred merge for 1st PU of CU with 2NxN/Nx2N partitions

The overall average of luma BD-rate, encoding time and decoding time of PART02 and PART12 are shown in Table 6.

Tables 1 and 6 show that PART02 is worse than PART01.

The performance of PART12 is similar to that of JCTVC-E115 (BD-rate; -0.45%, Enc time; 115%).

Table 6: Averaged results of disabling of inferred merge for 1st PU of a CU with 2NxN/Nx2N partitions

| Experiments | BD-rate [%] | Enc time [%] | Dec time [%] |
| --- | --- | --- | --- |
| PART02 | 0.25 | 100 | 100 |
| PART12 | -0.43 | 114.3 | 99 |

## Comparison of 10 experiments on the inferred merge

The overall average of luma BD-rate and encoding time of 10 experiments (PART01-PART06, PART12, PART14, PART15 and PART17) in section 4.4 of CE9 are shown in Table 7. The relationship of BD-rate and encoding time shown in Table 7 for each experiment is plotted in Figure 1.

Figure 1 shows that PART05, PART14 and PART17 achieved the top three performance-complexity trade-off. PART01 extends the inferred merge in the anchor to all CU levels. PART05 is the result of extending the inferred merge in PART04 to all CU levels.  PART17 is the result of combining PART01 and PART14.  This shows that inferred merge at all CU levels give good results.

Table 7: Averaged results of section 4.4 of CE9

| Experiments | BD-rate [%] | Enc time [%] |
| --- | --- | --- |
| PART01 (inferred merge for 1st PU in 2NxN/Nx2N CU at all CU levels) | 0.10 | 92.5 |
| PART02 (encode merge_flag=1 for 1st PU in 2NxN/Nx2N CU, CU > 8x8) | 0.25 | 100 |
| PART03 (PART01+PART02) | 0.63 | 92.3 |
| PART04 (inferred merge for 1st and 2nd PU in 2NxN/Nx2N CU, CU > 8x8) | 0.48 | 85.3 |
| PART05 (PART01+PART04) | 0.83 | 69.0 |
| PART06 (encode merge_flag=1 for 1st and 2nd PU in 2NxN/Nx2N CU, CU > 8x8) | 0.75 | 85.3 |
| PART12 (disabling inferred merge) | -0.43 | 114.3 |
| PART14 (Anchor + inferred merge for all NxN PU) | 0.00 | 89.8 |
| PART15 (PART02 + encode merge_flag=1 for all NxN PU) | 0.40 | 89.8 |
| PART17 (PART01 + PART14) | 0.33 | 82 |

Figure 1 Performance-encoder complexity trade-off

# Conclusion

This report confirms that inferred merge of the first PU in a CU with 2NxN/Nx2N partitions can be applied to all CU levels, with an average BD-rate loss of 0.1% and an encoding time reduction of 7.5%.

Starting with this and adding inferred merge to the second PU in a CU with 2NxN/Nx2N partitions or to all the NxN CUs give a reduction in encoding time of 31% and 18%, respectively.  However, an average BD-rate loss of 0.8% and 0.3% is incurred, respectively.

At the other spectrum, disabling the inferred merge for all PUs results in an average BD-rate gain of 0.4%, but a 14% increase in encoding time.

It is recommended that the inferred merge for the first PU or the first and second PUs in a CU with 2NxN/Nx2N partitions is applied to all CU levels to remove the CU size dependency. Please refer to the Annex for the proposed WD text.

# Reference

- Y.-W. Huang et. al, “Description of Core Experiment 9: MV Coding and Skip/Merge Operations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E709_r2, 5th Meeting: Geneva, CH, 16-23 March, 2011.

- Y. Suzuki et. al, “CE9: 3.1 PU Merge & Skip tools and proposed improvements”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D233, 4th Meeting: Daegu, Korea, 20-28 January, 2011.

- TK. Tan et. al, “BoG report of CE9: Motion Vector Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D441, 4th Meeting: Daegu, Korea, 20-28 January, 2011.

- S. Liu et. al, “Method and syntax for partial CU merge”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E085, 5th Meeting: Geneva, CH, 16-23 March, 2011.

- M. Zhou, “Evaluation results on merge mode in HM2.0”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E115, 5th Meeting: Geneva, CH, 16-23 March, 2011.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E700, 5th Meeting: Geneva, CH, 16-23 March, 2011.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex 1:  Proposed specification for the WD.

### Modification for PART01

### 5.1.1	Coding unit syntax

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize, 0 , 0 ) |  |
| else { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( slice_type == I && log2CUSize == Log2MinCUSize ) |  |
| intra_part_mode | u(1) |
| } else if( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) |  |
| pred_type | u(v) ¦ ae(v) |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize, 0 , 0 ) |  |
| } else if( PartMode == PART_2NxN ) { |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize – 1, 0 , 1log2CUSize > Log2MinCUSize ) |  |
| prediction_unit( x0, y1, log2CUSize, log2CUSize – 1, 1 , 0 ) |  |
| } else if( PartMode == PART_Nx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize - 1, log2CUSize, 0 , 1log2CUSize > Log2MinCUSize ) |  |
| prediction_unit( x1, y0, log2CUSize - 1, log2CUSize, 1 , 0 ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, log2CUSize – 1, log2CUSize – 1, 0 , 0 ) |  |
| prediction_unit( x1, y0, log2CUSize – 1, log2CUSize – 1, 1 , 0 ) |  |
| prediction_unit( x0, y1, log2CUSize – 1, log2CUSize – 1, 2 , 0 ) |  |
| prediction_unit( x1, y1, log2CUSize – 1, log2CUSize – 1, 3 , 0 ) |  |
| } |  |
| } |  |
| if( !pcm_flag ) { |  |
| transform_tree( x0, y0, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 1 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 2 ) |  |
| } |  |
| } |  |

### Modification for PART05

### 5.1.1	Coding unit syntax

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize, 0 , 0 ) |  |
| else { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( slice_type == I && log2CUSize == Log2MinCUSize ) |  |
| intra_part_mode | u(1) |
| } else if( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) |  |
| pred_type | u(v) ¦ ae(v) |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize, 0 , 0 ) |  |
| } else if( PartMode == PART_2NxN ) { |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize – 1, 0 , 1log2CUSize > Log2MinCUSize ) |  |
| prediction_unit( x0, y1, log2CUSize, log2CUSize – 1, 1 , 10 ) |  |
| } else if( PartMode == PART_Nx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize - 1, log2CUSize, 0 , 1log2CUSize > Log2MinCUSize ) |  |
| prediction_unit( x1, y0, log2CUSize - 1, log2CUSize, 1 , 10 ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, log2CUSize – 1, log2CUSize – 1, 0 , 0 ) |  |
| prediction_unit( x1, y0, log2CUSize – 1, log2CUSize – 1, 1 , 0 ) |  |
| prediction_unit( x0, y1, log2CUSize – 1, log2CUSize – 1, 2 , 0 ) |  |
| prediction_unit( x1, y1, log2CUSize – 1, log2CUSize – 1, 3 , 0 ) |  |
| } |  |
| } |  |
| if( !pcm_flag ) { |  |
| transform_tree( x0, y0, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 1 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 2 ) |  |
| } |  |
| } |  |

### Modification for PART17

### 5.1.1	Coding unit syntax

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize, 0 , 0 ) |  |
| else { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( slice_type == I && log2CUSize == Log2MinCUSize ) |  |
| intra_part_mode | u(1) |
| } else if( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) |  |
| pred_type | u(v) ¦ ae(v) |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize, 0 , 0 ) |  |
| } else if( PartMode == PART_2NxN ) { |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize – 1, 0 , 1log2CUSize > Log2MinCUSize ) |  |
| prediction_unit( x0, y1, log2CUSize, log2CUSize – 1, 1 , 0 ) |  |
| } else if( PartMode == PART_Nx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize - 1, log2CUSize, 0 , 1log2CUSize > Log2MinCUSize ) |  |
| prediction_unit( x1, y0, log2CUSize - 1, log2CUSize, 1 , 0 ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, log2CUSize – 1, log2CUSize – 1, 0 , 10 ) |  |
| prediction_unit( x1, y0, log2CUSize – 1, log2CUSize – 1, 1 , 10 ) |  |
| prediction_unit( x0, y1, log2CUSize – 1, log2CUSize – 1, 2 , 10 ) |  |
| prediction_unit( x1, y1, log2CUSize – 1, log2CUSize – 1, 3 , 10 ) |  |
| } |  |
| } |  |
| if( !pcm_flag ) { |  |
| transform_tree( x0, y0, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 1 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 2 ) |  |
| } |  |
| } |  |