| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F214 |
| --- | --- |

| Title: | Parallel deblocking improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaru Ikeda Teruhiko Suzuki Gate City Osaki West Tower East Tec, 1-11-1, Osaki, Shinagawa-ku , Tokyo, 141-0032 JAPAN | Tel: Email: | +81-3-5435-3308 Masaru.Ikeda@jp.sony.com Tanaka.Junichi@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution proposes two approaches to improve and develop the parallel deblocking filter (JCTVC-E181 [1] and JCTVC-E224 [2]).

- Both horizontal (H) and vertical (V) filtering process applied to reconstructed samples

This proposal makes the dependency between H and V filter reduced and then both H and V filtering process can be performed in parallel. In addition it is expected that the necessary memory to store for H filtered samples in AVC is made redundant.

- Decision by using the central non-deblocked samples in block boundary

Because the decision regarding “deblocking On/Off” and “strong/weak filter selection” is performed by the central non-deblocked samples, the decision with the central non-deblocked samples means the decision for reconstructed samples. Therefore it is expected that the necessary memory to store for the results of the decision is made redundant and the throughput for the decision decrease.

The average BD-Rate for luma in the first proposal is -0.3% for AI_HE, -0.1% for AI_LC, -0.2% for RA_HE, -0.1% for RA_LC, -0.2% for LD_HE and -0.1% for LD_LC and the performance is also improved.

The average BD-Rate for luma in the second proposal is -0.1% for AI_HE, -0.1% for AI_LC, -0.1% for RA_HE, -0.1% for RA_LC, 0.0% for LD_HE and 0.0% for LD_LC and the differences between this proposal and anchor are negligible. The decoding time is 99% for AI_HE, 98% for AI_LC, 98% for RA_HE, 96% for RA_LC, 100% for LD_HE and 97% for LD_LC and the throughput decrease.

# Introduction

Though two proposals regarding parallel deblocking filter is adopted, there are some issues to improve.

- The result of the decision for V filtering has to be stored in both LCU-based and Frame-based process.

- LCU-based process has the constraint that H filtering in #n+1 LCU has to be executed more early than V filtering in #n LCU.

We, Sony, would like to show two approaches to improve above with keeping the functionality of the parallel deblocking.

# Proposal

We, Sony, have two proposals. The first one improves and develops the parallel deblocking filter and the second one improves and simplifies the parallel deblocking filter.

### Both horizontal and vertical filtering process applied to reconstructed samples

This proposal makes both H and V filtering process applied to reconstructed samples as shown in Figure1. The output samples for luma are obtained by linear sum of both H and V filter as shown in List1. This description shows that the sample applied by either H or V filter is obtained by the output of the corresponding filter only. The filter coefficient is fixed value depending on the position from each boundary. The remarkable point is that both H and V filtering process use the same filter and decision as HM-3.0 respectively, and all parameters (β, tc and so on) are not changed. So it is possible to make this proposal apply to deblocking filter that is modified as suggested in the other contribution.

In Figure2, for example, sample#0, sample#1, sample#2, sample#3, sample#4, sample#5, sample#6, sample#7 and sample#8 are calculated as shown in List2.

The output samples for chroma are also obtained by linear sum of both H and V filter as shown in List1. But both H and V coefficient (PH and PV) for chroma are always equal to 1 at all positions from the block boundary.

Figure1: Both H and V filtering process

P OUT = PIN + C H*(PH–PIN) + C V*(PV–PIN)

PIN	: Reconstructed sample value

PH	: H filtered sample value (the same filtering process as HM-3.0)

PV	: V filtered sample value (the same filtering process as HM-3.0)

CH	: H filter coefficient

CV	: V filter coefficient

POUT	: Output sample value

List1: description of both H and V filtering process

Figure2: Linear sum of H/V filtering

Sample#0:	P0OUT = P0IN +  1*(P0H-P0IN) +  1*(P0V-P0IN)

Sample#1:	P1OUT = P1IN + 1/2*(P1H-P1IN) +  1*(P1V-P1IN)

Sample#2:	P2OUT = P2IN + 1/4*(P2H-P2IN) +  1*(P2V-P2IN)

Sample#3:	P3OUT = P3IN +  1*(P3H-P3IN) + 1/2*(P3V-P3IN)

Sample#4:	P4OUT = P4IN + 1/2*(P4H-P4IN) + 1/2*(P4V-P4IN)

Sample#5:	P5OUT = P5IN + 1/4*(P5H-P5IN) + 1/2*(P5V-P5IN)

Sample#6:	P6OUT = P6IN +  1*(P6H-P6IN) + 1/4*(P6V-P6IN)

Sample#7:	P7OUT = P7IN + 1/2*(P7H-P7IN) + 1/4*(P7V-P7IN)

Sample#8:	P8OUT = P8IN + 1/4*(P8H-P8IN) + 1/4*(P8V-P8IN)

List2: Example of filter calculation in Figure2

### Decision by using the central non-deblocked samples in block boundary

This proposal modifies the decision for reconstructed samples by changing the using samples into the central samples only (Line#3 and Line#4) as shown in Figure3. The decision shows both “deblocking on/off decision” and “strong/weak filter selection”.

Deblocking on/off decision is described as shown in List3 with the central samples only. The calculation for each line is the same one as HM-3.0.

Strong/weak filter selection for 8 lines is decided by the central samples only. If both central samples are decided as strong filter, 8 lines are decided as strong filter and if not, 8 lines are decided as weak filter. The decision for each central line is the same one as HM-3.0 and all parameters (β, tc and so on) are not changed.

di = |p2i–2*p1i+p0i|+|q2i–2*p1i+q0i| (i = 0..7)

Figure3: Central samples

|  | HM3.0 | Proposal |
| --- | --- | --- |
| Deblocking on/off | d2 + d5 < β | (d3 < (β>>1)) && (d4 < (β>>1)) |
| Strong/weak filter selection | Independent decision per line | Decision of 8 lines is AND of that of both line#3 and line#4 |

List3: Description of the deblocking on/off decision

# Results

The experiment is executed with using HM-3.0 based on JCTVC-E712 [3], and the configuration and common test condition are as JCTVC-E700[4].

Both the encoding and the decoding time are measured on the same machine as anchor, and the decoding time is measured without output of decoded YUV data.

## BD-rate and encoding/decoding time

### Both horizontal and vertical filtering process applied to reconstructed samples

The results of BD-Rate and encoding/decoding time are shown in Table3-1, Table3-2 and Table3-3. The average BD-Rate for luma is -0.3% for AI_HE, -0.1% for AI_LC, -0.2% for RA_HE, -0.1% for RA_LC, -0.2% for LD_HE and -0.1% for LD_LC, and the performance are improved a little. The average BD-Rate for chroma(U/V) is 0.1/0.1% for AI_HE, 0.3/0.3% for AI_LC, 0.1/0.1% for RA_HE, 0.1/0.2% for RA_LC, 0.2/0.1% for LD_HE and 0.0/-0.3% for LD_LC, and the performance has the slight losses but the losses are enough small.

Encoding/decoding time for all conditions is under ±1% in average. So Sony’s proposal has the almost same run-time as anchor.

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.5 | 0.1 | 0.1 | -0.3 | 0.3 | 0.2 |
| Class B | -0.4 | 0.1 | 0.1 | -0.1 | 0.3 | 0.3 |
| Class C | -0.2 | 0.1 | 0.1 | -0.1 | 0.3 | 0.3 |
| Class D | -0.2 | 0.1 | 0.1 | -0.1 | 0.3 | 0.3 |
| Class E | -0.4 | 0.0 | 0.0 | -0.1 | 0.2 | 0.2 |
| All | -0.3 | 0.1 | 0.1 | -0.1 | 0.3 | 0.3 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 104% |  |  |

Table 3-1: Results for Intra

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -0.1 | 0.2 | -0.2 | 0.2 | 0.3 |
| Class B | -0.3 | 0.0 | 0.1 | -0.1 | 0.2 | 0.2 |
| Class C | -0.1 | 0.2 | 0.0 | 0.0 | 0.2 | 0.2 |
| Class D | -0.1 | 0.1 | 0.1 | -0.1 | 0.0 | 0.2 |
| Class E |  |  |  |  |  |  |
| All | -0.2 | 0.1 | 0.1 | -0.1 | 0.1 | 0.2 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

Table 3-2: Results for RandomAccess

|  | Low delay |  |  | Low delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | 0.1 | 0.3 | 0.0 | -0.1 | -0.2 |
| Class C | -0.2 | 0.1 | -0.1 | 0.0 | 0.2 | -0.2 |
| Class D | -0.2 | 0.2 | 0.2 | -0.1 | 0.0 | -0.4 |
| Class E | -0.4 | 0.5 | -0.1 | -0.2 | -0.2 | -0.4 |
| All | -0.2 | 0.2 | 0.1 | -0.1 | 0.0 | -0.3 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |

Table 3-3: Results for LowDelay

### Decision by using the central non-deblocked samples in block boundary

The results of BD-Rate and encoding/decoding time are shown in Table3-1, Table3-2 and Table3-3. The average BD-Rate for luma is -0.1% for AI_HE, -0.1% for AI_LC, -0.1% for RA_HE, -0.1% for RA_LC, 0.0% for LD_HE and 0.0% for LD_LC and the differences between this proposal and anchor are negligible.

The encoding time is about 100% for all conditions and is the almost same run-time as anchor. The decoding time is 99% for AI_HE, 98% for AI_LC, 98% for RA_HE, 96% for RA_LC, 100% for LD_HE and 97% for LD_LC and the throughput decrease.

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | -0.2 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class C | -0.1 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
| Class D | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

Table 3-4: Results for Intra

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.1 | 0.2 | 0.1 | -0.1 | 0.2 |
| Class B | -0.1 | -0.1 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class C | -0.1 | 0.1 | 0.0 | -0.1 | 0.1 | -0.1 |
| Class D | 0.0 | -0.1 | -0.2 | -0.1 | -0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  |

Table 3-5: Results for RandomAccess

|  | Low delay |  |  | Low delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.2 | 0.3 | 0.0 | 0.0 | -0.1 |
| Class C | -0.1 | 0.1 | -0.1 | -0.1 | 0.1 | 0.0 |
| Class D | 0.0 | 0.4 | 0.1 | -0.1 | -0.1 | -0.1 |
| Class E | 0.2 | -0.1 | -0.4 | 0.1 | 0.1 | -0.3 |
| All | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 | -0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 97% |  |  |

Table 3-6: Results for LowDelay

## Subjective quality

Subjective quality was evaluated with using below conditions and equipments.

Conditions

- Kimono1:	RA_HE/QP=37

- Kimono1:	LD_LC/QP=32

- BQMall:	RA_HE/QP=37

- BQMall:	LD_LC/QP=32

- Vidyo3:	LD_HE/QP=37

- Vidyo3:	LD_LC/QP=32

Equipments

- Monitor: 	EIZO FlexScan SX3031W

- Processor : 	Intel Core i5 750 BOX

- RAM : 	DCDDR3-4GB-1333

- Graphic Card : 	ENGTX285 2DI 1GD3

The difference between Anchor and Sony’s proposal is visible in both cases, but it is difficult to say which one is better. So Sony’s proposal has no impact on subjective quality.

# Conclusion

We, Sony, recommend this proposal, especially both H and V filtering process applied to reconstructed samples, is adopted into HM for the following reasons.

- This proposal is friendly to the parallel deblocking process because the dependency between H and V filtering process decreases, and helpful to reduce the necessary buffer to store for the H filtered samples and the result of the V decision.

- It is possible to make this proposal apply to deblocking filter that is modified as suggested in the other contribution.

# References

[1] JCT-VC, “CE12 Subset2: Parallel deblocking filter”, JCTVC-E181,5th. JCT-VC Meeting, Geneva, CH, March 2011

[2] JCT-VC, “CE12: Results for parallel deblocking filter decisions”, JCTVC-E224,5th. JCT-VC Meeting, Geneva, CH, March 2011

[3] JCT-VC, “Description of CE12: deblocking filtering”, JCTVC-E712,5th. JCT-VC Meeting, Geneva, CH, March 2011

[4]JCT-VC, “Common test conditions and software reference configurations”, JCTVC-E700,5th. JCT-VC Meeting, Geneva, CH, March 2011

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).