| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F215 |
| --- | --- |

| Title: | Vertical tap length reduction to reduce line memory in deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaru Ikeda Teruhiko Suzuki Gate City Osaki West Tower East Tec, 1-11-1, Osaki, Shinagawa-ku , Tokyo, 141-0032 JAPAN | Tel: Email: | +81-3-5435-3308 Masaru.Ikeda@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to reduce the line memories required in deblocking filter. A lot of line memories are required in deblocking filter and ALF (Adaptive Loop Filter) in HM-3.0. Especially, deblocking filter is included in both high efficiency and low complexity and it is considered that deblocking filter is used in many cases, and so it is significant that the line memories in deblocking filter are reduced alone.

We, Sony, propose to reduce one line memory with keeping BD-Rate and subjective quality by reducing one pixel regarding vertical tap length. The average BD-Rate for luma in this proposal is 0.007% for AI_HE, 0.006% for AI_LC, -0.009% for RA_HE, -0.003% for RA_LC, 0.035% for LD_HE and -0.002% for LD_LC and the differences between Anchor and Sony’s proposal are negligible.

# Introduction

HW is usually proceeded in LCU-based and has line memories for deblocking filter process for block boundary between vertical LCUs. Vertical filter has the same tap length and shape as horizontal filter in current deblocking filter and maximum tap length is plus or minus 4 taps (total 8 taps) in the case of strong filter for luma as shown in Figure 1 and List1. So it is necessary to have 4 line memories in HW process. Though it is also necessary to have 4 line memories in AVC, one line memory for 4K image corresponds to 2 line memories for 1920 image in AVC. So it is significant even if one line memory is reduced.

Figure1: 8x8 Block boundary

p0 i’ = Clip1Y( ( p2i + 2*p1i + 2*p0i + 2*q0i + q1i + 4 ) >> 3 )		(1)

p1 i’ = Clip1Y( ( p2i + p1i + p0i + q0i + 2 ) >> 2 )				(2)

p2 i’ = Clip1Y( ( 2*p3i + 3*p2i + p1i + p0i + q0i + 4 ) >> 3 )			(3)

q0 i’ = Clip1Y( ( p1i + 2*p0i + 2*q0i + 2*q1i + q2i + 4 ) >> 3 )		(4)

q1 i’ = Clip1Y( ( p0i + q0i + q1i + q2i + 2 ) >> 2 )				(5)

q2 i’ = Clip1Y( ( p0i + q0i + q1i + 3*q2i + 2*q3i + 4 ) >> 3 )			(6)

List1: Formula of strong filter (i=0..7)

# Proposal

We, Sony, propose to reduce one sample (p3i in Figure 1) above horizontal edge. Sample p3i is used for calculating sample p2i’, and we, Sony, propose to use sample p1i as a substitute for sample p3i for all horizontal edges. This make one line memory (as a red line shown in Figure 2) reduced from 4 to 3 lines.

Figure2: 8x8 Block boundary

p0 i’ = Clip1Y( ( p2i + 2*p1i + 2*p0i + 2*q0i + q1i + 4 ) >> 3 )		(1)

p1 i’ = Clip1Y( ( p2i + p1i + p0i + q0i + 2 ) >> 2 )				(2)

p2 i’ = Clip1Y( ( 2*p1i + 3*p2i + p1i + p0i + q0i + 4 ) >> 3 )			(3)

= Clip1Y( ( 3*p2i + 3*p1i + p0i + q0i + 4 ) >> 3 )

q0 i’ = Clip1Y( ( p1i + 2*p0i + 2*q0i + 2*q1i + q2i + 4 ) >> 3 )		(4)

q1 i’ = Clip1Y( ( p0i + q0i + q1i + q2i + 2 ) >> 2 )				(5)

q2 i’ = Clip1Y( ( p0i + q0i + q1i + 3*q2i + 2*q3i + 4 ) >> 3 )			(6)

List2: Proposed formula of strong filter (i=0..7)

The circuit for both horizontal and vertical filter remains common because sample p1i is used as sample p3i , and the special control for this modification is not required because this modification is applied for all horizontal edges.

# Results

Experiment was carried out based on HM-3.0 and the configuration and common test condition are as JCTVC-E700 [1].

Both encoding and decoding time were measured on the platform which has Xeon X3470 CPU2.93GHz, 8GB memory and the Linux 64bit.

## BD-rate and encoding/decoding time

The results of BD-Rate and encoding/decoding time are shown in Table3-1, Table3-2 and Table3-3. The average BD-Rate for luma is 0.007% for AI_HE, 0.006% for AI_LC, -0.009% for RA_HE, -0.003% for RA_LC, 0.035% for LD_HE and -0.002% for LD_LC and the differences between Anchor and Sony’s proposal are negligible.

Encoding/decoding time for all conditions is under ±1% in average. So Sony’s proposal has the almost same run-time as anchor.

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3-1: Results for Intra

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.1 | 0.0 | -0.1 | 0.0 | 0.1 |
| Class B | 0.0 | -0.1 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class D | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3-2: Results for RandomAccess

|  | Low delay |  |  | Low delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.2 | 0.0 | 0.0 | -0.1 | -0.3 |
| Class C | 0.0 | -0.1 | -0.1 | 0.0 | 0.2 | 0.0 |
| Class D | 0.0 | 0.1 | 0.1 | 0.0 | -0.2 | 0.0 |
| Class E | 0.2 | 0.0 | 0.1 | 0.0 | -0.2 | -0.3 |
| All | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 | -0.2 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3-3: Results for LowDelay

## Subjective quality

Subjective quality was evaluated with using below conditions and equipments.

Conditions

- Kimono1:	RA_HE/QP=37

- Kimono1:	LD_LC/QP=32

- BQMall:	RA_HE/QP=37

- BQMall:	LD_LC/QP=32

- Vidyo3:	LD_HE/QP=37

- Vidyo3:	LD_LC/QP=32

Equipments

- Monitor: 	EIZO FlexScan SX3031W

- Processor : 	Intel Core i5 750 BOX

- RAM : 	DCDDR3-4GB-1333

- Graphic Card : 	ENGTX285 2DI 1GD3

The difference between Anchor and Sony’s proposal is visible, but it is difficult to say which one is better. So Sony’s proposal has no impact on subjective quality.

# Conclusion

We, Sony, recommend this proposal is adopted into HM for the following reasons.

- Line memory in HEVC processing 4K image has a large impact on HW processing.

- It is significant that the line memories in deblocking filter are reduced alone because deblocking filter (without ALF) is likely to be used in many cases.

- Reducing vertical tap length has little impact on BD-Rate and subjective quality.

# References

[1]JCT-VC, “Common test conditions and software reference configurations”, JCTVC-E700,5th. JCT-VC Meeting, Geneva, CH, March 2011

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).