| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting, Torino, Italy, 14-22 July, 2011 | Document: JCTVC-F216 |
| --- | --- |

| Title: | An Adaptive Interpolation Filtering Technique |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Faouzi Kossentini Nader Mahdi Hsan Guermazi Mohammed Ali Ben Ayed Michael Horowitz | Tel: Email: | faouzi@ebriskvideo.com nader@ebriskvideo.com hsan@ebriskvideo.com medali@ebriskvideo.com michael@ebriskvideo.com |
| Source: | eBrisk Video Inc. |  |  |

_____________________________

# Abstract

In this contribution, an Adaptive Interpolation Filtering (AIF) technique is proposed for interpolation filtering of the luminance samples of video sequences. For each video picture, the encoder first generates two new one-dimensional 8-tap filters for the first picture of the most-referenced picture list: one ½-pel filter and another (¼-pel, ¾-pel) filter, also called a ¼-pel filter. Then, the encoder decides, for each video slice, whether to use the HM 3.0 [1] default ½-pel filter or the new ½-pel filter, and whether to use the HM 3.0 [1] default ¼-pel filter or the new ¼-pel filter. The encoder then sends the resulting two-bit value of the luma_filter_mode_flag flag to the decoder. Compared to HM3.0, the proposed technique yields average BD-rate reductions of 2.3% for LDP/LC, 0.3% for LDP/HE, 1.2% for LD/LC, 0.1% for LD/HE, 0.4% for RA/LC and 0.1% for RA/HE, while maintaining the same average decoding times. The encoding time, however, increases by an average of 4%. When the same technique is also applied to the chrominance samples of video sequences, we achieved average U/V BD-rate reductions of 3.4%/2.6% for LDP/LC, 0.5%/0.8% for LDP/HE, 2.1%/1.6% for LD/LC, 0.3%/0.4% for LD/HE, 0.13%/0.14% for RA/LC and 0.1%/0.1% for RA/HE, while still maintaining the same average decoding times and an average encoding time that is 5% larger than that of HM3.0. Finally, while ALF affects the AIF performance and vice-versa, it is shown that the use of AIF allows for the reduction of the ALF decoder complexity, while still achieving similar or better performance than HM3.0.

# Introduction

This contribution describes an Adaptive Interpolation Filtering (AIF) technique for interpolation filtering during the motion compensation part of video encoding/decoding. The proposed technique is first applied to the interpolation of the luminance samples of video sequences, as described in the following section. The same technique is then applied to the chrominance samples of the same video sequences. This is described in Section 3. The syntax changes in the WD text are described in Section 4. The simulation results are presented in Section 5. In Section 6, we present simulation results that help explain the AIF-ALF interactions, and we then present a solution that exploits such interactions and that yields HM3.0 performance levels while requiring substantially less decoding complexity than that of HM3.0. Section 7 presents some conclusions.

# Proposed AIF Technique: Luminance Samples

In this section, we present some details on the proposed AIF technique, when applied to the luminance samples of video sequences, including the generation of the two new 8-tap filters, the selection procedure, and the transmission (if needed) of one or both of the newly-generated filters.

## Generation of Filter Coefficients

There are many ways of generating, for each video picture, the coefficients of the two AIF 8-tap filters. The encoder is free to employ any method, as long as the newly-generated filters have the same structure shown in Table 2.

In this contribution, we employ the filter-coefficients estimation method used in [2], as follows:

Step 1: The motion vectors (MVs) are first estimated using the HM3.0 8-tap filters. To reduce encoding complexity, only a fast version of the highest-depth motion estimation algorithm is used to generate the initial MVs.

Step 2: For the first picture of the most-referenced reference picture list, the values of the filter coefficients are estimated (through linear regression) by minimizing the energy of the prediction errors after performing motion-compensated prediction, using the MVs from Step 1.

## AIF Filter Selection

Figure 1 shows the sub-sample positions (un-shaded blocks with lower-case letters). Only one 1D filter is needed to generate the interpolated values of each of the vertical 1D positions (“di,j”, “hi,j” and “ni,j”) as well as each of the horizontal 1D positions (“ai,j”, “bi,j” and “ci,j”). For each of the remaining positions, a 2D separable filter is required that is a cascade of two 1D filters; one 1D filter for vertical filtering followed by a second 1D filter for horizontal filtering. In HM 3.0, (1) the same ½-pel filter (F_1/2) is used for vertical filtering of the “hi,j”, ”ii,j”, “ji,j” and “ki,j” positions and horizontal filtering of the “bi,j”, “fi,j”, “ji,j” and “qi,j” positions, (2) the same ¼-pel filter (F_1/4) is used for vertical filtering of the “di,j”, “ei,j”, “fi,j” and “gi,j” positions and horizontal filtering of the “ai,j”, “ei,j”, “ii,j” and “pi,j” positions and (3) the same ¾-pel filter (F_3/4) is used for vertical filtering of the “ni,j”, “pi,j”, “qi,j” and “ri,j” positions and horizontal filtering of the “ci,j”, “gi,j”, “ki,j” and “ri,j” positions. Table 1 shows the coefficient values of the F_1/2 and F_1/4 filters. Note that the F_1/4 and F_3/4 filters have the same coefficient values, although in opposite order. Table 2 summarizes the assignment of each of the filters (F_1/2, F_1/4 and F_3/4) to each of the sub-sample positions with respect to the direction of the filtering stage. Table 2 shows the coefficient of the F_1/4 and F_1/2 filters. We adopt the same HM 3.0 filtering structure, except that each of the F_1/2 and F_1/4 filters may be a default or a newly-generated filter (as shown in Table 1). Therefore, the encoder sends a two-bit value of the luma_filter_mode_flag flag (as shown in Table 3) to the decoder to indicate the result of the default-versus-new decision process. By sending the two-bit value of the luma_filter_mode_flag flag and any referenced new 1D filters (0, 1 or 2 new filters), we would have sent all of the filters needed for interpolation filtering at all 15 sub-sample positions.

Figure 1: Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for quarter sample luma interpolation

Table 1: Default (HM3.0) & newly-generated 8-tap filters (6-bit)

| ¼ | {-1, 4, -10, 57, 19, -7, 3, -1 } |
| --- | --- |
| ½ | {-1, 4, -11, 40, 40, -11, 4, -1 } |

| ¼ | {a0, a1, a2, a3, a4, a5, a6, a7} |
| --- | --- |
| ½ | {b0, b1, b2, b3, b3, b2, b1, b0} |

Table 2: Vertical/Horizontal filter assignment

| Sub-sample position | Vertical | Horizontal |
| --- | --- | --- |
| ai,j | None | F_1/4 |
| bi,j | None | F_1/2 |
| ci,j | None | F_3/4 |
| di,j | F_1/4 | None |
| ei,j | F_1/4 | F_1/4 |
| fi,j | F_1/4 | F_1/2 |
| gi,j | F_1/4 | F_3/4 |
| hi,j | F_1/2 | None |
| ii,j | F_1/2 | F_1/4 |
| ji,j | F_1/2 | F_1/2 |
| ki,j | F_1/2 | F_3/4 |
| ni,j | F_3/4 | None |
| pi,j | F_3/4 | F_1/4 |
| qi,j | F_3/4 | F_1/2 |
| ri,j | F_3/4 | F_3/4 |

Table 3: Luma_filter_mode_flag

| Luma_filter_mode_flag | F_1/4 | F_1/2 |
| --- | --- | --- |
| 0 | Default | Default |
| 1 | Default | AIF (New) |
| 2 | AIF (New) | Default |
| 3 | AIF (New) | AIF (New) |

In both H.264 & HEVC, up to two motion vectors could be used for motion compensation, with each motion vector pointing to a picture from a different reference picture list (List0 or List1).  Moreover, List0 and List1 can contain several reference pictures. Therefore, for each filter type (1/2-pel or ¼-pel), we might need to transmit a different filter for each referenced picture in each reference picture list. However, empirical data shows that similar performance levels can still be achieved (while also reducing encoding complexity substantially) by allowing the use of non-default filters only when referencing the first picture (RefIdx0) of the most referenced list. The selected reference picture list is signaled to the decoder by adding an aif_ref_pic_list flag to the bitstream.

Empirical data also shows that the use of newly-generated (non-default) filters helps very little in non-referenced B-slices (NRB slices). Therefore, the luma_filter_mode_flag flag is always set to zero for NRB slices. This reduces the number of additional computations at the encoder/decoder by essentially 50%.

## Transmission of Filter Coefficients

In order to limit the precision of the intermediate data to 16 bits, the newly-generated filter coefficients are first clipped using the clip values shown in Table 4. Then, the clipped coefficients of each newly-generated 8-tap AIF filter are encoded using 6-bit quantization of the differences between the values of the newly-generated filter coefficients and the corresponding values of the HM3.0 filter coefficients.

Table 4: Clip values of the AIF filter coefficients

| Coefficient index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| clip value | 7 | 15 | 31 | 63 | 63 | 31 | 15 | 7 |

# 3. Proposed AIF Technique: Chrominance Samples

The adaptive interpolation filtering of the chrominance sub-samples follows the same design as in the interpolation filtering of the luminance sub-samples, except that the interpolation is now applied to 63 sub-sample positions. Referring to Figure 2, we can summarize the proposed technique as follows:

- We adopt the same HM 3.0 filtering structure, except that each of the four default filters may now be a default or a newly-generated filter; In other words, the encoder sends a 4-bit value for the chroma_filter_mode_flag flag to the decoder to indicate whether each of the four default filters should be updated. Up to 4 filters per U+V of a single video slice may be changed at the decoder.

- The decoder is allowed to use any of the non-default filters only when referencing the first picture of the most-referenced reference picture list.

- The chroma_filter_mode_flag flag is always set to zero for NRB slices.

Moreover, to reduce further the computational complexity, we set chroma_filter_mode_flag to zero whenever luma_filter_mode_flag is set to zero. In other words, we employ only the default chrominance filters when only the default luminance filters are used. When luma_filter_mode_flag is not set to zero, chroma_filter_mode_flag may be set to zero or non-zero (see Figure 3).

Figure 2: Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for eighth sample chroma interpolation.

Table 5: Chroma_filter_mode_flag

| Chroma_filter_mode_flag | F_1/8 | F_1/4 | F_3/8 | F_1/2 |
| --- | --- | --- | --- | --- |
| 0 | Default | Default | Default | Default |
| 1 | Default | Default | Default | AIF/New |
| 2 | Default | Default | AIF/New | Default |
| 3 | Default | Default | AIF/New | AIF/New |
| 4 | Default | AIF/New | Default | Default |
| 5 | Default | AIF/New | Default | AIF/New |
| 6 | Default | AIF/New | AIF/New | Default |
| 7 | Default | AIF/New | AIF/New | AIF/New |
| 8 | AIF/New | Default | Default | Default |
| 9 | AIF/New | Default | Default | AIF/New |
| 10 | AIF/New | Default | AIF/New | Default |
| 11 | AIF/New | Default | AIF/New | AIF/New |
| 12 | AIF/New | AIF/New | Default | Default |
| 13 | AIF/New | AIF/New | Default | AIF/New |
| 14 | AIF/New | AIF/New | AIF/New | Default |
| 15 | AIF/New | AIF/New | AIF/New | AIF/New |

Figure 3: Luma/Chroma AIF dependency

# 4. Proposed WD Text Changes

### 4. 1 Proposed WD Text Changes for luma (highlighted in GREEN)

### 4.1.1. Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| ... | ue(v) |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) { |  |
| dec_ref_pic_marking( ) |  |
| aif_param() |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| ... |  |
| } |  |

### 4.1.2. Adaptive interpolation filter parameter syntax

| aif_param( ) { | Descriptor |
| --- | --- |
| use_generated_filter_flag | u(1) |
| if (use_generated_filter_flag) { |  |
| aif_ref_pic_list | u(1) |
| luma_filter_mode_flag | u(2) |
| if((luma_filter_mode_flag == 1) ¦¦ (luma_filter_mode_flag == 3)){ |  |
| for (i =0; i < 4; i ++ ) { |  |
| Luma_diff_filter_coef_h[i] | se(v) |
| } |  |
| } |  |
| if((luma_filter_mode_flag == 2) ¦¦ (luma_filter_mode_flag == 3)){ |  |
| for (i =0; i < 8 ; i ++ ) { |  |
| Luma_diff_filter_coef_q[i] | se(v) |
| } |  |
| } |  |
| } |  |

### 4.1.3. Adaptive interpolation filter parameter semantics

use_generated_filter_flag specifies whether adaptive interpolation filter applies or not for the current slice.

aif_ref_pic_list specifies for which reference picture list adaptive interpolation filter must be applied.

luma_filter_mode_flag specifies the type of the adaptive interpolation filter of luma samples, as shown in Table x-xx. The value of luma_filter_mode_flag shall be in the range of 0 to 3, inclusive.

Luma_diff_filter_coef_h specifies the predicted array of the 1/2-pel of luma filter coefficients.

Luma_diff_filter_coef_q specifies the predicted array of the 1/4-pel of luma filter coefficients.

Table x-xx – Relationship between luma_filter_mode_flag and the filter type for half and quarter positions

| luma_filter_mode_flag | 1/2 | 1/4 |
| --- | --- | --- |
| 0 | D | D |
| 1 | N | D |
| 2 | D | N |
| 3 | N | N |

D: Default; N: New/AIF

### 4.1.4. Derivation of the interpolation filter coefficients process

This process is invoked at the beginning of the decoding process for each P, SP, or B slice and when nal_ref_idc is not equal to 0.

Inputs to this process are:

–	the luma_filter_mode_flag flag,

–	the predicted filter coefficients arrays luma_diff_filter_coef_h and luma_diff_filter_coef_q.

Outputs of this process are:

–	filter coefficients arrays lumaFH and lumaFQ.

The lumaFH and lumaFQ arrays are derived as follows,

–	If the luma_filter_mode_flag flag is equal to 1or the luma_filter_mode_flag flag is equal to 3,

lumaFH[0] and lumaFH[7] are set to luma_diff_filter_coef_h[0] – 1

lumaFH[1] and lumaFH[6] are set to luma_diff_filter_coef_h[1] + 4

lumaFH[2] and lumaFH[5] are set to luma_diff_filter_coef_h[2]  -11

lumaFH[3] and lumaFH[4] are set to luma_diff_filter_coef_h[3] + 40

–	Otherwise, if the luma_filter_mode_flag flag is equal to 2 or the luma_filter_mode_flag flag is equal to 3,

lumaFQ[0] is set to luma_diff_filter_coef_q[0]  - 1

lumaFQ[1] is set to luma_diff_filter_coef_q[1]  + 4

lumaFQ[2] is set to luma_diff_filter_coef_q[2]  - 10

lumaFQ[3] is set to luma_diff_filter_coef_q[3]  + 57

lumaFQ[4] is set to luma_diff_filter_coef_q[4]  + 19

lumaFQ[5] is set to luma_diff_filter_coef_q[5]  - 7

lumaFQ[6] is set to luma_diff_filter_coef_q[6]  + 3

lumaFQ[7] is set to luma_diff_filter_coef_q[7]  - 1

### 4.1.5. Luma sample interpolation process

Inputs to this process are:

–	a luma location in full-sample units ( xIntL, yIntL ),

–	a luma location in fractional-sample units ( xFracL, yFracL ),

–	the luma reference sample array refPicLXL,

–	filter coefficients arrays lumaFH and lumaFQ,

–	the aif_ref_pic_list,

–	a reference picture list RefPicListX,

–	a reference index refIdxLX,

–	the luma_filter_mode_flag flag.

Output of this process is a predicted luma sample value predSampleLXL[ xL, yL ]

Figure x-x – Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for quarter sample luma interpolation

In Figure x-, the positions labelled with upper-case letters Ai, j within shaded blocks represent luma samples at full-sample locations inside the given two-dimensional array refPicLXL of luma samples. These samples may be used for generating the predicted luma sample value predSampleLXL[ xL, yL ]. The locations ( xAi, j, yAi, j ) for each of the corresponding luma samples Ai, j inside the given array refPicLXL of luma samples are derived as follows:

xAi, j = Clip3( 0, PicWidthInSamplesL – 1, xIntL +i )

yAi, j = Clip3( 0, PicHeightInSamplesL – 1, yIntL +j )

Variables shift1, shift2, shift3, offset1 and offset2 are derived as follows.

–	The variable shift1 is set equal to BitDepthY – 8, the variable shift2 is set equal to BitDepthY – 2, and the variable shift3 is set equal to 14 – BitDepthY.

–	If the variable shift1 is equal to 0, the variable offset1 is set equal to 0, otherwise, the variable offset1 is set equal to 1 << ( shift1 – 1 ).

–	The variable offset2 is set equal to 1 << ( shift2 – 1 ).

The arrays FH and FQ are derived as follows.

–	If the luma_filter_mode_flag is not equal to 0 and the RefPicListX is equal to aif_ref_pic_list and the refIdxLX is equal to 0,

FH array values are set to the lumaFH values,

FQ array values are set to the lumaFQ values.

–	Otherwise,

FH[0] and FH[7] are set to – 1

FH[1] and FH[6] are set to  4

FH[2] and FH[5] are set to 11

FH[3] and FH[4] are set to 40

FQ[0] and FQ[7]  are set to -1

FQ[1] is set to 4

FQ[2] is set to -10

FQ[3] is set to 57

FQ[4] is set to 19

FQ[5] is set to -7

FQ[6] is set to 3

Given the luma samples Ai, j at full-sample locations ( xAi, j, yAi, j ), the luma samples ‘a0,0’ to ‘r0,0’ at fractional sample positions are derived by the following rules.

–	The samples labelled a0,0, b0,0, c0,0, d0,0, h0,0, and n0,0 shall be derived by applying the 8-tap filter to the nearest integer position samples and clipping the filtered value:

a0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( - FQ[0]*A-3,0 + FQ[1]*A-2,0 + FQ[2]*A-1,0 + FQ[4]*A0,0 + lFQ[5]*A1,0 + FQ[6]*A2,0 + FQ[7]*A3,0 + FQ[7]*A4,0 + offset1 ) >> shift1 )

b0,0 = Clip3( 0, ( 1 << 14 ) – 1, (FH[0]*A-3,0 + FH[1]*A-2,0 + FH[2]*A-1,0 + FH[3]*A0,0 +  FH[4]*A1,0  + FH[5]*A2,0 + FH[6]*A3,0 + FH[7]*A4,0 + offset1 ) >> shift1 )	                         	 (0-1)

c0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( FQ[7]*A-3,0 + FQ[6]*A-2,0 + FQ[5]*A-1,0 + FQ[4]*A0,0 +  FQ[3]*A1,0 + FQ[2]*A2,0 + FQ[1]*A3,0 + FQ[0]*A4,0 + offset1 ) >> shift1 )

d0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( FQ[0]*A0,-3 + FQ[1]*A0,-2 + FQ[2]*A0,-1 + FQ[3]*A0,0 +
					 FQ[4]*A0,1 - FQ[5]*A0,2 + FQ[6]*A0,3 + FQ[7]*A0,4 + offset1 ) >> shift1 )

h0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( FH[0]*A0,-3 + FH[1]*A0,-2 + FH[2]*A0,-1 + FH[3]*A0,0 
					 + FH[4]*A0,1 + FH[5]*A0,2 + FH[6]*A0,3 + FH[7]*A0,4 + offset1 ) >> shift1 )

n0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( FQ[7]*A0,-3 + FQ[6]*A0,-2 + FQ[5]*A0,-1 + FQ[4]*A0,0 +
					 FQ[3]*A0,1 - FQ[2]*A0,2 + FQ[1]*A0,3 + FQ[0]*A0,4 + offset1 ) >> shift1 )

–	The samples labelled e0,0, f0,0, g0,0, i0,0, j0,0, k0,0, p0,0, q0,0 and r0,0 shall be derived by first calculating intermediate values denoted as d1i,0, h1i,0 and n1i,0 where i = -3..4 by applying the 8-tap filter to the nearest integer position samples in vertical direction: [Ed: (WJ) horizontal first yields the same result]

d1i,0 =  FQ[0]*Ai,-3 + FQ[1]*Ai,-2 + FQ[2]*Ai,-1 + FQ[3]*Ai,0 + FQ[4]*Ai,1 + FQ[5]*Ai,2 + FQ[6]*Ai,3 + FQ[7]* Ai,4

h1i,0 =  FH[0]*Ai,-3 + FH[1]*Ai,-2 + FH[2]*Ai,-1 + FH[3]*Ai,0 + FH[4]*Ai,1 + FH[5]*Ai,2 + FH[6]*Ai,3 + FH[7]*Ai,4

n1i,0 =  FQ[7]*Ai,-3 + FQ[6]*Ai,-2 + FQ[5]*Ai,-1 + FQ[4]*Ai,0 + FQ[3]*Ai,1 + FQ[2]*Ai,2 + FQ[1]*Ai,3 + FQ[0]*Ai,4

–	The final prediction values e0,0, f0,0, g0,0, i0,0, j0,0, k0,0, p0,0, q0,0 and r0,0 shall be derived by applying the 8-tap filter to the intermediate values d1i,0, h1i,0 and n1i,0 where i = -3..4 in horizontal direction:

e0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( FQ[0]*d1-3,0 + FQ[1]*d1-2,0 + FQ[2]*d1-1,0 + FQ[3]*d10,0 
					 + FQ[4]*d11,0 + FQ[5]*d12,0 + FQ[6]*d13,0 + FQ[7]*d14,0 + offset2 ) >> shift2 )

f0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( FH[0]*d1-3,0 + FH[1]*d1-2,0 + FH[2]*d1-1,0 + FH[3]*d10,0
					 + FH[4]*d11,0 + FH[5]*d12,0 + FH[6]*d13,0 + FH[7]*d14,0 + offset2 ) >> shift2 )

g0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( FQ[7]*d1-3,0 ++ FQ[6]*d1-2,0 + FQ[5]*d1-1,0 + FQ[4]*d10,0 
					 + FQ[3]*d11,0 + FQ[2]*d12,0 + FQ[1]*d13,0 + FQ[0]*d14,0 + offset2 ) >> shift2 )

i0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( FQ[0]*h1-3,0 + FQ[1]*h1-2,0 + FQ[2]*h1-1,0 + FQ[3]*h10,0
					 + FQ[4]*h11,0 + FQ[5]*h12,0 + FQ[6]*h13,0 + FQ[7]*h14,0 + offset2 ) >> shift2 )

j0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( FH[0]*h1-3,0 + FH[1]*h1-2,0 + FH[2]*h1-1,0 + FH[3]*h10,0
					 + FH[4]*h11,0 + FH[5]*h12,0 + FH[6]*h13,0 + FH[7]*h14,0 + offset2 ) >> shift2 )

k0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( FQ[7]*h1-3,0 + FQ[6]*h1-2,0 + FQ[5]*h1-1,0 + FQ[4]*h10,0 
					+ FQ[3]*h11,0 + FQ[2]*h12,0 + FQ[1]*h13,0 + FQ[0]*h14,0 + offset2 ) >> shift2 )

p0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( FQ[0]*n1-3,0 + FQ[1]*n1-2,0 + FQ[2]*n1-1,0 + FQ[3]*n10,0 
					 + FQ[4]*n11,0 + FQ[5]*n12,0 + FQ[6]*n13,0 + FQ[7]*n14,0 + offset2 ) >> shift2 )

q0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( FH[0]*n1-3,0 + FH[1]*n1-2,0 + FH[2]*n1-1,0 + FH[3]*n10,0
					 + FH[4]*n11,0 + FH[5]*n12,0 + FH[6]*n13,0 + FH[7]*n14,0 + offset2 ) >> shift2 )

r0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( FQ[7]*n1-3,0 + FQ[6]*n1-2,0 + FQ[5]*n1-1,0 + FQ[4]*n10,0 
					 + FQ[3]*n11,0 + FQ[2]*n12,0 + FQ[1]*n13,0 + FQ[0]*n14,0 + offset2 ) >> shift2 )

The positions labelled with lower-case letters within un-shaded blocks represent luma samples at quarter-pel sample fractional locations. The luma location offset in fractional-sample units ( xFracL, yFracL ) specifies which of the generated luma samples at full-sample and fractional-sample locations is assigned to the predicted luma sample value predSampleLXL[ xL, yL ]. This assignment is done according to Table x-. The value of predSampleLXL[ xL, yL ] shall be the output.

Table x-x – Assignment of the luma prediction sample predSampleLXL[ xL, yL ]

| xFracL | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracL | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 |
| predSampleLXL[ xL, yL ] | A << shift3 | d | h | n | a | e | i | p | b | f | j | q | c | g | k | r |

### 4. 2 Proposed WD Text Changes for Chroma

### 4.2.1. Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| ... | ue(v) |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) { |  |
| dec_ref_pic_marking( ) |  |
| aif_param() |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| ... |  |
| } |  |

### 4.2.2. Adaptive interpolation filter parameter syntax

| aif_param( ) { | Descriptor |
| --- | --- |
| use_generated_filter_flag | u(1) |
| if (use_generated_filter_flag) { |  |
| aif_ref_pic_list | u(1) |
| luma_filter_mode_flag | u(2) |
| if((luma_filter_mode_flag == 1) ¦¦ (luma_filter_mode_flag == 3)){ |  |
| for (i =0; i < 4; i ++ ) { |  |
| Luma_diff_filter_coef_h[i] | se(v) |
| } |  |
| } |  |
| if((luma_filter_mode_flag == 2) ¦¦ (luma_filter_mode_flag == 3)){ |  |
| for (i =0; i < 8 ; i ++ ) { |  |
| Luma_diff_filter_coef_q[i] | se(v) |
| } |  |
| } |  |
| chroma_filter_mode_flag | u(4) |
| if ((chroma_filter_mode_flag == 1) ¦¦ (chroma_filter_mode_flag == 3) ¦¦ (chroma_filter_mode_flag == 5) ¦¦ (chroma_filter_mode_flag == 7)¦¦ (chroma_filter_mode_flag == 9) ¦¦ (chroma_filter_mode_flag == 11)¦¦ (chroma_filter_mode_flag == 13) ¦¦ (chroma_filter_mode_flag == 15)){ |  |
| for (i =0; i < 2; i ++ ) { |  |
| chroma_diff_filter_coef_0[i] | se(v) |
| } |  |
| } |  |
| if ((chroma_filter_mode_flag == 2) ¦¦ (chroma_filter_mode_flag == 3) ¦¦ (chroma_filter_mode_flag == 6) ¦¦ (chroma_filter_mode_flag == 7)¦¦ (chroma_filter_mode_flag == 10) ¦¦ (chroma_filter_mode_flag == 11)¦¦ (chroma_filter_mode_flag == 14) ¦¦ (chroma_filter_mode_flag == 15)){ |  |
| for (i =0; i < 4; i ++ ) { |  |
| chroma_diff_filter_coef_1[i] | se(v) |
| } |  |
| } |  |
| if ((chroma_filter_mode_flag == 4) ¦¦ (chroma_filter_mode_flag == 5) ¦¦ (chroma_filter_mode_flag == 6) ¦¦ (chroma_filter_mode_flag == 7)¦¦ (chroma_filter_mode_flag == 12) ¦¦ (chroma_filter_mode_flag == 13)¦¦ (chroma_filter_mode_flag == 14) ¦¦ (chroma_filter_mode_flag == 15)){ |  |
| for (i =0; i < 4; i ++ ) { |  |
| chroma_diff_filter_coef_2[i] | se(v) |
| } |  |
| } |  |
| if ((chroma_filter_mode_flag == 8) ¦¦ (chroma_filter_mode_flag == 9) ¦¦ (chroma_filter_mode_flag == 10) ¦¦ (chroma_filter_mode_flag == 11)¦¦ (chroma_filter_mode_flag == 12) ¦¦ (chroma_filter_mode_flag == 13)¦¦ (chroma_filter_mode_flag == 14) ¦¦ (chroma_filter_mode_flag == 15)){ |  |
| for (i =0; i < 2; i ++ ) { |  |
| chroma_diff_filter_coef_3[i] | se(v) |
| } |  |
| } |  |
| } |  |

### 4.2.3. Adaptive interpolation filter parameter semantics

use_generated_filter_flag specifies whether adaptive interpolation filter applies or not for the current slice.

aif_ref_pic_list specifies for which reference picture list adaptive interpolation filter must be applied.

luma_filter_mode_flag specifies the type of the adaptive interpolation filter of luma samples, as shown in Table x-xx. The value of luma_filter_mode_flag shall be in the range of 0 to 3, inclusive.

Luma_diff_filter_coef_h specifies the predicted array of the 1/2-pel of luma filter coefficients.

Luma_diff_filter_coef_q specifies the predicted array of the 1/4-pel of luma filter coefficients.

Table x-xx – Relationship between luma_filter_mode_flag and the filter type for half and quarter positions

| luma_filter_mode_flag | 1/2 | ¼ |
| --- | --- | --- |
| 0 | D | D |
| 1 | N | D |
| 2 | D | N |
| 3 | N | N |

D: Default; N: New/AIF

chroma_filter_mode_flag specifies the type of the adaptive interpolation filter of chroma samples, as shown in Table y-yy. The value of chroma _filter_mode_flag shall be in the range of 0 to 15, inclusive.

chroma _diff_filter_coef_0 specifies the predicted array of the 1/2-pel of chroma filter coefficients.

chroma _diff_filter_coef_1 specifies the predicted array of the 1/4-pel of chroma filter coefficients.

chroma _diff_filter_coef_2 specifies the predicted array of the 1/8-pel of chroma filter coefficients.

chroma _diff_filter_coef_3 specifies the predicted array of the 3/8-pel of chroma filter coefficients.

Table y-yy – Relationship between chroma _filter_mode_flag and the filter type for half, quarter and eighth positions

| chroma_filter_mode_flag | 1/8 | ¼ | 3/8 | 1/2 |
| --- | --- | --- | --- | --- |
| 0 | D | D | D | D |
| 1 | D | D | D | N |
| 2 | D | D | N | D |
| 3 | D | D | N | N |
| 4 | D | N | D | D |
| 5 | D | N | D | N |
| 6 | D | N | N | D |
| 7 | D | N | N | N |
| 8 | N | D | D | D |
| 9 | N | D | D | N |
| 10 | N | D | N | D |
| 11 | N | D | N | N |
| 12 | N | N | D | D |
| 13 | N | N | D | N |
| 14 | N | N | N | D |
| 15 | N | N | N | N |

D: Default; N: New/AIF

### 4.2.4. Derivation of the interpolation filter coefficients process

This process is invoked at the beginning of the decoding process for each P, SP, or B slice and when nal_ref_idc is not equal to 0.

Inputs to this process are:

–	the luma_filter_mode_flag flag,

–	the predicted filter coefficients arrays luma_diff_filter_coef_h and luma_diff_filter_coef_q.

–	the chroma_filter_mode_flag flag,

–	the predicted filter coefficients arrays chroma _diff_filter_coef_0, chroma _diff_filter_coef_1, chroma _diff_filter_coef_2 and chroma _diff_filter_coef_3.

Outputs of this process are:

–	filter coefficients arrays lumaFH and lumaFQ.

–	filter coefficients arrays chromaF0, chromaF1, chromaF2 and chromaF3.

The lumaFH and lumaFQ arrays are derived as follows,

–	If the luma_filter_mode_flag flag is equal to 1or the luma_filter_mode_flag flag is equal to 3,

lumaFH[0] and lumaFH[7] are set to luma_diff_filter_coef_h[0] – 1

lumaFH[1] and lumaFH[6] are set to luma_diff_filter_coef_h[1] + 4

lumaFH[2] and lumaFH[5] are set to luma_diff_filter_coef_h[2]  -11

lumaFH[3] and lumaFH[4] are set to luma_diff_filter_coef_h[3] + 40

–	Otherwise, if the luma_filter_mode_flag flag is equal to 2 or the luma_filter_mode_flag flag is equal to 3,

lumaFQ[0] is set to luma_diff_filter_coef_q[0]  - 1

lumaFQ[1] is set to luma_diff_filter_coef_q[1]  + 4

lumaFQ[2] is set to luma_diff_filter_coef_q[2]  - 10

lumaFQ[3] is set to luma_diff_filter_coef_q[3]  + 57

lumaFQ[4] is set to luma_diff_filter_coef_q[4]  + 19

lumaFQ[5] is set to luma_diff_filter_coef_q[5]  - 7

lumaFQ[6] is set to luma_diff_filter_coef_q[6]  + 3

lumaFQ[7] is set to luma_diff_filter_coef_q[7]  - 1

The chromaF0, chromaF1, chromaF2 and lumaF3 arrays are derived as follows,

–	If chroma_filter_mode_flag & 1 is equal to 1

chromaF0[0] and chromaF0[3] are set to chroma_diff_filter_coef_0[0] – 4

chromaF0[1] and chromaF0[2] are set to chroma_diff_filter_coef_0[1] + 36

–	If chroma_filter_mode_flag & 2 is equal to 1

chromaF1[0] is set to chroma_diff_filter_coef_1[0] - 5

chromaF1[1] is set to chroma_diff_filter_coef_1[1] + 46

chromaF1[2] is set to chroma_diff_filter_coef_1[2] + 27

chromaF1[3] is set to chroma_diff_filter_coef_1 [3] – 4

–	If chroma_filter_mode_flag & 4 is equal to 1

chromaF2[0] is set to chroma_diff_filter_coef_2[0] - 4

chromaF2[1] is set to chroma_diff_filter_coef_2[1] + 54

chromaF2[2] is set to chroma_diff_filter_coef_2[2] + 16

chromaF2[3] is set to chroma_diff_filter_coef_2 [3] – 2

–	If chroma_filter_mode_flag & 8 is equal to 1

chromaF3[0] is set to chroma_diff_filter_coef_3[0] - 3

chromaF3[1] is set to chroma_diff_filter_coef_3[1] + 60

chromaF3[2] is set to chroma_diff_filter_coef_3[2] + 8

chromaF3[3] is set to chroma_diff_filter_coef_3 [3] - 4

### 4.2.5. Chroma sample interpolation process

Inputs to this process are:

–	a chroma location in full-sample units ( xIntC, yIntC ),

–	a chroma location in fractional-sample units ( xFracC, yFracC ),

–	the chroma reference sample array refPicLXC,

–	filter coefficients arrays chromaF0, chromaF1, chromaF2 and chromaF3,

–	the aif_ref_pic_list,

–	a reference picture list RefPicListX,

–	a reference index refIdxLX,

–	the chroma_filter_mode_flag flag.

Output of this process is a predicted chroma sample value predSampleLXC[ xC, yC ]

Figure y-y – Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for eighth sample chroma interpolation

In Figure x-y, the positions labelled with upper-case letters Bi, j within shaded blocks represent chroma samples at full-sample locations inside the given two-dimensional array refPicLXC of chroma samples. These samples may be used for generating the predicted chroma sample value predSampleLXC[ xC, yC ]. The locations ( xBi, j, yBi, j ) for each of the corresponding chroma samples Bi, j inside the given array refPicLXC of chroma samples are derived as follows:

xBi, j = Clip3( 0, PicWidthInSamplesC – 1, xIntC +i )

yBi, j = Clip3( 0, PicHeightInSamplesC – 1, yIntC +j )

Variables shift1, shift2, shift3, offset1 and offset2 are derived as follows.

–	The variable shift1 is set equal to BitDepthC – 8, the variable shift2 is set equal to BitDepthC – 2, and the variable shift3 is set equal to 14 – BitDepthC.

–	If the variable shift1 is equal to 0, the variable offset1 is set equal to 0, otherwise, the variable offset1 is set equal to 1 << ( shift1 – 1 ).

–	The variable offset2 is set equal to 1 << ( shift2 – 1 ).

The arrays F0, F1, F2 and F3 are derived as follows.

–	If the luma_filter_mode_flag is not equal to 0 and the RefPicListX is equal to aif_ref_pic_list and the refIdxLX is equal to 0,

F0 array values are set to the chromaF0 values,

F1 array values are set to the chromaF1 values,

F2 array values are set to the chromaF2 values,

F3 array values are set to the chromaF3 values.

–	Otherwise,

F0[0] and F0[3] are set to – 4

F0[1] and F0[2] are set to  36

F1[0] is set to - 5

F1[1] is set to 46

F1[2] is set to 27

F1[3] is set to – 4

F2[0] is set to - 4

F2[1] is set to 54

F2[2] is set to 16

F2[3] is set to – 2

F3[0] is set to - 3

F3[1] is set to 60

F3[2] is set to 8

F3[3] is set to - 4

Given the chroma samples Bi, j at full-sample locations ( xBi, j, yBi, j ), the chroma samples ‘ab0,0’ to ‘hh0,0’ at fractional sample positions are derived by the following rules.

–	The samples labelled ab0,0, ac0,0, ad0,0, ae0,0, af0,0, ag0,0, and ah0,0 shall be derived by applying the 4-tap filter to the nearest integer position samples and clipping the filtered value:

ab0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( F3[0]*B-1,0 + F3[1]*B0,0 +F3[2]*B1,0 +F3[3]* B2,0 + offset1 ) >> shift1 )

ac0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( F2[0]*B-1,0 + F2[1]*B0,0 + F2[2]*B1,0 +F2[3]*B2,0 + offset1 ) >> shift1 )

ad0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( F1[0]*B-1,0 + F1[1]*B0,0 + F1[2]*B1,0 +F1[3]*B2,0 + offset1 ) >> shift1 )

ae0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( F0[0]*B-1,0 + F0[1]*B0,0 + F0[2]*B1,0 +F0[3]*B2,0 + offset1 ) >> shift1 )

af0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( F1[3]*B-1,0 + F1[2]*B0,0 + F1[1]*B1,0 + F1[0]*B2,0 + offset1 ) >> shift1 )

ag0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( F2[3]*B-1,0 + F2[2]*B0,0 + F2[1]*B1,0 +F2[0]*B2,0 + offset1 ) >> shift1 )

ah0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( F3[3]*B-1,0 + F3[2]*B0,0 + F3[1]*B1,0 +F3[0]*B2,0 + offset1 ) >> shift1 )

–	The samples labelled ba0,0, ca0,0, da0,0, ea0,0, fa0,0, ga0,0, and ha0,0 shall be derived by applying the 4-tap filter to the nearest integer position samples and clipping the filtered value:

ba0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( F3[0]*B0,-1 + F3[1]*B0,0 + F3[2]*B0,1 +F3[3]*B0,2 + offset1 ) >> shift1 )

ca0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( F2[0]*B0,-1 + F2[1]*B0,0 + F2[2]*B0,1 +F2[3]*B0,2 + offset1 ) >> shift1 )

da0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( F1[0]*B0,-1 + F1[1]*B0,0 + F1[2]*B0,1 +F1[3]*B0,2 + offset1 ) >> shift1 )

ea0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( F0[0]*B0,-1 + F0[1]*B0,0 + F0[2]*B0,1 +F0[3]*B0,2 + offset1 ) >> shift1 )

fa0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( F1[3]*B0,-1 + F1[2]*B0,0 + F1[1]*B0,1 + F1[0]*B0,2 + offset1 ) >> shift1 )

ga0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( F2[3]*B0,-1 + F2[2]*B0,0 + F2[1]*B0,1 +F2[0]*B0,2 + offset1 ) >> shift1 )

ha0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( F3[3]*B0,-1 + F3[2]*B0,0 + F3[1]*B0,1 +F3[0]*B0,2 + offset1 ) >> shift1 )

–	The samples labelled Xb0,0, Xc0,0, Xd0,0, Xe0,0, Xf0,0, Xg0,0 and Xh0,0 for X being replaced by b, c, d, e, f, g and h, respectively, shall be derived by first calculating intermediate values denoted as bai,0, cai,0, dai,0, eai,0, fai,0, gai,0 and hai,0 where i = -1..2 by applying the 4-tap filter to the nearest integer position samples in vertical direction: [Ed: (WJ) horizontal first yields the same result]

bai,0 =  F3[0]*B0,-1 + F3[1]*B0,0 + F3[2]*B0,1 + F3[3]*B0,2

cai,0 =  F2[0]*B0,-1 + F2[1]*B0,0 + F2[2]*B0,1 + F2[3]*B0,2

dai,0 =  F1[0]*B0,-1 + F1[1]*B0,0 + F1[2]*B0,1 + F1[3]*B0,2

eai,0 =  F0[0]*B0,-1 + F0[1]*B0,0 + F0[2]*B0,1 + F0[3]*B0,2

fai,0 =  F1[3]*B0,-1 + F1[2]*B0,0 + F1[1]*B0,1 + F1[0]*B0,2

gai,0 =  F2[3]*B0,-1 + F2[2]*B0,0 + F2[1]*B0,1 + F2[0]*B0,2

hai,0 =  F3[3]*B0,-1 F3[2]*B0,0 + F3[1]*B0,1 + F3[0]*B0,2

–	The final prediction values Xb0,0, Xc0,0, Xd0,0, Xe0,0, Xf0,0, Xg0,0 and Xh0,0 for X being replaced by b, c, d, e, f, g and h, respectively, shall be derived by applying the 4-tap filter to the intermediate values Xai,0 where i = -1..2 in horizontal direction:

Xb0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( F3[0]*Xa-1,0 + F3[1]*Xa0,0 + F3[2]*Xa1,0 + F3[3]*Xa2,0 + offset2 ) >> shift2 )

Xc0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( F2[0]*Xa-1,0 + F2[1]*Xa0,0 + F2[2]*Xa1,0 + F2[3]*Xa2,0 + offset2 ) >> shift2 )

Xd0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( F1[0]*Xa-1,0 + F1[1]*Xa0,0 + F1[2]*Xa1,0 + F1[3]*Xa2,0 + offset2 ) >> shift2 )

Xe0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( F0[0]*Xa-1,0 + F0[1]*Xa0,0 + F0[2]*Xa1,0 + F0[3]*Xa2,0 + offset2 ) >> shift2 )

Xf0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( F1[3]*Xa-1,0 + F1[2]*Xa0,0 + F1[1]*Xa1,0 + F1[0]*Xa2,0 + offset2 ) >> shift2 )

Xg0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( F2[3]*Xa-1,0 + F2[2]*Xa0,0 + F2[1]*Xa1,0 + F2[0]*Xa2,0 + offset2 ) >> shift2 )

Xh0,0 = Clip3( 0, ( 1 << 14 ) – 1, ( F3[3]*Xa-1,0 + F3[2]*Xa0,0 + F3[1]*Xa1,0 + F3[0]*Xa2,0 + offset2 ) >> shift2 )

The positions labelled with lower-case letters within un-shaded blocks represent chroma samples at eighth-pel sample fractional locations. The chroma location offset in fractional-sample units ( xFracC, yFracC ) specifies which of the generated chroma samples at full-sample and fractional-sample locations is assigned to the predicted chroma sample value predSampleLXC[ xC, yC ]. This assignment is done according to Table y-y-y. The value of predSampleLXC[ xC, yC ] shall be the output.

Table y-y – Assignment of the chroma prediction sample predSampleLXC[ xC, yC ] for ( X, Y ) being replaced by ( 1, b ), ( 2, c ), ( 3, d ), ( 4, e ), ( 5, f ), ( 6, g ), and ( 7, h ), respectively

| xFracC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | X | X | X | X | X | X | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracC | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| predSampleLXC[ xC, yC ] | B << shift3 | ba | ca | Da | Ea | fa | Ga | ha | aY | bY | cY | dY | eY | fY | gY | hY |

# 5. Experimental Results

The simulation experiments were conducted following the testing conditions in JCTVC-E700 [3].

The proposed AIF technique was compared to the HM3.0 technique (1) when it is applied to the luminance samples only and (2) when it is applied to both the luminance and chrominance samples. The coding efficiency (BD-Rate) numbers were obtained for the Low Delay P, Low Delay and Random Access configurations, and they are shown in Table 6 and Table 7 for (1) and (2), respectively. The detailed BD-Rate results for (1) for the Luminance pictures are also shown in Table 8; those for (2) for the chrominance pictures are shown in Table 9 (U) and Table 10 (V).

The encoding/decoding times were estimated using a PC with the following configuration:

- CPU: Intel Core 2 Quad Q8300 @2.50 GHz.

- RAM: 4 Go

- OS:  Windows XP Professional.

It is clear, from the tables, that the proposed AIF method achieves coding efficiency gains over the HM3.0 fixed filtering technique, especially for the LC configurations. It is also important to note that the proposed AIF technique performs very well (by an average of more than 2%) for the new test video sequences (i.e., Nebuta and SteamLocomotive).

The required additional complexity at the encoder and/or decoder can be quite small. Except for the decoding of at most two new filters (the ¼-pel and ¾-pel AIF filters employ the same coefficients, though in opposite order) for the luminance samples and at most four new filters for the chrominance samples (the ¼-pel and ¾-pel filters employ the same coefficients; the 1/8-pel and 7/8-pel filters employ the same coefficients; and the 3/8-pel and 5/8-pel filters employ the same coefficients), for each video picture/slice, there is no additional decoder complexity. Moreover, except for the encoding of the AIF filters for each video picture/slice, the added complexity at the encoder side depends on the application at hand. For example, the new technique can be used to generate special filters for each slice, picture, group of pictures or even a set of video sequences that have similar characteristics (e.g., in a special application space).

As shown in Tables 6 & 7, even when we generate such filters on-line on a slice basis, the average additional encoding times needed for the generation, selection and application procedures are only 4% for LDP, 7% for LDP/LC, 3% for LD, 6% LD/LC, 2% for RA and 3% for RA/LC.

Table 6: Simulation Results (Luminance Only)

|  | Low Delay P |  |  | Low Delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.14 | -0.16 | -0.25 | -2.65 | 0.29 | 0.12 |
| Class C | -0.29 | -0.13 | -0.29 | -1.09 | -0.25 | -0.13 |
| Class D | 0.01 | 0.01 | 0.01 | -0.61 | -0.27 | -0.13 |
| Class E | -0.39 | 0.35 | -0.39 | -5.38 | -0.85 | -0.82 |
| All | -0.25 | -0.01 | -0.31 | -2.26 | -0.20 | -0.18 |
| Enc Time[%] | 104% (102%) |  |  | 107% (108%) |  |  |
| Dec Time[%] | 100% (100%) |  |  | 100% (100%) |  |  |

|  | Low Delay |  |  | Low Delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.12 | 0.25 | -0.11 | -1.34 | 0.53 | 0.11 |
| Class C | -0.13 | -0.15 | 0.03 | -0.61 | 0.04 | 0.04 |
| Class D | 0.01 | 0.01 | 0.01 | -0.30 | -0.14 | 0.20 |
| Class E | -0.16 | 0.32 | -0.41 | -2.78 | 0.04 | -0.51 |
| All | -0.10 | 0.10 | -0.10 | -1.17 | 0.15 | 0.00 |
| Enc Time[%] | 103%(103%) |  |  | 106% (108%) |  |  |
| Dec Time[%] | 100%(100%) |  |  | 100% (100%) |  |  |

|  | Random Access |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.33 | -0.35 | -0.14 | -1.31 | -0.25 | -0.42 |
| Class B | 0.01 | -0.10 | -0.07 | -0.19 | -0.09 | -0.12 |
| Class C | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Class D | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Class E |  |  |  |  |  |  |
| All | -0.07 | -0.11 | -0.05 | -0.36 | -0.08 | -0.13 |
| Enc Time[%] | 102% (102%) |  |  | 103% (103%) |  |  |
| Dec Time[%] | 100% (100%) |  |  | 100% (100%) |  |  |

Table 7: Simulation Results (Luminance + Chrominance)

|  | Low Delay P |  |  | Low Delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.15 | -0.29 | -0.46 | -2.66 | -2.19 | -2.35 |
| Class C | -0.25 | -0.01 | -0.23 | -1.00 | -0.47 | -0.50 |
| Class D | 0.01 | 0.01 | 0.01 | -0.54 | -0.35 | -0.47 |
| Class E | -0.24 | -1.60 | -2.20 | -4.84 | -13.49 | -8.85 |
| All | -0.15 | -0.39 | -0.61 | -2.13 | -3.42 | -2.63 |
| Enc Time[%] | 104% (102%) |  |  | 108% (109%) |  |  |
| Dec Time[%] | 100% (100%) |  |  | 100% (100%) |  |  |

|  | Low Delay |  |  | Low Delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.11 | -0.07 | -0.03 | -1.34 | -1.04 | -1.17 |
| Class C | -0.10 | -0.10 | -0.15 | -0.57 | -0.27 | -0.15 |
| Class D | 0.01 | 0.01 | 0.01 | -0.21 | -0.34 | -0.12 |
| Class E | -0.01 | -1.25 | -1.94 | -2.27 | -8.74 | -6.75 |
| All | -0.06 | -0.28 | -0.41 | -1.04 | -2.12 | -1.67 |
| Enc Time[%] | 104%(103%) |  |  | 107% (109%) |  |  |
| Dec Time[%] | 100%(100%) |  |  | 100% (100%) |  |  |

|  | Random Access |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.37 | -0.28 | -0.33 | -1.32 | -0.68 | -0.67 |
| Class B | 0.03 | 0.03 | 0.09 | -0.17 | 0.11 | 0.11 |
| Class C | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Class D | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Class E |  |  |  |  |  |  |
| All | -0.08 | -0.05 | -0.05 | -0.36 | -0.12 | -0.12 |
| Enc Time[%] | 102% (102%) |  |  | 103% (103%) |  |  |
| Dec Time[%] | 100% (100%) |  |  | 100% (100%) |  |  |

Table 8: Detailed Luminance (Y) BD-Rate Results

|  | Sequences | RA | RA LC | LD | LD LC | LD P | LD P LC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S01 | Traffic | 0.1 | -0.7 |  |  |  |  |
| S02 | PeopleOnStreet | 0.1 | -0.3 |  |  |  |  |
| S0X | Nebuta | -0.9 | -2.2 |  |  |  |  |
| S0Y | SteamLocomotive | -0.6 | -2.0 |  |  |  |  |
| S03 | Kimono | 0.0 | -0.2 | 0.0 | -2.1 | 0.0 | -3.5 |
| S04 | ParkScene | 0.3 | 0.2 | -0.1 | -0.7 | 0.0 | -1.2 |
| S05 | Cactus | 0.1 | -0.1 | -0.2 | -1.0 | 0.0 | -1.9 |
| S06 | BasketballDrive | -0.2 | -0.2 | -0.2 | -0.9 | -0.2 | -1.9 |
| S07 | BQTerrace | -0.2 | -0.6 | -0.2 | -2.1 | -0.5 | -4.7 |
| S08 | BasketballDrill | 0.0 | 0.0 | -0.3 | -0.8 | -0.3 | -0.9 |
| S09 | BQMall | 0.0 | 0.0 | -0.2 | -0.7 | -0.5 | -1.8 |
| S10 | PartyScene | 0.0 | 0.0 | 0.0 | -0.3 | -0.2 | -0.5 |
| S11 | RaceHorses | 0.0 | 0.0 | -0.1 | -0.6 | -0.1 | -1.1 |
| S12 | BasketballPass | 0.0 | 0.0 | 0.0 | -0.4 | 0.0 | -1.0 |
| S13 | BQSquare | 0.0 | 0.0 | 0.0 | -0.3 | 0.0 | -0.4 |
| S14 | BlowingBubbles | 0.0 | 0.0 | 0.0 | -0.4 | 0.0 | -0.7 |
| S15 | RaceHorses | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | -0.2 |
| S16 | Vidyo1 |  |  | 0.0 | -1.5 | -0.5 | -2.9 |
| S17 | Vidyo3 |  |  | -0.2 | -4.8 | -0.4 | -8.9 |
| S18 | Vidyo4 |  |  | -0.3 | -2.1 | -0.3 | -4.3 |
| Average | Class A | -0.3 | -1.3 |  |  |  |  |
|  | Class B | 0.0 | -0.2 | -0.1 | -1.3 | -0.1 | -2.7 |
|  | Class C | 0.0 | 0.0 | -0.1 | -0.6 | -0.3 | -1.1 |
|  | Class D | 0.0 | 0.0 | 0.0 | -0.3 | 0.0 | -0.6 |
|  | Class E |  |  | -0.2 | -2.8 | -0.4 | -5.4 |
|  | ALL | -0.1 | -0.4 | -0.1 | -1.2 | -0.2 | -2.3 |

Table 9: Detailed Luminance (U) BD-Rate Results

|  | Sequences | RA | RA LC | LD | LD LC | LD P | LD P LC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S01 | Traffic | 0.0 | -0.1 |  |  |  |  |
| S02 | PeopleOnStreet | 0.3 | 0.3 |  |  |  |  |
| S0X | Nebuta | 0.5 | -1.8 |  |  |  |  |
| S0Y | SteamLocomotive | -1.9 | -1.0 |  |  |  |  |
| S03 | Kimono | 0.1 | -0.1 | -0.1 | -4.6 | -0.2 | -6.2 |
| S04 | ParkScene | 0.4 | 0.8 | 0.1 | 0.0 | 0.2 | -0.1 |
| S05 | Cactus | 0.1 | -0.2 | -0.3 | -0.7 | -0.5 | -1.1 |
| S06 | BasketballDrive | -0.2 | 0.3 | -0.3 | 0.0 | -0.5 | -1.7 |
| S07 | BQTerrace | -0.3 | -0.3 | 0.2 | 0.1 | -0.4 | -1.9 |
| S08 | BasketballDrill | 0.0 | 0.0 | -0.5 | -0.4 | 0.4 | -0.5 |
| S09 | BQMall | 0.0 | 0.0 | 0.1 | -0.6 | -0.2 | -1.0 |
| S10 | PartyScene | 0.0 | 0.0 | -0.1 | 0.4 | -0.4 | -0.2 |
| S11 | RaceHorses | 0.0 | 0.0 | 0.1 | -0.5 | 0.2 | -0.2 |
| S12 | BasketballPass | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | -0.7 |
| S13 | BQSquare | 0.0 | 0.0 | 0.0 | -1.0 | 0.0 | 0.0 |
| S14 | BlowingBubbles | 0.0 | 0.0 | 0.0 | -0.3 | 0.0 | -0.6 |
| S15 | RaceHorses | 0.0 | 0.0 | 0.0 | 0.4 | 0.0 | -0.1 |
| S16 | Vidyo1 |  |  | -0.6 | -4.2 | -0.6 | -8.0 |
| S17 | Vidyo3 |  |  | -1.1 | -11.3 | -1.2 | -16.2 |
| S18 | Vidyo4 |  |  | -2.0 | -10.8 | -3.0 | -16.3 |
| Average | Class A | -0.4 | -0.7 |  |  |  |  |
|  | Class B | 0.0 | -0.2 | -0.1 | -1.0 | -0.3 | -2.7 |
|  | Class C | 0.0 | 0.0 | -0.1 | -0.3 | 0.0 | -1.0 |
|  | Class D | 0.0 | 0.0 | 0.0 | -0.3 | 0.0 | -0.5 |
|  | Class E |  |  | -1.2 | -8.7 | -1.6 | -4.8 |
|  | ALL | -0.1 | -0.1 | -0.3 | -2.1 | -0.4 | -2.1 |

Table 10: Detailed Chrominance (V) BD-Rate Results

|  | Sequences | RA | RA LC | LD | LD LC | LD P | LD P LC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S01 | Traffic | 0.0 | 0.1 |  |  |  |  |
| S02 | PeopleOnStreet | 0.1 | 0.0 |  |  |  |  |
| S0X | Nebuta | 0.0 | -1.9 |  |  |  |  |
| S0Y | SteamLocomotive | -1.4 | -0.9 |  |  |  |  |
| S03 | Kimono | 0.3 | 0.2 | -0.1 | -2.4 | 0.0 | -3.7 |
| S04 | ParkScene | 0.1 | 0.2 | 0.2 | -1.0 | -0.4 | -1.3 |
| S05 | Cactus | 0.2 | 0.1 | -0.3 | -0.4 | -0.2 | -0.7 |
| S06 | BasketballDrive | 0.0 | 0.3 | -0.3 | -0.1 | -0.3 | -1.4 |
| S07 | BQTerrace | -0.2 | -0.3 | 0.4 | -2.0 | -1.5 | -4.6 |
| S08 | BasketballDrill | 0.0 | 0.0 | 0.1 | -0.1 | -0.2 | -0.2 |
| S09 | BQMall | 0.0 | 0.0 | -0.1 | -0.1 | 0.0 | -0.4 |
| S10 | PartyScene | 0.0 | 0.0 | -0.1 | -0.2 | -0.7 | -0.5 |
| S11 | RaceHorses | 0.0 | 0.0 | -0.4 | -0.2 | 0.0 | -0.8 |
| S12 | BasketballPass | 0.0 | 0.0 | 0.0 | -0.3 | 0.0 | -0.8 |
| S13 | BQSquare | 0.0 | 0.0 | 0.0 | -0.3 | 0.0 | -0.6 |
| S14 | BlowingBubbles | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.6 |
| S15 | RaceHorses | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 |
| S16 | Vidyo1 |  |  | -1.3 | -4.4 | -1.3 | -7.1 |
| S17 | Vidyo3 |  |  | -1.8 | -4.4 | -1.2 | -5.9 |
| S18 | Vidyo4 |  |  | -2.7 | -11.0 | -4.0 | -13.6 |
| Average | Class A | -0.3 | -0.7 |  |  |  |  |
|  | Class B | 0.0 | -0.2 | -0.1 | -1.2 | -0.5 | -2.4 |
|  | Class C | 0.0 | 0.0 | -0.1 | -0.1 | -0.2 | -0.5 |
|  | Class D | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | -0.5 |
|  | Class E |  |  | -0.1 | -6.6 | -2.2 | -8.8 |
|  | ALL | 0.0 | -1.2 | -0.1 | -1.7 | -0.6 | -2.6 |

# 6. AIF/ALF Interaction: Towards Lower Decoder Complexity

Table 11 shows the BD-Rate gains for the LDP/LC, LD/LC and RA/LC configurations when ALF is enabled, as well as the BD-Rate gains for the LDP, LD and RA configurations when ALF is disabled.

Table 12 also shows the BD-Rate gains when both the proposed AIF technique and the ALF technique proposed in [JCTVC-F041 and JCTVC-F042] are enabled for the (1) luminance samples only and (2) for the luminance and chrominance samples. The proposed combined AIF-ALF technique achieves similar Y BD-Rate levels to, and substantially better U/V BD-Rate levels than, those of HM 3.0. Moreover, the average decoding computational complexity is reduced by 10%, while the average encoding complexity is increased by an average of 4%.

Table 11: Simulation Results (Luminance Only)

|  | Low Delay P (ALF OFF) |  |  | Low Delay P LC (ALF ON) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -3.28 | -0.85 | -1.18 | -0.12 | -0.11 | -0.21 |
| Class C | -1.42 | -0.71 | -0.38 | -0.16 | 0.04 | 0.01 |
| Class D | -0.84 | 0.08 | 0.18 | -0.19 | -0.10 | -0.08 |
| Class E | -6.73 | -1.47 | -1.42 | -0.22 | -0.66 | 0.19 |
| All | -2.85 | -0.70 | -0.69 | -017 | -0.21 | -0.02 |

|  | Low Delay (ALF OFF) |  |  | Low Delay LC (ALF ON) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.57 | 0.01 | -0.05 | -0.10 | 0.22 | -0.9 |
| Class C | -0.73 | -0.07 | 0.08 | -0.08 | -0.08 | -0.04 |
| Class D | -0.29 | 0.31 | 0.18 | 0.01 | 0.01 | 0.01 |
| Class E | -3.22 | -0.70 | -0.46 | -0.14 | 0.17 | 0.33 |
| All | -1.35 | -0.07 | -0.04 | -0.08 | 0.08 | -0.15 |

|  | Random Access (ALF OFF) |  |  | Random Access LC (ALF ON) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.51 | -0.15 | -0.62 | -0.25 | -0.15 | -0.09 |
| Class B | -0.17 | -0.04 | -0.09 | 0.04 | -0.05 | -0.05 |
| Class C | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Class D | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Class E |  |  |  |  |  |  |
| All | -0.42 | 0.05 | -0.18 | -0.05 | -0.05 | -0.03 |

Table 12: Simulation Results (Proposed AIF-ALF Technique)

|  | Low Delay P (Luminance Only) |  |  | Low Delay P (Luminance + Chrominance) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.18 | -0.33 | -0.61 | 0.38 | -1.44 | -1.52 |
| Class C | -0.35 | -0.27 | -0.59 | -0.19 | -1.63 | -1.68 |
| Class D | -0.38 | -0.19 | -0.14 | -0.18 | -1.06 | -0.84 |
| Class E | -0.30 | -0.18 | -0.71 | -0.02 | -1.93 | -3.17 |
| All | -0.18 | -0.26 | -0.52 | 0.00 | -1.51 | -1.80 |
| Enc Time[%] | 107% |  |  | 103% |  |  |
| Dec Time[%] | 95% |  |  | 94% |  |  |

|  | Low Delay (Luminance Only) |  |  | Low Delay (Luminance + Chrominance) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.27 | 0.00 | -0.09 | 0.30 | -0.75 | -0.61 |
| Class C | -0.05 | -0.10 | -0.22 | -0.05 | -1.52 | -1.45 |
| Class D | 0.03 | 0.17 | 0.12 | 0.05 | -0.44 | -0.76 |
| Class E | -0.01 | 0.10 | -0.17 | 0.40 | -1.23 | -2.50 |
| All | 0.08 | 0.03 | -0.09 | 0.18 | -0.99 | -1.33 |
| Enc Time[%] | 106% |  |  | 103% |  |  |
| Dec Time[%] | 95% |  |  | 94% |  |  |

|  | Random Access (Luminance Only) |  |  | Random Access (Luminance + Chrominance) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.15 | -0.75 | -0.35 | 0.17 | -0.26 | -0.10 |
| Class B | 0.24 | -0.08 | -0.12 | 0.25 | -0.15 | 0.04 |
| Class C | 0.04 | 0.10 | 0.03 | 0.03 | -0.81 | -0.42 |
| Class D | 0.10 | 0.02 | -0.07 | 0.08 | -0.68 | -0.58 |
| Class E |  |  |  |  |  |  |
| All | 0.06 | -0.18 | -0.17 | 0.12 | -0.71 | -0.67 |
| Enc Time[%] | 104% |  |  | 105% |  |  |
| Dec Time[%] | 97% |  |  | 95% |  |  |

# 7. Conclusion and recommendation

The proposed technique provides an average of 0.7% in Y BD-Rate reduction when applied to the luminance samples only, and an average of 0.7%/1.1%/0.9% in Y/U/V BD-Rate reduction when applied to the luminance and chrominance samples, relative to the HM 3.0 fixed filtering technique, while also requiring essentially the same decoding complexity. The additional encoder complexity can vary greatly, depending on the video application. In some cases, the new filters can be decided off-line, and the encoder will only need to encode and transmit at most two filters per slice for Luma and at most 4 filters per slice for Chroma. In other cases, the encoder will generate the coefficients as part of a pre-analysis phase. Even when we generate such filters on-line on a slice basis, the average additional encoding complexity needed for the generation, selection and application procedures is only ~4%.

# 8. References

- https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-3.0/

- S. Wittmann, T. Wedi, “Separable Adaptive Interpolation Filter,” ITU-T SG16/Q.6 Doc. T05-SG16-C-0219, Geneva, Switzerland, June 2007.

- F. Bossen, “Common TM Test Conditions,” JCTVC-D600, 4th. JCT-VC Meeting, Daegu, KR, 20-28 January, 2011.

# 9. Patent rights declaration(s)

eBrisk Video Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).