| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F217 |
| --- | --- |

| Title: | CE12: Cross-verification of Panasonic’s proposal JCTVC-F191 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Masaru Ikeda Teruhiko Suzuki Gate City Osaki West Tower East Tec, 1-11-1, Osaki, Shinagawa-ku , Tokyo, 141-0032 JAPAN | Tel: Email: | +81-3-5435-3308 Masaru.Ikeda@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports the cross verification results on JCTVC-F191 [1] proposed by Panasonic. In the test, the coding performance and complexity was measured under common test conditions, which is defined in JCTVC-E700 [2]. Proposed tools have been evaluated on the common test condition. The detailed results are summarized in attached the excel sheet. In addition the subjective quality is evaluated on test sequences, QP and coding conditions selected in CE12. The source code was provided by Panasonic to cross verify. It is confirmed that all results perfectly match with ones provided by Panasonic.

# Introduction

Panasonic’s proposal is included in CE12 which is techniques related to improvement for deblocking filter. The BD-rate, the measurement of encoding/decoding time, and subjective evaluation is shown as below.

# Results

## Test condition

The experiment is executed with using HM-3.0 based on JCTVC-E712 [3], and the configuration and common test condition are as JCTVC-E700.

Both the encoding and the decoding time are measured on the same machine as anchor, and the decoding time is measured without output of decoded YUV data.

## BD-rate and encoding/decoding time

The test for Panasonic’s proposal has two parts. First one corresponds to “Individual Decision (Config1)” and second one corresponds to “Indivisual Decision Second Derivation at Border (Config2)”. The experimental results are summarized in the following tables respectively. The detailed results are shown in the attached excel-file respectively.

### Individual Decision (Config1)

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class B | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class C | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class D | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class E | -0.2 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| All | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Table2-1: Results for Intra

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | -0.2 | -0.2 | -0.2 | 0.0 | 0.2 |
| Class B | -0.2 | -0.1 | -0.1 | -0.2 | 0.0 | 0.0 |
| Class C | -0.2 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
| Class D | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -0.2 | -0.1 | -0.1 | -0.2 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table2-2: Results for RandomAccess

|  | Low delay |  |  | Low delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | 0.1 | 0.1 | -0.3 | -0.1 | -0.4 |
| Class C | -0.3 | -0.2 | -0.2 | -0.3 | 0.0 | -0.4 |
| Class D | -0.2 | 0.1 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class E | -0.5 | -0.2 | -0.1 | -0.5 | -0.3 | 0.0 |
| All | -0.3 | 0.0 | 0.0 | -0.3 | -0.1 | -0.2 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Table2-3: Results for LowDelay

|  | Low delay (P) |  |  | Low delay LC (P) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | 0.1 | -0.2 | -0.2 | 0.0 | 0.2 |
| Class C | -0.2 | 0.1 | -0.1 | -0.2 | 0.0 | 0.0 |
| Class D | -0.2 | -0.1 | -0.5 | -0.1 | -0.3 | 0.0 |
| Class E | -0.6 | 0.0 | -0.2 | -0.2 | -0.4 | -0.6 |
| All | -0.3 | 0.0 | -0.2 | -0.2 | -0.2 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table2-4: Results for LowDelay(P)

### Individual Decision Second Derivation at Border (Config2)

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.5 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class B | -0.4 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
| Class C | -0.4 | 0.0 | 0.0 | -0.3 | 0.0 | 0.0 |
| Class D | -0.3 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
| Class E | -0.5 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| All | -0.4 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Table2-5: Results for Intra

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.8 | -0.3 | -0.5 | -0.2 | 0.0 | 0.1 |
| Class B | -0.6 | -0.1 | -0.1 | -0.4 | 0.0 | 0.0 |
| Class C | -0.4 | 0.1 | 0.0 | -0.3 | -0.1 | 0.0 |
| Class D | -0.3 | 0.0 | -0.1 | -0.2 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.1 | -0.2 | -0.3 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table2-6: Results for RandomAccess

|  | Low delay |  |  | Low delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.1 | 0.0 | -0.2 | -0.6 | -0.2 | -0.4 |
| Class C | -0.7 | 0.1 | 0.0 | -0.4 | 0.1 | -0.1 |
| Class D | -0.5 | -0.3 | -0.1 | -0.4 | 0.2 | -0.1 |
| Class E | -2.1 | -0.2 | -0.8 | -1.0 | 0.0 | -0.9 |
| All | -1.0 | -0.1 | -0.2 | -0.6 | 0.0 | -0.4 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Table2-7: Results for LowDelay

|  | Low delay (P) |  |  | Low delay LC (P) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.9 | 0.0 | -0.6 | -0.2 | 0.0 | 0.0 |
| Class C | -0.6 | 0.1 | 0.0 | -0.2 | 0.1 | 0.0 |
| Class D | -0.3 | -0.2 | 0.0 | -0.2 | 0.3 | -0.1 |
| Class E | -2.0 | -0.4 | -0.4 | -0.3 | 0.1 | -0.2 |
| All | -0.9 | -0.1 | -0.2 | -0.2 | 0.1 | -0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table2-8: Results for LowDelay(P)

## Subjective quality

Subjective quality was evaluated with using below conditions and equipments.

Conditions

- Kimono1 randomaccess QP=37

- Kimono1 lowdelay_loco QP=32

- BQMall randomaccess QP=37

- BQMall lowdelay QP=32

- Vidyo3 lowdelay QP=37

- Vidyo3 lowdelay_loco QP=32

Equipments

- Monitor: EIZO FlexScan SX3031W

- Processor : Intel Core i5 750 BOX

- RAM : DCDDR3-4GB-1333

- Graphic Card : ENGTX285 2DI 1GD3

Though the subjective quality is similar to anchor on average, the trailing edges and so on, which seem not to be caused directly, are found in two sequences regarding “Individual Decision Second Derivation at Border”. sThese aren’t found in the same sequences regarding “PANASONIC_INDIVIDUAL_DECISIONS”.

# Conclusion

In this contribution, the cross verified results of deblocking filter by Panasonic are reported. It is confirmed that the results perfectly match with the ones provided by Panasonic.

# References

[1] JCT-VC, “Results for decisions for deblocking”, JCTVC-F191,6th. JCT-VC Meeting, Torino, IT, July 2011

[2]JCT-VC, “Common test conditions and software reference configurations”, JCTVC-E700,5th. JCT-VC Meeting, Geneva, CH, March 2011

[3]JCT-VC, “Description of CE12: deblocking filtering”, JCTVC-E712,5th. JCT-VC Meeting, Geneva, CH, March 2011

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).