| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F220 |
| --- | --- |

| Title: | Cross-verification of TI’s proposal JCTVC-F256 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Masaru Ikeda Teruhiko Suzuki Gate City Osaki West Tower East Tec, 1-11-1, Osaki, Shinagawa-ku , Tokyo, 141-0032 JAPAN | Tel: Email: | +81-3-5435-3308 Masaru.Ikeda@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports the cross verification results on JCTVC-F256 [1] proposed by TI. In the test, the coding performance and complexity was measured under common test conditions, which is defined in JCTVC-E700 [2]. Proposed tools have been evaluated on the common test condition. The detailed results are summarized in attached the excel sheet. The source code was provided by TI to cross-verify. It is confirmed that all results perfectly match with ones provided by TI.

# Introduction

TI’s proposal is new proposal which is techniques related to improvement for deblocking filter. The BD-rate, the measurement of encoding/decoding time, and subjective evaluation is shown as below.

# Results

## Test condition

The experiment is executed with using HM-3.0 based on JCTVC-E712 [3], and the configuration and common test condition are as JCTVC-E700.

Both the encoding and the decoding time are measured on the same machine as anchor, and the decoding time is measured without output of decoded YUV data.

## BD-rate and encoding/decoding time

The experimental results are summarized in the following tables. The detailed results are shown in the attached excel-file.

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.4 | -0.2 | 0.0 | -0.6 | -0.4 |
| Class B | 0.0 | -0.2 | -0.1 | 0.0 | -0.7 | -0.5 |
| Class C | 0.0 | -0.2 | -0.2 | 0.0 | -0.7 | -0.6 |
| Class D | 0.0 | -0.2 | -0.1 | 0.0 | -0.6 | -0.5 |
| Class E | 0.0 | 0.1 | 0.1 | 0.0 | 0.1 | -0.1 |
| All | 0.0 | -0.2 | -0.1 | 0.0 | -0.5 | -0.5 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

Table2-1: Results for Intra

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.6 | -0.6 | 0.0 | -0.7 | -0.4 |
| Class B | 0.0 | -0.6 | -0.5 | 0.0 | -0.7 | -0.6 |
| Class C | 0.0 | -0.4 | -0.5 | 0.0 | -0.6 | -0.7 |
| Class D | 0.0 | -0.5 | -0.5 | 0.0 | -0.7 | -0.5 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | -0.6 | -0.5 | 0.0 | -0.7 | -0.6 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

Table2-2: Results for RandomAccess

|  | Low delay |  |  | Low delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | -0.8 | -0.8 | 0.0 | -1.0 | -1.1 |
| Class C | -0.1 | -0.8 | -0.8 | -0.1 | -0.6 | -0.8 |
| Class D | 0.0 | -0.3 | 0.0 | 0.0 | -0.4 | -0.5 |
| Class E | 0.0 | -1.2 | -1.3 | -0.1 | -2.4 | -1.9 |
| All | 0.0 | -0.7 | -0.7 | 0.0 | -1.0 | -1.0 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table2-3: Results for LowDelay

# Conclusion

In this contribution, the cross verified results of deblocking filter by TI are reported. It is confirmed that the results perfectly match with the ones provided by TI.

# References

[1] JCT-VC, “Efficient Chroma Deblock filter”, JCTVC-F256,6th. JCT-VC Meeting, Torino, IT, July 2011

[2]JCT-VC, “Common test conditions and software reference configurations”, JCTVC-E700,5th. JCT-VC Meeting, Geneva, CH, March 2011

[3]JCT-VC, “Description of CE12: deblocking filtering”, JCTVC-E712,5th. JCT-VC Meeting, Geneva, CH, March 2011

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).