| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, Italy, 14-22 July, 2011 | Document: JCTVC-F221 |
| --- | --- |

| Title: | CE4 Subtest1: the signaling of minCUDQPsize at LCU level |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Oscar C. Au, Xing Wen, Jingjing Dai, Feng Zou and Xingyu Zhang HKUST, Clear Water Bay, KLN, Hong Kong | Email: | pcece@ust.hk eeau@ust.hk wxxab@ust.hk |
| Source: | The Hong Kong University of Science and Technology |  |  |

_____________________________

# Abstract

This contribution presents the results of CE4 Subtest1 in JCTVC-E704 [1] which aims to investigate the potential of the signaling of minCUDQPsize at LCU level. From the experimental results, we can see that this signaling scheme performs worse than signaling at PPS level as defined in WD/HM3. The decoding time remains same for all experiments.

# Signaling minCUDQPsize at LCU level

In current WD/HM3, the minCUDQPsize is signaled at PPS level as described in JCTVC-E436 (method 1) [2] which is adopted in last Geneva meeting. We further proposed that the minCUDQPsize can also be sent at LCU level in JCTVC-E436 [2]. Therefore, in this proposal, we investigate its potential from various experiments. In detail, the value of LCU-level minCUDQPsize is equal to or smaller than the value minCUDQPsize indicated at PPS level, and the difference between the LCU-level minCUDQPsize and PPS-level minCUDQPsize is transmitted in the bitstream. The syntax changes are as follows:

| slice_data( ) { | Descriptor |
| --- | --- |
| CurrTbAddr = first_tb_in_slice |  |
| moreDataFlag = 1 |  |
| do { |  |
| xCU = HorLumaLocation( CurrTbAddr ) |  |
| yCU = VerLumaLocation( CurrTbAddr ) |  |
| minCUDQPsize_Flag = 1 |  |
| coding_tree( xCU, yCU, Log2TbSize ){ |  |
| … |  |
| coding_unit( x0, y0, log2CUSize ) { |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if(!skip_flag[ x0 ][ y0 ] && lcu_qp_depthFlag){ |  |
| minCUDQPsize | ue(v) |

| minCUDQPsize_Flag = 0 |  |
| --- | --- |
| } |  |
| } |  |
| … |  |
| } |  |
| … |  |
| } while( moreDataFlag ) |  |
| } |  |

# Experimental results

The comparison is conducted using the recommended common test condition presented in JCTVC-E700 [3]. The test results for all sequences from class A to E (the value of PPS-level minCUDQPsize is 3) are shown in Table 1. In the experiments, the signaling of minCUDQPsize at PPS level is selected as the anchor. From the table1, we can see that the signaling of minCUDQPsize at PPS level always performs better than the signaling at LCU level.

Table 1 Experimental results

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 0.1 | 0.1 | 0.2 | 6.5% | 0.2 | 0.2 | 0.2 | 9.1% |
| Class B | 0.1 | 0.1 | 0.1 | 4.1% | 0.2 | 0.1 | 0.1 | 4.9% |
| Class C | 0.0 | 0.0 | 0.0 | 0.9% | 0.1 | 0.1 | 0.1 | 1.4% |
| Class D | 0.0 | 0.0 | 0.0 | 0.8% | 0.0 | 0.0 | 0.0 | 1.0% |
| Class E | 0.2 | 0.3 | 0.2 | 4.4% | 0.2 | 0.2 | 0.2 | 5.0% |
| All | 0.1 | 0.1 | 0.1 | 3.3% | 0.1 | 0.1 | 0.1 | 4.3% |
| Enc Time[%] |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 0.4 | 0.3 | 0.4 | 14.7% | 0.6 | 0.6 | 0.6 | 18.7% |
| Class B | 0.4 | 0.2 | 0.4 | 9.7% | 0.4 | 0.4 | 0.4 | 11.5% |
| Class C | 0.3 | 0.3 | 0.3 | 6.4% | 0.3 | 0.3 | 0.3 | 6.9% |
| Class D | 0.2 | 0.1 | 0.2 | 4.4% | 0.2 | 0.2 | 0.2 | 4.4% |
| Class E |  |  |  |  |  |  |  |  |
| All | 0.3 | 0.2 | 0.3 | 8.8% | 0.4 | 0.4 | 0.4 | 10.5% |
| Enc Time[%] | 100% |  |  |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 0.5 | 0.4 | 0.3 | 13.2% | 0.7 | 0.6 | 0.6 | 15.1% |
| Class C | 0.4 | 0.4 | 0.4 | 8.2% | 0.4 | 0.4 | 0.4 | 8.9% |
| Class D | 0.2 | 0.4 | 0.9 | 5.6% | 0.2 | 0.2 | 0.2 | 5.6% |
| Class E | 0.9 | 0.7 | 2.2 | 17.9% | 1.1 | 1.1 | 1.1 | 19.6% |
| All | 0.5 | 0.5 | 0.8 | 10.9% | 0.6 | 0.6 | 0.6 | 12.0% |
| Enc Time[%] | 100% |  |  |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  |  |  |

# References

[1] K. Sato, M. Budagavi, M. Coban, H. Aoki and X. Li , “Description of Core Experiment 4: Quantization”, JCTVC-E704, March. 2011

[2] C. Pang, O. C. Au, J. Dai, F. Zou, X. Wen, L. Fang and X. Zhang, “Sub-LCU QP representation”, JCTVC-E436, March. 2011

[3] F. Bossen, “Common Test Conditions and Software Reference Configurations”, JCTVC-E700, March. 2011

# Patent rights declaration(s)

The Hong Kong University of Science and Technology may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).