| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F222 |
| --- | --- |

| Title: | CE3: Verification results of Sony's Proposal Tools 4.2 and 4.5 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Faouzi Kossentini Nader Mahdi | Email: | faouzi@ebriskvideo.com nader@ebriskvideo.com |
| Source: | eBrisk Video Inc. |  |  |

_____________________________

# Abstract

This contribution provides a summary of the cross-checks performed by eBrisk Video Inc on Test Tool 4.2 & Tool 4.5 of CE3.  This contribution documents the findings based on the study and simulation conducted using the software provided by Sony.

# Verification

The target of this core experiment is to verify the performance of Sony MC Boundary filter [1]. The simulations were conducted using HM3.0 software [2] with Sony’s modifications.

Six configurations (RA HE, LD HE, LD-P HE, RA LC, LD LC, and LD-P LC) were run for the test with the provided code.

The encoding/decoding times were estimated using a PC with the following configuration:

- CPU: Intel Core 2 Quad Q8300 @2.50 GHz.

- RAM: 4 Go

- OS:  Windows XP Professional.

# Observations

The obtained BD-Rate results, for both experiments, perfectly match the results provided by Sony and run times are quiet close.

# Simulation Results

## Tool 4.5 results

|  | Random access |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0,08 | -0,10 | 0,04 | 0,05 | 0,14 | 0,24 |
| Class B | 0,06 | -0,03 | 0,01 | 0,03 | 0,08 | 0,08 |
| Class C | 0,15 | 0,12 | 0,02 | 0,16 | 0,06 | 0,07 |
| Class D | 0,21 | -0,01 | -0,02 | 0,32 | 0,03 | 0,09 |
| Class E |  |  |  |  |  |  |
| All | 0,12 | -0,01 | 0,01 | 0,13 | 0,08 | 0,12 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

|  | Low delay |  |  | Low delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0,02 | 0,26 | -0,04 | -0,03 | 0,10 | -0,13 |
| Class C | 0,12 | 0,23 | 0,02 | 0,07 | 0,04 | -0,14 |
| Class D | 0,03 | -0,14 | 0,17 | 0,10 | 0,05 | -0,03 |
| Class E | 0,13 | 0,19 | -0,59 | -0,03 | 0,05 | -0,40 |
| All | 0,06 | 0,14 | -0,08 | 0,03 | 0,06 | -0,15 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

|  | Low delay (P) |  |  | Low delay LC (P) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0,06 | 0,11 | -0,09 | -0,04 | 0,16 | 0,32 |
| Class C | 0,17 | 0,20 | 0,06 | 0,13 | 0,07 | 0,18 |
| Class D | 0,16 | -0,05 | 0,22 | 0,18 | 0,45 | 0,13 |
| Class E | 0,08 | -0,02 | -0,28 | -0,02 | -0,18 | -0,27 |
| All | 0,12 | 0,07 | -0,01 | 0,06 | 0,15 | 0,13 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

## Tool 4.2 results

|  | Random access |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0,01 | 0,03 | 0,12 | -0,04 | -0,03 | 0,12 |
| Class B | 0,04 | -0,03 | 0,05 | 0,03 | 0,02 | 0,04 |
| Class C | 0,10 | 0,08 | 0,03 | 0,14 | 0,03 | 0,03 |
| Class D | 0,21 | 0,12 | 0,07 | 0,24 | 0,02 | -0,04 |
| Class E |  |  |  |  |  |  |
| All | 0,09 | 0,04 | 0,07 | 0,09 | 0,01 | 0,04 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

|  | Low delay |  |  | Low delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0,02 | 0,18 | 0,02 | -0,07 | 0,01 | -0,13 |
| Class C | 0,07 | 0,04 | -0,11 | 0,04 | 0,23 | -0,09 |
| Class D | -0,01 | 0,14 | 0,14 | 0,06 | 0,36 | -0,12 |
| Class E | 0,23 | -0,04 | 0,01 | -0,12 | -0,19 | -0,56 |
| All | 0,05 | 0,09 | 0,02 | -0,02 | 0,11 | -0,20 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

|  | Low delay (P) |  |  | Low delay LC (P) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0,03 | -0,01 | -0,33 | -0,06 | 0,11 | 0,23 |
| Class C | 0,15 | 0,13 | 0,04 | 0,12 | 0,01 | 0,01 |
| Class D | 0,13 | -0,53 | -0,08 | 0,17 | -0,29 | 0,04 |
| Class E | 0,05 | -0,20 | 0,02 | -0,04 | 0,05 | -0,24 |
| All | 0,09 | -0,14 | -0,11 | 0,05 | -0,03 | 0,04 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

The results presented by Sony are confirmed.

# References

- K. Kondo and T. Suzuki, “CE3: Reports on MC boundary filter” , JCTVC-F100, July 2011.

[2]     https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-3.0

[3]   JCT-VC, “Common test conditions and software reference configurations”, JCTVC-E700, March 2011

# Patent rights declaration(s)

eBrisk Video Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).