Change of Woking Darft for proposal F224

“CE7: Mode Dependent 2-step Transform for Intra Coding”

The document is based on JCTVC-E603_d8.doc and change is colored yellow.

## Scaling, transformation and array construction process prior to deblocking filter process

### Scaling and transformation process

### Inverse scanning process for transform coefficientsI

### Scaling process for transform coefficients

### Transformation process for scaled transform coefficients

Inputs of this process are:

–	a variable nS specifying the size of the current transform unit,

–	a (nS)x(nS) array d of scaled transform coefficients with elements dij.

–	a variable cIdx specifying the chroma component of the current block,

Output of this process is residual samples as a (nS)x(nS) array r with elements rij.

The variable trafoPrecisionExt is derived as follows:

–	If nS is greater than 4,

- If cIdx is equal to 0 and bit_depth_luma_minus8 is equal to 0, trafoPrecisionExt is set equal to 4.

- Otherwise, if cIdx is not equal to 0 and bit_depth_chroma_minus8 is equal to 0, trafoPrecisionExt is set equal to 4.

- Otherwise, trafoPrecisionExt is set equal to 0.

–	Otherwise, trafoPrecisionExt is set equal to 0.

[Ed.: (WJ) wait decision on transform precision extension]

[Ed: (WJ) derivation of trafoPrecisionExt is duplicated to that of the scaling process. Better to remove one of two.]

Depending on PredMode and IntraPredMode, the following applies:

–	If PredMode is equal to MODE_INTRA, nS is equal to 4 or 8 or 16 or 32, and cIdx is equal to 0, the variables horizTrType and vertTrType are specified as Table 8-1 with IntraPredMode as input. [Ed. (WJ): DST is applied only for luma 4x4 block]

–	Otherwise, the variables horizTrType and vertTrType are set equal to 0.

Table 8-1 – Specification of horizTrType and vertTrType

| IntraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| horizTrType | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 |
| vertTrType | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 |

| IntraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| horizTrType | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| vertTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |

The constructed residual samples are derived as specified in the following ordered steps.

- Each (horizontal) row of scaled transform coefficients dij (i, j=0..nS-1) is transformed to eij (i, j=0..nS-1) by invoking the one-dimensional transformation process as specified in subclause 8.1.4.1 to 8.1.4.4 according to the size of the transform unit nS, with the (nS)x(nS) array d and the transform type variable horizTrType as the inputs and the output is the (nS)x(nS) array e.

- Each (vertical) column of the resulting matrix eij (i, j=0..nS-1) is transformed to fij (i, j=0..nS-1) by invoking the one-dimensional transformation process as specified in subclause 8.1.4.1 to 8.1.4.4 according to the size of the transform unit nS, with the (nS)x(nS) array e and the transform type variable vertTrType as the inputs and the output is the (nS)x(nS) array f.

- If nS is equal to 4 or 8, the residual sample value rij is derived by

rij = ( fij + 25+trafoPrecisionExt ) >> ( 6 + trafoPrecisionExt ), with i, j=0.. (nS)x(nS)-1	(8-1)

- Otherwise (nS is equal to 16 or 32), the residual sample value rij is derived by

rij = ( fij + 29+trafoPrecisionExt ) >> ( 10 + trafoPrecisionExt ), with i, j=0.. (nS)x(nS)-1	(8-2)

[Ed.: (WJ) dependent on transformation process – should be modified again after transformation process is fixed]

#### Transformation process for 4 samples

Inputs of this process are an array of 4 samples of the scaled transform coefficients x with elements xi, with i = 0..3 and the transform type variable trType.

Output of this process is an array of 4 samples of the residual samples y with elements yi, with i = 0..3.

Depending on trType, the following applies:

–	If tyType is equal to 1, the following ordered steps apply:

- A set of intermediate values c0, c1, c2 and c3 is calculated as follows:

c0 = x0 + x2		(8-3)
c1 = x2 + x3		(8-4)
c2 = x0 – x3		(8-5)
c3 = 74 * x1		(8-6)

- The output values yi with i = 0..3 are then specified as follows:

y0 = ( 29*c0 + 55*c1 + c3 + 64 ) >> 7		(8-7)
y1 = ( 55*c2 – 29*c1 + c3 + 64 ) >> 7		(8-8)
y2 = ( 74*( x0 - x2 + x3 ) + 64 ) >> 7		(8-9)
y3 = ( 55*c0 + 29 * c2 – c3 + 64 ) >> 7		(8-10)

[Ed. (WJ): proponent’s comment - to share the quantization scaling value, transform precision (7-bit in this case) should be synchronized with DCT]

–	Otherwise, the following ordered steps apply: [Ed. (WJ): TBD – 4-point DCT]

#### Transformation process for 8 samples

Inputs of this process are an array of 8 samples of the scaled transform coefficients x with elements xi, with i = 0..7 and the transform type variable trType.

Output of this process is an array of 8 samples of the residual samples y with elements yi, with i = 0..7.

[Ed: (WJ) TBD]

Depending on trType, the following applies:

–	If tyType is equal to 1, the following ordered steps apply:

- A set of intermediate values t0, t1, t2 and t3 store input as follows:

t0 = x0			(8-11)
t1 = x1			(8-12)
t2 = x2			(8-13)
t3 = x3			(8-14)

- The input values xi with i = 0..3 are then modified as follows:

x0 = ( 118*t0 + 39*t1 + 23*t2 + 18*t3 + 64 ) >> 7		(8-15)
x1 = ( – 47*t0 + 109*t1 + 39*t2 + 18 *t3 + 64 ) >> 7	(8-16)
x2 = ( – 10*t0 – 53*t1 + 109*t2 + 39*t3 + 64 ) >> 7	(8-17)
x3 = ( – 4*t0 – 15*t1 – 50*t2 + 117*t3 + 64 ) >> 7		(8-18)

iDCT8x8 is then invoked.

#### Transformation process for 16 samples

Inputs of this process are an array of 16 samples of the scaled transform coefficients x with elements xi, with i = 0..15 and the transform type variable trType.

Output of this process is an array of 16 samples of the residual samples y with elements yi, with i = 0..15.

[Ed: (WJ) TBD]

Depending on trType, the following applies:

–	If tyType is equal to 1, the following ordered steps apply:

- A set of intermediate values t0, t1, t2 and t3 store input as follows:

t0 = x0			(8-19)
t1 = x1			(8-20)
t2 = x2			(8-21)
t3 = x3			(8-22)

- The input values xi with i = 0..3 are then modified as follows:

x0 = ( 118*t0 + 39*t1 + 23*t2 + 18*t3 + 64 ) >> 7		(8-23)
x1 = ( – 47*t0 + 109*t1 + 39*t2 + 18 *t3 + 64 ) >> 7	(8-24)
x2 = ( – 10*t0 – 53*t1 + 109*t2 + 39*t3 + 64 ) >> 7	(8-25)
x3 = ( – 4*t0 – 15*t1 – 50*t2 + 117*t3 + 64 ) >> 7		(8-26)

iDCT16x16 is then invoked.

#### Transformation process for 32 samples

Inputs of this process are an array of 32 samples of the scaled transform coefficients x with elements xi, with i=0..31 and the transform type variable trType.

Output of this process is an array of 32 samples of the residual samples y with elements yi, with i=0..31.

[Ed: (WJ) TBD]

Depending on trType, the following applies:

–	If tyType is equal to 1, the following ordered steps apply:

- A set of intermediate values t0, t1, t2 and t3 store input as follows:

t0 = x0			(8-27)
t1 = x1			(8-28)
t2 = x2			(8-29)
t3 = x3			(8-30)

- The input values xi with i = 0..3 are then modified as follows:

x0 = ( 118*t0 + 39*t1 + 23*t2 + 18*t3 + 64 ) >> 7		(8-31)
x1 = ( – 47*t0 + 109*t1 + 39*t2 + 18 *t3 + 64 ) >> 7	(8-32)
x2 = ( – 10*t0 – 53*t1 + 109*t2 + 39*t3 + 64 ) >> 7	(8-33)
x3 = ( – 4*t0 – 15*t1 – 50*t2 + 117*t3 + 64 ) >> 7		(8-34)

iDCT32x32 is then invoked.