| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F225 WG11 Number: m20644 |
| --- | --- |

| Title: | Consideration of reference pixel availability for mode-dependent DCT/DST decision |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Youji Shibahara Takahiro Nishi | Tel: Email: | +81-50-3587-2044 shibahara.youji@jp.panasonic.com nishi.takahiro@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

In the Working Draft 3 (WD3) of HEVC, 4x4 DST is used for intra prediction unit depending on its intra prediction mode. The DST shows better coding gain than DCT when it is used together with the coupled intra prediction modes. In the latest design of HEVC, however, availability of reference pixels is not considered for the choice of DST, although it may affect the coding gain of the transform. In this contribution, it is proposed to check the availability of the reference pixels before deciding to use DST An experimental result reportedly shows that the proposed method provides 0.01%, 0.01 %, 0.02% and 0.00% gain for AI HE, AI LC, RA HE and RA LC, respectively on the common condition. As for the condition using slice (1500 byte / slice), an experimental result reportedly shows that the proposed method provides 0.01%, 0.10 %, 0.10%, 0.03% and 0.05% gain for AI HE, AI LC, RA HE and RA LC, respectively. And as for the condition of CIP enabled (constrained intra prediction), an experimental result shows that the proposed method provides 0.07% and 0.05% gain for RA HE and RA LC, respectively. The increase of encoding and decoding time is less than 1 % for all results.

- Introduction

The DST 4x4 in HM3.0 is optimal for intra prediction error signal. However when the neighboring pixel is not available or not reliable due to either duplication or interpolation of pixel located far away (reference sample padding), the error signal does not follow the ideal characteristic considered in the theory of DST.

# Solutions and experimental results

## Solution

The DST model assumes that the referenced pixel locates nearest neighbor of the predicted pixel. It can be found in the contribution of DST [1][2]. This document shows one basic example to explain it. The figure 1 shows the covariance matrix which provides a transform matrix of the DST. The diagonal elements indicate the power of input to the DST. We would see that the numerical value is {1, 2, 3, 4} and it is in proportion to the distance to the referenced neighboring pixel. Probably we may agree that it is only true if the referenced pixel locates nearest neighbor as shown in figure 2.

1111122212331234

Figure 1. Covariance Matrix derives the DST

Figure 2. The power of prediction error which is assumed on the DST model

However it is not always correct that reference sample locates nearest. For example, at the picture boundary, at the slice boundary and at the block under the restriction of CIP (“constrained intra prediction”). The reference sample padding generates the substitute pixel for such non available pixel. But it is either duplicated or interpolated from the pixel locate far away. The prediction error signal will be large than the case pixel is available at nearest neighbor. In this contribution, we propose to turn off DST in that case. And in order to avoid a burden to support additional transform, we propose to use existing DCT instead of the DST. The actual condition of the proposed method is very simple and low complex, it is that “if the Above (Left) pixel is not available or either duplicated or interpolated from pixel locates far away, the vertical (horizontal) transform is set to DCT. Otherwise, the transform may be switched depending on the intra prediction mode”.

## Simulation Result

The proposed method was evaluated on HM3.0 software. We had following 3 conditions.

- Test 1: common condition

- Test 2: enabled slice (1500 byte/slice)

- Test 3: enabled cip (“constrained intra prediction”)

Table 1 shows a result on the common condition. Table 2 shows a result on the condition using slice coding. The both anchor and proposal are evaluated in the condition of 1500 byte/per slice (SliceMode=2, SliceArgument=1500). Table 3 shows a result on the condition using CIP. Since CIP is a tool used in Inter Picture, the result of All Intra was not measured (it is identical to a result of AI in test 1). We would like to thank NHK and Samsung for conducting the cross-check for this in JCTVC-F485, JCTVC-F656 respectively.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00 | -0.05 | 0.03 | 0.00 | -0.03 | 0.00 |
| Class B | -0.01 | 0.03 | 0.02 | 0.00 | -0.01 | -0.01 |
| Class C | -0.02 | 0.00 | 0.02 | -0.01 | 0.00 | -0.03 |
| Class D | -0.02 | -0.03 | -0.04 | -0.02 | -0.02 | -0.01 |
| Class E | 0.02 | 0.04 | 0.01 | -0.02 | -0.03 | -0.02 |
| Overall | -0.01 | 0.00 | 0.01 | -0.01 | -0.02 | -0.01 |
| Enc Time[%] | 100.5% |  |  | 100.6% |  |  |
| Dec Time[%] | 100.4% |  |  | 100.5% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.03 | -0.35 | -0.07 | 0.01 | -0.07 | -0.03 |
| Class B | -0.03 | 0.02 | 0.05 | 0.01 | -0.04 | 0.01 |
| Class C | -0.03 | 0.22 | -0.17 | -0.01 | -0.08 | 0.00 |
| Class D | 0.00 | 0.07 | -0.24 | 0.01 | -0.41 | -0.14 |
| Class E |  |  |  |  |  |  |
| Overall | -0.02 | -0.01 | -0.10 | 0.00 | -0.14 | -0.04 |
| Enc Time[%] | 100.3% |  |  | 99.9% |  |  |
| Dec Time[%] | 99.8% |  |  | 100.0% |  |  |

Table 1. Result of the proposal on common condition

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.06 | -0.04 | -0.03 | -0.04 | -0.03 | -0.02 |
| Class B | -0.08 | -0.07 | -0.04 | -0.08 | -0.06 | -0.04 |
| Class C | -0.12 | -0.15 | -0.17 | -0.13 | -0.12 | -0.12 |
| Class D | -0.08 | -0.14 | -0.14 | -0.08 | -0.08 | -0.08 |
| Class E | -0.18 | -0.07 | -0.11 | -0.19 | -0.04 | -0.04 |
| Overall | -0.10 | -0.09 | -0.09 | -0.10 | -0.07 | -0.06 |
| Enc Time[%] | 100.2% |  |  | 101.0% |  |  |
| Dec Time[%] | 100.3% |  |  | 100.5% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.07 | 0.15 | 0.01 | -0.08 | -0.08 | -0.02 |
| Class B | -0.03 | -0.12 | -0.09 | -0.05 | 0.09 | -0.04 |
| Class C | -0.01 | -0.02 | 0.03 | -0.04 | -0.04 | -0.13 |
| Class D | -0.02 | 0.29 | -0.22 | -0.05 | 0.07 | -0.09 |
| Class E |  |  |  |  |  |  |
| Overall | -0.03 | 0.06 | -0.07 | -0.05 | 0.02 | -0.07 |
| Enc Time[%] | 100.6% |  |  | 100.8% |  |  |
| Dec Time[%] | 100.0% |  |  | 100.2% |  |  |

Table 2. Result of the proposal on the condition with Slice / 1500 byte

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.10 | -0.04 | -0.09 | -0.03 | -0.06 | -0.15 |
| Class B | -0.04 | 0.01 | -0.04 | -0.03 | -0.05 | -0.04 |
| Class C | -0.10 | 0.02 | -0.17 | -0.08 | -0.20 | -0.13 |
| Class D | -0.04 | -0.06 | -0.06 | -0.05 | -0.33 | -0.12 |
| Class E |  |  |  |  |  |  |
| Overall | -0.07 | -0.02 | -0.09 | -0.05 | -0.15 | -0.11 |
| Enc Time[%] | 100.2% |  |  | 100.1% |  |  |
| Dec Time[%] | 100.1% |  |  | 100.1% |  |  |

Table 3. Result of the proposal on the condition with CIP

# Conclusion

As explained above, the availability of reference pixels is not considered in HM3.0, but it should be done theoretically. The proposed change is to add a switch based on availability of reference pixels for DST / DCT selection. Increase of encoding and decoding time is less than 1 % for all results. Small but consistent BD-gain was observed for all above conditions. We suggest that this proposal be adopted in HM.

# References

- C. Yeo, Y. Tan, Z. Li, S. Rahardja, “Mode-Dependent Fast Separable KLT for Block-based Intra Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-B024, Geneva, CH, July 2010.

- A. Saxena, F. Fernandes, “Jointly optimal intra prediction and adaptive primary transform”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C108/m18131, Guangzhou, CN, October 2010

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

## Proposed changes on Working Draft 3.0

The proposed changes on Working Draft 3.0 is presented in the enclosed F225_WD.doc