| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F228 |
| --- | --- |

| Title: | A study of SCC test sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Authors: | Xingyu Zhang Oscar C. Au Wei Dai Sijin Li Chao Pang Xing Wen | Emails: | eexyzhang@ust.hk eeau@ust.hk weidai@ust.hk sliae@ust.hk pcece@ust.hk wxxab@ust.hk |
| Source: | The Hong Kong University of Science and Technology |  |  |

_____________________________

# Abstract

This report provides the coding results of submitted test sequences for SCC ad-hoc group. Results by both HM3.0 and HM3.0 SDIP are provided. Compared to HM3.0, it is reported that the average BD-rate gain from HM3.0 SDIP for AI-HE, AI-LC, RA-HE, RA-LC, LB-HE, LB-LC, LP-HE and LP-LC are 8.0 %, 9.1 %, 5.8%, 6.6%, 3.3%, 3.9%, 3.1%, and 3.7% respectively.

- Introduction

Screen content sequences have different characteristics from natural content video sequences. In order to study such sequences, we use both HM3.0 and HM3.0 SDIP software to encode them using default configurations. Except that, in “random access” and “random access loco” configurations, the intra period is set to 16, 24, 32, 48, and 64 according to frame rate 10, 20, 30, 50 and 60fps. For all test conditions (intra, random access, low delay), QP 22, 27, 32, 37 are used.

The sequences we used are downloaded from the ftp of SCC ad-hoc group, listing in Table 1.

Table 1 Sequences list on the ftp of SCC

| Class | Sequence name | Frame count | Frame rate | Bit depth | Intra | Random access | Low-delay |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | HKUST1 | 450 | 30fps | 8 | X | X | X |
| F | HKUST2 | 450 | 30fps | 8 | X | X | X |
| F | HKUST3 | 450 | 30fps | 8 | X | X | X |
| F | BasketballDrillText | 500 | 50fps | 8 | X | X | X |
| F | BasketballDriveText | 500 | 50fps | 8 | X | X | X |
| F | BQMallText | 600 | 60fps | 8 | X | X | X |
| F | BQTerraceText | 600 | 60fps | 8 | X | X | X |
| F | PartySceneText | 500 | 50fps | 8 | X | X | X |
| F | RaceHorsesText | 300 | 30fps | 8 | X | X | X |
| F | CactusText | 500 | 50fps | 8 | X | X | X |
| F | doc | 400 | 10fps | 8 | X | X | X |
| F | slide | 500 | 20fps | 8 | X | X | X |
| F | web | 500 | 10fps | 8 | X | X | X |

# Simulation results

The simulations are running on 64bit computers with Windows 7 operating system. In the following, a summary of the test result is shown in Table 2. Results from HM3.0 are compared with results from HM3.0 SDIP. A negative value means HM3.0 is less efficient than HM3.0 SDIP when encoding SCC sequences.

Sequences from HKUST and BJUT are much more artificial than Huawei’s sequences. From the results, HM3.0 SDIP outperforms HM3.0 a lot for HKUST’s and BJUT’s sequences. This shows the potential to develop suitable SCC tools for SCC scenarios.

Table 2 BD-rate increase relative to HM3.0 anchor

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class F_Huawei | -2.9 | -3.7 | -4.2 | -3.7 | -3.3 | -3.9 |
| Class F_HKUST | -15.7 | -14.4 | -14.8 | -18.4 | -15.3 | -15.2 |
| Class F_BJUT | -12.1 | -11.4 | -12.2 | -12.3 | -10.2 | -10.9 |
| All | -8.0 | -7.9 | -8.5 | -9.1 | -7.6 | -8.1 |
| Enc Time[%] | 127% |  |  | 143% |  |  |
| Dec Time[%] | 100% |  |  | 104% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class F_Huawei | -1.3 | -2.1 | -2.4 | -1.7 | -1.8 | -2.0 |
| Class F_HKUST | -12.7 | -12.1 | -12.1 | -14.7 | -13.1 | -13.1 |
| Class F_BJUT | -9.2 | -9.1 | -10.3 | -9.7 | -8.3 | -8.5 |
| All | -5.8 | -6.0 | -6.4 | -6.6 | -5.9 | -6.1 |
| Enc Time[%] | 112% |  |  | 103% |  |  |
| Dec Time[%] | 103% |  |  | 97% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class F_Huawei | -0.5 | -1.3 | -1.5 | -0.7 | -1.2 | -1.4 |
| Class F_HKUST | -6.3 | -5.2 | -5.0 | -8.4 | -6.3 | -6.1 |
| Class F_BJUT | -6.8 | -7.1 | -8.1 | -6.9 | -5.6 | -5.5 |
| All | -3.3 | -3.6 | -3.8 | -3.9 | -3.4 | -3.4 |
| Enc Time[%] | 107% |  |  | 104% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay P |  |  | Low delay P LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class F_Huawei | -0.6 | -1.4 | -1.8 | -0.8 | -1.6 | -1.5 |
| Class F_HKUST | -5.2 | -4.1 | -3.9 | -7.9 | -6.0 | -5.9 |
| Class F_BJUT | -6.8 | -6.7 | -7.4 | -6.1 | -5.0 | -6.1 |
| All | -3.1 | -3.3 | -3.6 | -3.7 | -3.4 | -3.6 |
| Enc Time[%] | 108% |  |  | 113% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

# Conclusions

This contribution provides the simulation results of SCC sequences under HM3.0 and HM3.0 SDIP respectively. Result shows the potential to improve the coding efficiency for SCC sequences.

# Patent rights declaration(s)

The Hong Kong University of Science and Technology may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).