| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F229 |
| --- | --- |

| Title: | CE7.5: Performance analysis of adaptive DCT/DST selection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Atsuro Ichigaya Yasuko Sugito Shin-ichi Sakaida NHK Science & Technology Research Laboratories 1-10-11, Kinuta, Setagaya-ku, Tokyo, Japan | Tel: Email: | +81-3-5494-3354 ichigaya.a-go@nhk.or.jp sugitou.y-gy@nhk.or.jp |
| Source: | NHK (Japan Broadcasting Corporation) |  |  |

_____________________________

# Abstract

This contribution reports the performance of the adaptive DCT/DST selection by using DST-II. The main feature of this proposal is a decoder needs only one inverse transform process with several additional switches for decoding DCT/DST coefficients both. This tool requires one bit signaling flag of each TU. Therefore the larger the TU size is, the less the ratio of signaling bit cost per coefficients bit cost is. And another feature is applicability to inter residue. In this contribution, the encoding results of 4 different setting are reported for analyzing encoding performance of TU size and intra and/or inter prediction modes.

The test (1) and (2) were conducted for evaluating fast transform type decision algorithm. The setting (1) was to apply this tool to 8x8 ~ 32x32 TUs of intra and inter both. The transform type was decided by RD-optimization. The setting (2) was to use fast transform type decision for only intra large TUs. In this case, the fast decision was applied to 16x16 and 32x32 intra TUs. The results showed the encoding time decreasing over 10% without significantly coding performance loss than the setting (1).

The test (3) and (4) were conducted for evaluating the coding performance against only inter TUs. The transform type was decided by RD-optimization. The setting (3) shows the encoding performance against inter TUs. The tool was applied to 4x4 ~ 32x32 inter TUs. The setting provided 0.7% (HE) and 0.4% (LC) gain on average. The gain is larger for higher resolutions, e.g. the average gain for ClassB on Low Delay B/P HE is 0.9% on average. And the setting (4) shows the performance against larger TUs of inter. It was applied to from 16x16 to 32x32 inter TUs. The average gain is reduced about 0.3% on average from the setting (3). However the encoding time was decreased about 10% from the setting (3) and the coding gain of ClassB is 0.7% (HE) and 0.5% (LC). In all cases except AL conditions, the decoding time is less than HM3.0.

# Introduction

The additional transform, DST-II, was proposed for transform coding in JCTVC-A122 [1]. In JCTVC-D182 [2], the implementation merit of DST-II with the conventional core transform were described. It was pointed out that the DST could be designed from DCT and did not need new additional transformation process in a decoder. Figure 1 shows N-point IDCT/IDST process. The decoder only uses this inverse transform process for IDCT and IDST both. CTRL switches coefficients reorder process and signal alternative process. It is not required an additional full inverse transform process. In short, a decoder requires only one single inverse core transform unit for DCT and DST both and it can achieve simple decoder architecture. JCTVC-E107 [3] proposed shared butterfly calculation for DCT and DST. The new butterfly calculation shares a part of operations at an encoder. The proposal achieved the reduced number of operations to calculate both transformations at same time. From the coding results, this method achieved bit reduction effect for the sequences with high complex textures.

Figure 1 N-point IDCT/IDST process

The current HM design does not use parallel implementation. Therefore our DCT/DST tool required two times RD-optimization for deciding transforms selection. In the case of using parallel processing design, the process time can be shorter in half from current encoding time as shown in Fig.2. The left one shows the flowchart of serial processing and another depicts the flowchart of parallel processing. By using parallel processing, the process time can be reduced in a half. However, additional RD-optimization process is complicated processing. Then we developed a fast algorithm to reduce encoder side complexity in the transform type decision process.

- The flowchart of serial processing 		(b)The flowchart of parallel processing

Figure 2  The flowchart of transform type decision process.

# Fast decision of transform type

This coding tool uses same type DCT and DST whose type is II. These bases are orthogonal relationship between each component. Therefore relationship of both transformations with DFT is as follows:

Xl(N)=γlWN−l2Cl(N2)−jSl(N2)

γl=1      (l=0)1/2   (l≠0)，　　WN=exp−j2πN

X(N) : DFT coefficient of N-point samples

Cl(N2) : DCT coefficient of N/2-point samples

Sl(N2) : DST coefficient of N/2-point samples

It shows the signal energy divides into DCT and DST without any energy loss and overlap. As energy compaction of one transformation is increased, another one is decreased. Both transformations have a complementary function on energy compaction. The easy way for estimating energy compaction is to calculate L1 norms. Then the transformation between DCT and DST is to be decided by comparing L1 norms of both transform coefficients.

for(i = 0; i < u iNumCoeff; i++)

{

l1 += dct_coeff[i];

l2 += dst_coeff[i];

}

if( l1 <= l2 )

{

eTransType=DCT;

}

else

{

eTransType=DST;

}

The flowcharts of this mode decision process are shown below. One is the flowchart for software implementation without parallel processing. Another one is the flowchart for hardware implementation with parallel processing. The parallel processing flowchart shows the processing time can be reduced in half.

- The flowchart of serial processing 		(b)The flowchart of parallel processing

Figure 3   Flowchart of fast decision algorithm.

# Coding experiments

We conducted 4 different coding experiments with common setting JCTVC-E700[4] for analyzing coding performance against intra and inter predictions and TU sizes. The settings are as follows.

- Normal setting: Apply to 8x8 ~ 32x32 intra and inter TUs.DCT/DST are selected by using RD-optimization.

- Fast intra TU setting: In the case of intra 16x16 and 32x32 TUs, DCT/DST are selected by using fast mode decision described in section 2

|  | Intra |  | Inter |  |
| --- | --- | --- | --- | --- |
|  | 8x8 | 16x16, 32x32 | 8x8 | 16x16, 32x32 |
| Test 1 | RDO | RDO | RDO | RDO |
| Test 2 | RDO | Fast | RDO | RDO |

- Only inter TU setting: Not apply to intra TUs. DCT/DST are selected by using RD-optimization.

- Only large inter TU setting: Apply to 16x16 and 32x32 inter TUs.

|  | Intra | Inter |  |
| --- | --- | --- | --- |
|  | 4x4~32x32 | 4x4, 8x8 | 16x16, 32x32 |
| Test 3 | N/A | RDO | RDO |
| Test 4 | N/A | N/A | RDO |

## Test1: Coding results with RD-optimization

This test was performed as a normal setting. The coding tool settings were:

-Apply DCT/DST selection to 8x8 ~ 32x32 TUs of intra and inter both

-Decide transformation type by using RD-optimization.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4 | 0.0 | -0.1 | 0.2 | -0.1 | -0.1 |
| Class B | -0.4 | -0.1 | 0.0 | 0.4 | 0.2 | 0.2 |
| Class C | -0.3 | 0.0 | 0.0 | 0.2 | 0.2 | 0.1 |
| Class D | -0.2 | 0.0 | 0.1 | 0.1 | 0.1 | 0.1 |
| Class E | -0.2 | -0.2 | -0.1 | 1.3 | 0.1 | 0.3 |
| Overall | -0.3 | -0.1 | -0.1 | 0.4 | 0.1 | 0.1 |
| Enc Time[%] | 122% |  |  | 139% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6 | 0.5 | -0.1 | 0.3 | 0.4 | 0.4 |
| Class B | -0.6 | 0.4 | 0.5 | 0.0 | 0.9 | 1.0 |
| Class C | -0.3 | 0.3 | 0.2 | 0.1 | 0.7 | 0.8 |
| Class D | -0.4 | 0.2 | 0.3 | -0.1 | 0.5 | 0.7 |
| Class E |  |  |  |  |  |  |
| Overall | -0.5 | 0.4 | 0.3 | 0.1 | 0.6 | 0.7 |
| Enc Time[%] | 115% |  |  | 115% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.0 | 0.8 | 0.5 | -0.5 | 0.7 | 0.7 |
| Class C | -0.6 | 0.4 | 0.4 | -0.2 | 0.9 | 1.0 |
| Class D | -0.7 | 0.7 | 1.2 | -0.4 | 0.9 | 1.2 |
| Class E | -0.2 | 1.7 | 2.4 | 0.6 | 2.5 | 1.6 |
| Overall | -0.7 | 0.8 | 1.0 | -0.2 | 1.1 | 1.1 |
| Enc Time[%] | 113% |  |  | 112% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9 | 0.8 | 0.6 | -0.4 | 1.0 | 1.4 |
| Class C | -0.6 | 0.5 | 0.4 | -0.2 | 0.9 | 1.0 |
| Class D | -0.7 | 1.0 | 1.0 | -0.4 | 0.8 | 0.4 |
| Class E | -0.2 | 2.1 | 1.9 | 0.5 | 1.9 | 1.7 |
| Overall | -0.6 | 1.0 | 0.9 | -0.2 | 1.1 | 1.1 |
| Enc Time[%] | 119% |  |  | 120% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

|  | AIHE | AILC | RAHE | RALC | LBHE | LBLC | LPHE | LPLC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic | -0.3 | 0.6 | -0.5 | 0.0 |  |  |  |  |
| PeopleOnStreet | -0.6 | 0.1 | -0.4 | 0.1 |  |  |  |  |
| Nebuta | -0.3 | -0.1 | -0.7 | -0.1 |  |  |  |  |
| SteamLocomotive | -0.4 | 0.4 | -0.5 | 1.2 |  |  |  |  |
| Kimono | -0.3 | 0.5 | -0.6 | -0.1 | -1.2 | -0.7 | -1.2 | -0.6 |
| ParkScene | -0.8 | -0.3 | -0.9 | -0.7 | -1.3 | -1.4 | -1.2 | -1.4 |
| Cactus | -0.5 | 0.2 | -0.5 | 0.2 | -0.7 | -0.2 | -0.6 | -0.2 |
| BasketballDrive | -0.1 | 1.4 | -0.2 | 0.9 | -0.5 | 0.8 | -0.4 | 0.9 |
| BQTerrace | -0.2 | 0.3 | -0.7 | -0.5 | -1.1 | -1.1 | -0.8 | -0.9 |
| BasketballDrill | -0.1 | 0.3 | -0.2 | 0.3 | -0.5 | 0.0 | -0.6 | 0.1 |
| BQMall | -0.2 | 0.6 | -0.3 | 0.2 | -0.5 | -0.1 | -0.6 | -0.1 |
| PartyScene | -0.2 | 0.0 | -0.5 | -0.3 | -0.9 | -0.6 | -0.8 | -0.6 |
| RaceHorses | -0.5 | -0.2 | -0.4 | 0.0 | -0.5 | -0.2 | -0.6 | -0.2 |
| BasketballPass | -0.1 | 0.6 | -0.2 | 0.3 | -0.4 | 0.2 | -0.4 | 0.1 |
| BQSquare | -0.1 | -0.1 | -0.7 | -0.4 | -1.1 | -0.7 | -1.1 | -0.6 |
| BlowingBubbles | -0.2 | 0.0 | -0.4 | -0.3 | -0.8 | -0.8 | -0.9 | -0.7 |
| RaceHorses | -0.4 | -0.2 | -0.3 | -0.1 | -0.4 | -0.2 | -0.4 | -0.4 |
| Vidyo1 | -0.1 | 1.5 |  |  | 0.0 | 0.9 | 0.0 | 0.7 |
| Vidyo3 | -0.1 | 1.2 |  |  | -0.4 | 0.3 | -0.3 | 0.3 |
| Vidyo4 | -0.2 | 1.1 |  |  | -0.2 | 0.5 | -0.4 | 0.5 |
| Average | -0.3 | 0.4 | -0.5 | 0.1 | -0.7 | -0.2 | -0.6 | -0.2 |

## Test 2: Coding results with a fast decision algorithm for intra TUs

These coding results were performed by using a fast decision algorithm. The coding tool settings were:

-Apply DCT/DST selection to 8x8 ~ 32x32 TUs

-Decide transformation type by using fast algorithm for 16x16 and 32x32 intra TUs and RD-optimization for 8x8 intra and 8x8 ~ 32x32 inter TUs.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2 | 0.0 | 0.1 | 0.5 | 0.2 | 0.2 |
| Class B | -0.2 | 0.1 | 0.2 | 0.6 | 0.5 | 0.5 |
| Class C | -0.2 | 0.1 | 0.1 | 0.3 | 0.3 | 0.3 |
| Class D | -0.1 | 0.1 | 0.1 | 0.2 | 0.2 | 0.3 |
| Class E | 0.0 | -0.1 | 0.0 | 1.4 | 0.4 | 0.5 |
| Overall | -0.2 | 0.1 | 0.1 | 0.5 | 0.3 | 0.4 |
| Enc Time[%] | 111% |  |  | 123% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6 | 0.0 | -0.2 | 0.3 | 0.4 | 0.6 |
| Class B | -0.5 | 0.6 | 0.6 | 0.1 | 1.1 | 1.1 |
| Class C | -0.4 | 0.4 | 0.1 | 0.1 | 0.9 | 0.8 |
| Class D | -0.4 | 0.5 | 0.3 | -0.1 | 0.7 | 0.8 |
| Class E |  |  |  |  |  |  |
| Overall | -0.5 | 0.4 | 0.2 | 0.1 | 0.8 | 0.8 |
| Enc Time[%] | 114% |  |  | 113% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9 | 0.9 | 0.8 | -0.5 | 1.1 | 0.9 |
| Class C | -0.6 | 0.3 | 0.5 | -0.2 | 0.9 | 1.0 |
| Class D | -0.7 | 0.7 | 0.9 | -0.4 | 1.0 | 0.8 |
| Class E | -0.2 | 0.8 | 1.5 | 0.6 | 3.7 | 1.3 |
| Overall | -0.7 | 0.7 | 0.9 | -0.2 | 1.5 | 1.0 |
| Enc Time[%] | 112% |  |  | 111% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9 | 1.0 | 0.8 | -0.4 | 1.4 | 1.5 |
| Class C | -0.6 | 0.7 | 0.6 | -0.1 | 1.0 | 1.4 |
| Class D | -0.7 | 0.9 | 0.5 | -0.4 | 1.4 | 0.7 |
| Class E | -0.1 | 1.5 | 0.9 | 0.5 | 3.7 | 1.7 |
| Overall | -0.6 | 1.0 | 0.7 | -0.2 | 1.7 | 1.3 |
| Enc Time[%] | 116% |  |  | 117% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

|  | AIHE | AILC | RAHE | RALC | LBHE | LBLC | LPHE | LPLC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic | -0.2 | 0.7 | -0.5 | 0.1 |  |  |  |  |
| PeopleOnStreet | -0.4 | 0.3 | -0.4 | 0.1 |  |  |  |  |
| Nebuta | -0.1 | 0.2 | -0.6 | -0.1 |  |  |  |  |
| SteamLocomotive | -0.1 | 0.7 | -0.7 | 1.1 |  |  |  |  |
| Kimono | -0.1 | 0.8 | -0.5 | 0.0 | -1.2 | -0.6 | -1.1 | -0.5 |
| ParkScene | -0.7 | -0.2 | -0.9 | -0.6 | -1.3 | -1.4 | -1.3 | -1.3 |
| Cactus | -0.3 | 0.4 | -0.4 | 0.3 | -0.7 | -0.1 | -0.7 | -0.1 |
| BasketballDrive | 0.1 | 1.5 | -0.2 | 1.0 | -0.4 | 0.9 | -0.3 | 0.9 |
| BQTerrace | -0.1 | 0.4 | -0.6 | -0.4 | -1.1 | -1.1 | -0.9 | -0.9 |
| BasketballDrill | -0.1 | 0.5 | -0.2 | 0.4 | -0.5 | 0.0 | -0.6 | 0.2 |
| BQMall | -0.1 | 0.7 | -0.3 | 0.3 | -0.6 | -0.2 | -0.6 | -0.1 |
| PartyScene | -0.2 | 0.0 | -0.5 | -0.3 | -0.9 | -0.6 | -0.8 | -0.6 |
| RaceHorses | -0.5 | 0.0 | -0.5 | 0.1 | -0.4 | -0.1 | -0.5 | -0.2 |
| BasketballPass | 0.0 | 0.7 | -0.1 | 0.4 | -0.4 | 0.2 | -0.4 | 0.2 |
| BQSquare | -0.1 | 0.0 | -0.8 | -0.4 | -1.2 | -0.8 | -1.0 | -0.6 |
| BlowingBubbles | -0.2 | 0.0 | -0.4 | -0.3 | -0.8 | -0.7 | -0.8 | -0.7 |
| RaceHorses | -0.3 | 0.0 | -0.2 | 0.1 | -0.5 | -0.2 | -0.5 | -0.4 |
| Vidyo1 | -0.1 | 1.6 |  |  | 0.0 | 0.8 | -0.2 | 0.7 |
| Vidyo3 | 0.0 | 1.3 |  |  | -0.4 | 0.3 | -0.2 | 0.2 |
| Vidyo4 | -0.1 | 1.4 |  |  | -0.2 | 0.7 | 0.0 | 0.5 |
| Average | -0.2 | 0.5 | -0.5 | 0.1 | -0.7 | -0.2 | -0.6 | -0.2 |

## Test 3: Coding results for inter TUs

These coding results were conducted under the coding tool settings as below.

-Apply DCT/DST selection to 4x4 ~ 32x32 “Inter” TUs

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4 | -0.1 | 0.2 | -0.1 | 0.6 | 0.8 |
| Class B | -0.4 | 0.6 | 0.7 | -0.3 | 1.4 | 1.5 |
| Class C | -0.3 | 0.4 | 0.5 | -0.1 | 1.1 | 1.1 |
| Class D | -0.4 | 0.6 | 0.6 | -0.4 | 1.4 | 1.4 |
| Class E |  |  |  |  |  |  |
| Overall | -0.4 | 0.4 | 0.5 | -0.2 | 1.1 | 1.2 |
| Enc Time[%] | 117% |  |  | 115% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9 | 1.1 | 1.1 | -0.6 | 1.4 | 1.4 |
| Class C | -0.7 | 0.8 | 0.7 | -0.5 | 1.7 | 1.8 |
| Class D | -0.7 | 1.0 | 1.4 | -0.7 | 2.1 | 2.1 |
| Class E | -0.1 | 1.4 | 1.4 | 0.6 | 2.0 | 2.1 |
| Overall | -0.7 | 1.1 | 1.1 | -0.4 | 1.7 | 1.8 |
| Enc Time[%] | 115% |  |  | 112% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9 | 1.2 | 0.9 | -0.5 | 1.7 | 2.1 |
| Class C | -0.7 | 1.0 | 0.8 | -0.5 | 1.8 | 2.0 |
| Class D | -0.7 | 1.0 | 1.2 | -0.7 | 2.1 | 2.2 |
| Class E | -0.2 | 1.8 | 1.3 | 0.5 | 2.5 | 2.4 |
| Overall | -0.7 | 1.2 | 1.0 | -0.4 | 2.0 | 2.1 |
| Enc Time[%] | 120% |  |  | 120% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

|  | RAHE | RALC | LBHE | LBLC | LPHE | LPLC |
| --- | --- | --- | --- | --- | --- | --- |
| Traffic | -0.2 | -0.2 |  |  |  |  |
| PeopleOnStreet | -0.3 | 0.1 |  |  |  |  |
| Nebuta | -0.6 | -0.1 |  |  |  |  |
| SteamLocomotive | -0.6 | 0.0 |  |  |  |  |
| Kimono | -0.4 | -0.3 | -1.1 | -0.7 | -1.1 | -0.7 |
| ParkScene | -0.5 | -0.5 | -1.4 | -1.5 | -1.3 | -1.5 |
| Cactus | -0.2 | 0.0 | -0.7 | -0.2 | -0.7 | -0.3 |
| BasketballDrive | -0.2 | 0.2 | -0.4 | 0.4 | -0.4 | 0.5 |
| BQTerrace | -0.6 | -0.6 | -0.9 | -1.0 | -0.8 | -0.6 |
| BasketballDrill | -0.1 | 0.3 | -0.7 | -0.1 | -0.6 | -0.1 |
| BQMall | -0.2 | 0.0 | -0.5 | -0.2 | -0.6 | -0.2 |
| PartyScene | -0.5 | -0.6 | -0.9 | -1.0 | -1.0 | -1.1 |
| RaceHorses | -0.3 | -0.2 | -0.5 | -0.6 | -0.6 | -0.5 |
| BasketballPass | -0.2 | 0.1 | -0.4 | 0.1 | -0.3 | 0.0 |
| BQSquare | -0.8 | -1.0 | -1.1 | -1.5 | -1.2 | -1.4 |
| BlowingBubbles | -0.4 | -0.6 | -0.9 | -1.0 | -0.9 | -1.0 |
| RaceHorses | -0.2 | -0.1 | -0.5 | -0.4 | -0.4 | -0.5 |
| Vidyo1 |  |  | 0.1 | 0.8 | 0.1 | 0.8 |
| Vidyo3 |  |  | -0.2 | 0.4 | -0.4 | 0.3 |
| Vidyo4 |  |  | -0.3 | 0.5 | -0.3 | 0.4 |
| Average | -0.4 | -0.2 | -0.7 | -0.4 | -0.7 | -0.4 |

## Test 4: Coding results for large inter TUs

These coding results were conducted under the coding tool settings as below.

-Apply DCT/DST selection to 16x16 ~ 32x32 “Inter” TUs

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4 | -0.2 | 0.0 | -0.2 | 0.2 | 0.4 |
| Class B | -0.3 | 0.1 | 0.1 | -0.2 | 0.4 | 0.4 |
| Class C | -0.1 | 0.1 | 0.0 | 0.0 | 0.3 | 0.3 |
| Class D | -0.1 | 0.0 | 0.0 | 0.0 | 0.1 | 0.2 |
| Class E |  |  |  |  |  |  |
| Overall | -0.2 | 0.0 | 0.0 | -0.1 | 0.3 | 0.3 |
| Enc Time[%] | 107% |  |  | 108% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.7 | 0.5 | 0.5 | -0.5 | 0.4 | 0.4 |
| Class C | -0.3 | 0.0 | 0.2 | -0.2 | 0.7 | 0.4 |
| Class D | -0.3 | 0.2 | 0.6 | -0.2 | 0.5 | 0.5 |
| Class E | -0.2 | 0.8 | 0.3 | 0.0 | 1.1 | 0.7 |
| Overall | -0.4 | 0.4 | 0.4 | -0.2 | 0.6 | 0.5 |
| Enc Time[%] | 107% |  |  | 106% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.6 | 0.3 | 0.0 | -0.4 | 0.8 | 0.7 |
| Class C | -0.3 | 0.2 | 0.2 | -0.2 | 0.4 | 0.5 |
| Class D | -0.2 | 0.1 | 0.0 | -0.2 | 0.5 | 0.6 |
| Class E | -0.3 | 0.7 | 0.2 | 0.0 | 1.3 | 1.0 |
| Overall | -0.3 | 0.3 | 0.1 | -0.2 | 0.7 | 0.7 |
| Enc Time[%] | 109% |  |  | 111% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

|  | RAHE | RALC | LBHE | LBLC | LPHE | LPLC |
| --- | --- | --- | --- | --- | --- | --- |
| Traffic | -0.2 | -0.2 |  |  |  |  |
| PeopleOnStreet | -0.2 | -0.1 |  |  |  |  |
| Nebuta | -0.6 | -0.2 |  |  |  |  |
| SteamLocomotive | -0.5 | -0.4 |  |  |  |  |
| Kimono | -0.4 | -0.3 | -1.0 | -0.7 | -1.0 | -0.7 |
| ParkScene | -0.2 | -0.2 | -0.7 | -0.9 | -0.6 | -0.7 |
| Cactus | -0.2 | 0.0 | -0.5 | -0.2 | -0.5 | -0.1 |
| BasketballDrive | -0.2 | 0.1 | -0.3 | 0.1 | -0.3 | 0.1 |
| BQTerrace | -0.4 | -0.3 | -0.7 | -0.7 | -0.5 | -0.5 |
| BasketballDrill | -0.1 | 0.1 | -0.5 | -0.2 | -0.3 | -0.1 |
| BQMall | -0.1 | -0.1 | -0.4 | -0.2 | -0.4 | -0.2 |
| PartyScene | -0.1 | -0.1 | -0.3 | -0.3 | -0.2 | -0.2 |
| RaceHorses | -0.1 | 0.0 | -0.2 | -0.1 | -0.2 | 0.0 |
| BasketballPass | 0.0 | 0.1 | -0.2 | -0.1 | -0.2 | -0.2 |
| BQSquare | -0.2 | 0.0 | -0.3 | -0.2 | -0.3 | -0.2 |
| BlowingBubbles | -0.1 | -0.1 | -0.4 | -0.2 | -0.3 | -0.2 |
| RaceHorses | -0.1 | 0.0 | -0.1 | -0.2 | 0.0 | -0.1 |
| Vidyo1 |  |  | -0.2 | 0.1 | -0.2 | 0.1 |
| Vidyo3 |  |  | -0.3 | -0.3 | -0.3 | -0.1 |
| Vidyo4 |  |  | -0.1 | 0.1 | -0.4 | 0.1 |
| Average | -0.2 | -0.1 | -0.4 | -0.2 | -0.3 | -0.2 |

# Conclusion

The HE coding results of the adaptive DCT/DST selection with normal setting showed bit reduction effects from 0.1% up to 1.0% from the result of test 1. And the results of test 2 showed the proposed fast decision algorithm performed reducing encoding time approximately 10% from the processing time of normal setting. The fast algorithm worked well for intra TUs. When the adaptive DCT/DST selection is applied to intra TUs, the fast algorithm is effective. On the other hand, the coding results on AILC and RALC settings showed bit increasing in both cases. This is because our proposal needs signaling flags for decoding process. It is the factor of bit increasing. The performance of the proposed tool has rooms to improve coding efficiency by improving entropy coding.

This tool had a better effect on inter TUs. The coding results of test 2 for inter TUs show up to 1.5% bit reduction. It indicates this tool works well against inter TUs. According to test 4 results, when this tool was applied to 16x16 and 32x32 inter TUs, the maximum encoding time ratio was within 111%. Current implementation does not use parallel processing. When a parallel implementation design is adopted, the encoding time is decreased in a half. The important feature is that it is possible to design a low complexity decoder for an additional transformation. And the decoding time did not increase in all cases except AI conditions. We recommend that this tool is adopted for inter residue coding.

# Reference

[1] A. Ichigaya et al., “Description of video coding technology proposal by NHK and Mitsubishi,” Doc JCTVC-A122, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Dresden, DE, April. 2010

[2] A. Ichigaya et al., “Performance report of adaptive DCT/DST selection.” Doc. JCTVC-D182, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Daegu, KR, January 2011

[3] A. Ichigaya et al., “CE7.6: Simplified adaptive transform selection.” Doc. JCTVC-E107, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, March 2011

[4] “Common test conditions and software reference configurations.” Doc. JCTVC-E700, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, March 2011

[5] Y. Shibahara and T. Nishi, “CE7: Cross Check Report for NHK's proposal (JCTVC-F229) by Panasonic.” Doc. JCTVC-F226, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Torino, IT, July 2011

[6] R. Cohen, A. Vetro and H. Sun, “CE7: Cross check of JCTVC-F229, Performance analysis of adaptive DCT/DST selection.” Doc. JCTVC-F600, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Torino, IT, July 2011

# Patent rights declaration(s)

NHK (Japan Broadcasting corporation) may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).