# Adaptive DCT/DST selection

## Inverse transformation process

The inverse transform process is depicted in figure 1. It can be implemented by two ways. The transform_type_flag distinguishes the coefficients between DCT (0) and DST (1).

(a)                                     (b)

Figure 1. The flowcharts of implementation design

Design (a) is composed with re-ordering process and signal alternation process as additional processing, Fig 1-(a). When transform_type_flag equals to 1, both additional processing become enable. The re-ordering process outputs coefficient in reverse order. These coefficients are processed by IDCT. The sign of outputted signals are inverted in the signal alternation process. The IDCT is identical to inverse core transform. Both processes are shown as follows.

Re-ordering process:

for ( Int j = 0; j < N; j++ )

{

for ( Int k = 0; k < N; k++ )

{

sCoeff[(N-1)-j][(N-1)-k] = (Short)plCoeff[j*N+k];

}

}

Signal alternation process:

for ( Int j = 0; j < N; j+=2 )

{

for ( Int k = 0; k < N; k+=2 )

{

pResidual[k] = iResi[j][k];

pResidual[k+1] = (~iResi[j][k+1])+1;

pResidual[uiStride+k] = (~iResi[j+1][k])+1;

pResidual[uiStride+k+1] = iResi[j+1][k+1];

}

pResidual += 2*uiStride;

}

Design (b) is differently-constructed. IDST process is equal to IDCT process merged with both re-order process and signal alternation process. In short, this design means in-line expansion design of former one. Both designs are identical in theory.

## The complexity analysis

The number of operations: Additional inverse DST can be composed by sharing IDCT, Fig 1-(a). Additional processes are only re-ordering and signal alternation process. The re-ordering process does not require any arithmetic operations. The signal alternation process is to alternate signs against half of the coefficients. Therefore the additional operations are only NxN/2 (N: the size of TU) adds. This design is valuable for hardware implementation. In the case of in-lined implementation, Fig 1-(b), the number of arithmetic operations is equal to the number of a core transform.

| Transform size | Operation | Design (a) | Design (b) |
| --- | --- | --- | --- |
| 4x4 | Mults Adds Shifts | 48 104 32 | 48 96 32 |
| 8x8 | Mults Adds Shifts | 352 608 128 | 352 576 128 |
| 16x16 | Mults Adds Shifts | 3752 3840 512 | 3752 3712 512 |
| 32x32 | Mults Adds Shifts | 21888 26368 2048 | 21888 25856 2048 |

Transform matrices: No additional transform matrices are required. The core transform matrices are shared.

Bit shift at intermediate levels: It is the same as a core transform.

Scanning: It is the same of HM3.0 scans.

## Syntax

| transform_coeff( x0, y0, log2TrafoSize, trafoDepth, cIdx ) { | Descriptor |
| --- | --- |
| if( ( ( cIdx = = 0 && cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 1 && cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 2 && cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) ) ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | se(v) ¦ ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| transform_coeff( x0, y0, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y0, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x0, y1, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y1, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| } else { |  |
| if( cIdx = = 0 ) { |  |
| transform_type_flag[x0][y0][ trafoDepth ] | u(1) ¦ ae(v) |
| } |  |
| if ( PredMode = = MODE_INTRA ) { |  |
| if (cIdx = = 0) |  |
| scanIdx = ScanType[ log2TrafoSize -2][ IntraPredMode ] |  |
| else |  |
| scanIdx = ScanTypeC[ log2TrafoSize -2][ IntraPredModeC ] |  |
| } else { |  |
| scanIdx = 0 |  |
| } |  |
| if ( entropy_coding_mode_flag ) |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |
| else if ( !entropy_coding_mode_flag ) |  |
| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |
| } |  |
| } |  |
| } |  |

## Semantics

transform_type_flag[x0][y0][ trafoDepth ] specifies the transform type of coefficients. When this flag is 0, it means the coefficients are DCT. On the other hand, this flag is 1, it means the coefficients are DST. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.