| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F230 |
| --- | --- |

| Title: | CE7.3: Cross-check report on Mode Dependent 2 step Transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Imformation |  |  |
| Author(s) or Contact(s): | Atsuro Ichigaya Yasuko Sugito NHK Science & Technology Research Laboratories 1-10-11, Kinuta, Setagaya-ku, Tokyo, JAPAN | Tel: Email: | +81-3-5494-3354 ichigaya.a-go@nhk.or.jp sugitou.y-gy@nhk.or.jp |
| Source: | NHK (Japan Broadcasting Corporation) |  |  |

_____________________________

# Abstract

The contribution reports cross-verification results on Mode Dependent 2 step Transform (MD2T). MD2T is joint proposal by Panasonic and Samsung. This tool is based on JCTVC-E074 [1] and JCTVC-E380 [2]. The cross-verification was conducted according to CE7 guideline. The verification task has been done successfully and the results match exactly provided by proponents.

# Cross verification

## Test conditions

The source code, which was integrated MD2T into HM3.0, was provided from proponents. Because this proposal applies to only intra TUs, the cross-verification conditions are AI (All Intra) and RA (Random Access) cases. The all verifications were performed on Linux 64 bit OS with gcc-4.1.2 with the same computer systems. Default conditions were according to JCEVC-E700 [3].

## Test results

The test results are shown in Table 1.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.5 | -0.1 | -0.2 | -0.4 | -0.4 | -0.5 |
| Class B | -0.5 | -0.3 | -0.2 | -0.3 | -0.5 | -0.5 |
| Class C | -0.3 | -0.4 | -0.3 | -0.3 | -0.4 | -0.5 |
| Class D | -0.2 | -0.3 | -0.2 | -0.2 | -0.3 | -0.2 |
| Class E | -0.7 | -0.7 | -0.7 | -0.6 | -0.6 | -0.9 |
| Overall | -0.4 | -0.3 | -0.3 | -0.3 | -0.4 | -0.5 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4 | 0.7 | 0.2 | -0.2 | -0.1 | 0.2 |
| Class B | -0.4 | -0.1 | 0.1 | -0.2 | -0.2 | -0.3 |
| Class C | -0.2 | -0.1 | -0.2 | -0.2 | -0.3 | -0.3 |
| Class D | -0.1 | -0.2 | -0.5 | -0.2 | -0.2 | -0.2 |
| Class E |  |  |  |  |  |  |
| Overall | -0.3 | 0.1 | -0.1 | -0.2 | -0.2 | -0.2 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

# Conclusion

We confirmed the implementation of MD2T and cross-checked the coding results. MD2T is implemented clear and the results were identical to their ones.

# References

[1] Y. Shibahara and T. Nishi, “CE7: Mode Dependent 2-step Transform for Intra Coding,” Doc. JCTVC-E074, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, March, 2011

[2] E. Alshina et al, “CE7: Experimental results of ROT by Samsung,” Doc. JCTVC-E380, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, March, 2011

[3] “Common test conditions and software reference configurations.” Doc. JCTVC-E700, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, March 2011