| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F234 |
| --- | --- |

| Title: | CE8, Subset 4, Tool 3: ALF decode with reduced vertical filter size (JCTVC-E060) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA Vivienne Sze Texas Instruments Inc., USA Minhua Zhou Texas Instruments Inc., USA | Tel: Email: Tel: Email: Tel: Email: | madhukar@ti.com sze@ti.com zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution presents results of Nx5-Set1 filter presented in JCTVC-E060 when integrated into HM-3.0. Nx5-Set1 filter set has a vertical size of 5 and requires a line buffer of 4 lines when compared to 6 lines used by the Nx7 filter in HM-3.0. Nx5-Set1 filter is reported to reduce memory bandwidth/memory size requirements by 33% and worst case computations by 15%. Nx5-Set1 filter is reported to result in average Y-BD-Rate loss of: 0.1% AI-HE, 0.2% RA-HE, 0.3% LB-HE.

# Introduction

HM 3.0 ALF uses three diamond shaped filters of sizes 9x7, 7x7, 5x5 as shown in Figure 1. A maximum of up to 16 filters can be signaled per slice. The worst case number of multiplications required is for 9x7 filter: 20 multiplications. The number of pixels that need to be read to carry out one filtering operation is 39.

9x7-Diamond

7x7-Diamond

5x5-Diamond

Nx7 filter set

Figure 1: HM 3.0 Luma ALF filter set.

ALF decode filtering can be carried out in either frame-based or LCU-based fashion. In frame-based ALF filtering, the SAO filter output is first stored in a frame buffer. The SAO filter output is then read to carry out ALF filtering. Figure 2 shows an example frame-based ALF filtering with size-5 filter. The red-lines in Figure 2 are pixel lines. For a size-9 filter, the SAO frame buffer needs to be read 39 times which is not feasible since the memory bandwidth requirements go up to 39 frame buffer reads. Clearly a caching mechanism is needed to reduce memory bandwidth requirements.

Figure 2: Frame-based ALF decode.

1

2

3

4

5

6

7

Filtering at (4, x)

Filtering at (5, x+1)

1

2

3

4

5

6

7

Figure 3: Frame-based ALF decode filtering with line buffer cache.

Figure 3 shows an example of frame-based ALF decode filtering with line buffer cache. The figure on left shows filtering of line 4. The figure on right shows filtering of line 5. From the Figure 3, one can see that Lines 3, 4, 5, 6 get used in filtering of both lines 4 and 5. Data on these lines can be cached in line buffers to avoid reading them again from SAO frame output buffer. With the line buffer caching mechanism shown in Figure 3, the SAO frame output buffer needs to be read only once (also written once). However, this is still expensive. Memory bandwidth requirements can be further reduced by using LCU-based filtering instead as shown in Figure 4.

Input

Output

Input

Lines that need to be cached

Output

Figure 4: LCU-based ALF decode filtering with line buffer cache.

In LCU-based filtering, ALF filtering is carried out on an LCU basis as and when the SAO filter output gets generated. The left part of Figure 4 shows ALF filtering of LCU(0,0) and the right part shows ALF filtering of LCU(1,0). The red lines show the SAO filter pixel lines (which is input to ALF filters) and the blue lines are the ALF filtered output. Since the ALF filter is a non-causal filter and uses right and bottom LCU data (which are not yet available), not all pixels in a LCU can be ALF filtered. The ALF filtered pixels are shown in solid blue line and the LCU pixels that cannot be ALF filtered when the LCU is generated are shown in dotted blue lines.

From Figure 4 it should be noted that even for LCU-based filtering, the previous N-1 lines need to be cached in line buffers to avoid additional memory bandwidth as shown in Figure 4.

Assuming a 4Kx2K image with maximum ALF filter size = 9 and IBDI on, the line buffer memory size for luma is: 4K * 6 lines *12bits = 36 Kbytes which is a significant amount for single chip video codecs.

# ALF filters with reduced vertical size

For a given image size, the vertical size of ALF filters determines the size of line buffer/memory bandwidth requirements. These requirements can be reduced by reducing the vertical size of ALF filter. This contribution presents Nx5-Set1 ALF filter set that reduces vertical size of filter to 5. Since this filter operate on only five lines instead of the original seven, the size of the line buffers goes down from 6 lines to 4 lines. Nx5-Set1 reduces memory bandwidth/memory size requirements by 33% and worst case computations by 15%.

Nx5-Set 1

Figure 5: ALF filters with reduced vertical size.

# Simulation results

Filters Nx5-Set1 was integrated into HM 3.0. Table below provides the BD-Rate savings of existing HM 3.0 ALF filter set and proposed Nx5-Set1. It can be seen from tables below that the proposed ALF filter sets capture most of the HM-3.0 ALF coding gains while reducing line buffer size/memory bandwidth by 33%.

# Proposed modifications to WD 3.0

- Proposed modifications to WD 3.0 are highlighted in yellow below.

#### 8.6.3.3.  Filtering process for luma samples

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a filter index array of (nS)x(nS), fIdx.

Output of this process is the filtered reconstruction of luma picture.

Let s’ be a variable specifying luma sample array recPictureL.

A variable nS is set equal to ( 1 << log2CUSize ) and a variable alfTap is set equal to ( alf_length_luma_minus_5_div2 << 1 ) + 5.

Each sample of luma picture recFiltPictureL[ xC + x ][ yC + y ] with x, y = 0..(nS)-1, is derived as follows.

recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=0N−1s'[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]	(8-1)
where N is set equal to AlfNumCoeffLuma-1 and horPos[i] and verPos[i] are specified in Table 8- and Table 8-, respectively.

Table 8-12 – Specification of horPos[ i ] according to alfTap for adaptive loop filter process of luma samples

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alfTap = 5 | 0 | -1 | 0 | 1 | -2 | -1 | 0 | 1 | 2 | -1 | 0 | 1 | 0 | - | - | - | - | - | - | - |
| alfTap = 7 | -1 | 0 | 1 | -2 | -1 | 0 | 1 | 2 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | -2 | -1 | 0 | 1 | 2 |
| alfTap = 9 | -2 | -1 | 0 | 1 | 2 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | -4 | -3 | -2 | -1 | 0 | 1 | 2 | 3 |

| i | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alfTap = 5 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| alfTap = 7 | -1 | 0 | 1 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| alfTap = 9 | 4 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | -2 | -1 | 0 | 1 | 2 | - | - | - | - | - | - |

Table 8-3 – Specification of verPos[ i ] according to alfTap for adaptive loop filter process of luma samples

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alfTap = 5 | -2 | -1 | -1 | -1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 2 |  |  |  |  |  |  |  |
| alfTap = 7 | -2 | -2 | -2 | -1 | -1 | -1 | -1 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 |
| alfTap = 9 | -2 | -2 | -2 | -2 | -2 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| i | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alfTap = 5 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| alfTap = 7 | 2 | 2 | 2 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| alfTap = 9 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | - | - | - | - | - | - |

Figure 8-8 Mapping between geometric position and luma adaptive loop filter index according to alfTap (informative)

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).