| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F240 |
| --- | --- |

| Title: | CE6.c: Crosscheck of DCIM (case 4) (JCTVC-F566) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-verification |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: | madhukar@ti.com |
| Source: | Texas Instruments |  |  |

_____________________________

# Abstract

This contribution presents results of cross-verification of DCIM proposal of JCTVC-F566. Software provided by JCTVC-E318 proponents was used in simulation. The following flags were set: Case4: USE_DCIM=1, DCIM_SUBDIR=1, DCIM_BIPRED=1, TAP4_FILT=0. The RD data of this test case was compared with HM-3.0 anchor to see how much gain can be obtained without 4Tap in DCIM. It is reported that the BD-Rate savings over HM-3.0 are: AI HE: -0.4, AI-LC: -0.1. The bit-rate numbers matched with those provided by E318 proponents. Decoder time ratios on heterogeneous Linux cluster is reported to be in range from 98%-105% and encoder time ratios on heterogeneous Linux cluster is asserted to be in range from 109%-121%.

# Simulation results

Encoder and decoder simulations were run on Linux cluster. Table below presents a summary of BD-Rate and encoder/decoder run-times in comparison with HM-3.0 anchors.

|  | TI cross-check | Sharp | TI cross-check | Sharp | TI cross-check | Sharp |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rate | BD-Rate | Enc Time | Enc Time | Dec Time | Dec Time |
| AI HE | -0.4 | -0.4 | 109% | 111% | 98% | 103% |
| AI LC | -0.1 | -0.1 | 121% | 121% | 105% | 105% |