| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F241 |
| --- | --- |

| Title: | CE10: Crosscheck of Samsung/FastVDO's contribution on core transform - normal QP range (JCTVC-F251) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-verification |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: | madhukar@ti.com |
| Source: | Texas Instruments |  |  |

_____________________________

# Abstract

This contribution presents results of cross-verification of Samsung/FastVDO core transform proposal in JCTVC-F251. Samsung/FastVDO and TI BD-Rate numbers matched for normal QP range. Samsung/FastVDO’s proposal is asserted to have average Y BD-Rate in the range of: 0.0%-0.1%. Decoder time ratios on heterogeneous Linux cluster is asserted to be in range from 88%-96% and encoder time ratios on heterogeneous Linux cluster is asserted to be in range from 90%-96%.

# Simulation results

Encoder and decoder simulations were run on Linux cluster. Table below presents a summary of BD-Rate and encoder/decoder run-times in comparison with HM 3.0 anchors.

|  | TI cross-check | Samsung/ FastVDO | TI cross-check | Samsung/ FastVDO | TI cross-check | Samsung/ FastVDO |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rate | BD-Rate | Enc Time | Enc Time | Dec Time | Dec Time |
| AI HE | 0.1 | 0.1 | 92% | 100% | 90% | 98% |
| RA HE | 0.0 | 0.0 | 94% | 99% | 88% | 99% |
| LB HE | 0.0 | 0.0 | 96% | 99% | 94% | 99% |
| AI LC | 0.1 | 0.1 | 95% | 97% | 93% | 96% |
| RA LC | 0.0 | 0.0 | 90% | 99% | 80% | 98% |
| LB LC | 0.0 | 0.0 | 96% | 99% | 96% | 97% |

Details:

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2 | -0.1 | -0.1 | 0.3 | -0.1 | -0.2 |
| Class B | 0.0 | 0.0 | -0.1 | 0.1 | -0.1 | -0.1 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class D | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | -0.1 | 0.0 | 0.1 | -0.1 | 0.0 |
| Overall | 0.1 | -0.1 | -0.1 | 0.1 | -0.1 | -0.1 |
| Enc Time[%] | 92% |  |  | 95% |  |  |
| Dec Time[%] | 90% |  |  | 93% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | -0.2 | -0.5 | 0.1 | -0.2 | -0.2 |
| Class B | 0.0 | 0.0 | 0.1 | 0.0 | -0.1 | -0.1 |
| Class C | 0.0 | 0.2 | -0.2 | 0.0 | -0.1 | -0.1 |
| Class D | 0.0 | -0.1 | -0.2 | 0.0 | -0.3 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | -0.2 | 0.0 | -0.2 | -0.1 |
| Enc Time[%] | 94% |  |  | 90% |  |  |
| Dec Time[%] | 88% |  |  | 80% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | -0.2 | 0.0 | -0.1 | -0.6 |
| Class C | 0.0 | -0.1 | -0.2 | 0.0 | 0.1 | -0.2 |
| Class D | -0.1 | -0.1 | 0.2 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | -0.8 | 0.4 | 0.0 | -0.2 | -1.4 |
| Overall | 0.0 | -0.2 | 0.0 | 0.0 | -0.1 | -0.5 |
| Enc Time[%] | 96% |  |  | 96% |  |  |
| Dec Time[%] | 94% |  |  | 96% |  |  |