| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F245 |
| --- | --- |

| Title: | CE8: Crosscheck of CE8.5: Unified Chroma Filter Shapes with Luma's Shapes for ALF (JCTVC-F157) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-verification |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: | madhukar@ti.com |
| Source: | Texas Instruments |  |  |

_____________________________

# Abstract

This contribution presents results of cross-verification of CE8.5: Unified Chroma Filter Shapes with Luma's Shapes for ALF (JCTVC-F157). Configuration tested had: maximum vertical size of chroma filter: 7, diamond + cross filter shapes selected for chroma using RDO. It is reported that the BD-Rate savings over HM31-dev-adcs are in the range: Y:0.0, U:-0.5 to 0.0, V:-0.4 to 0.1. BD-Rate numbers matched with those provided by the proponents. Decoder time ratios on heterogeneous Linux cluster is reported to be in range from 93%-99% and encoder time ratios on heterogeneous Linux cluster is reported to be in range from 95%-98%. Due to variance in runtime of simulation environment, the runtime is inaccurate and should not be used to infer the complexity of the proposed method.

# Simulation results

Encoder and decoder simulations were run on Linux cluster. Table below presents a summary of BD-Rate, dQP bits, and encoder/decoder run-times in comparison with HM31-dev-adcs anchor. The following options were set in TypeDef.h:

// restrict vertical size for rectangle chroma filter up to 7

#define ALF_CHROMA_MAX_VSIZE7          1

// use diamond and cross shape filter for chroma

#define ALF_CHROMA_NEW_SHAPES          1

// use RDO for decide chroma filter shape (diamond or cross, w/o filtering)

#define ALF_CHROMA_RDO                 1

|  | TI cross-check | MQT chroma | TI cross-check | MQT chroma | TI cross-check | MQT chroma | TI cross-check | MQT chroma | TI cross-check | MQT chroma |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | Y | U | U | V | V | Enc Time | Enc Time | Dec Time | Dec Time |
| AI HE | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 | 95% | 101% | 93% | 99% |
| RA HE | 0.0 | 0.0 | -0.2 | -0.2 | 0.1 | 0.1 | 98% | 100% | 98% | 99% |
| LB HE | 0.0 | 0.0 | -0.5 | -0.5 | -0.4 | -0.4 | 97% | 100% | 99% | 100% |