| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F247 |
| --- | --- |

| Title: | CE3: DCT derived interpolation filter test by Samsung. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A.Alshin, E.Alshina #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +82 10 30261305 alexander_b.alshin@samsung.com elena_a.alshina@samsung.com |
| Source: | Samsung Electronics, Ltd |  |  |

_____________________________

# Abstract

This is Samsung’s test in scope of CE3. Filter coefficients generation algorithm is described for both odd and even filter length. In total 5 different test are presented from pure 6 taps (1.19% drop compare to HM3.0) to combination for different 8 taps filters (0.12% gain). Complexity of each interpolation filter solution is measured based on actual hit-ratio of each filter usage in full-frame test.

# Introduction

DCT derived interpolation filter is used by Samsung and other companies in their interpolation schemes. HM3.0 uses 8 taps DCTIF for Luma and 4 taps DCTIF for Chroma.

Generation algorithm. Here we provide the formula for filter coefficients generation. Before we set the number of taps in DCTIF to even number [1]. There is generalization for arbitrary number of taps. For example, it can be odd number, as shown on Fig. 1.

Fig. 1 Odd number of interpolation filter taps.

Let’s assume that we have values in integer positions pl, -MminlMmax and we need to calculate p in fractional position . Filter size is Size = Mmax- Mmin.

.

(1)

Here is smoothing-window size, .are boundary of reference pixels template. Window size controls smoothing properties of windowing function.

For real application float-point filter (10) coefficients are scaled by S and rounded. During rounding normalization (filter coefficients sum is equal to S) should be considered.

For example, HM3.0 interpolation filter coefficients are generated by formulas (1) for even number of taps. Parameters values are: Mmin = -3, Mmax=4, =1/4 and 1 *.sh/2, N=13 (smoothing window).

Table 1. HM3.0 DCTIF coefficients generation parameters.

|  | Filter coefficients for reference pixels {p-3, p-2, p-1, p0, p1, p2, p3, p4,} | N window size | S | “*” | “+” |
| --- | --- | --- | --- | --- | --- |
| 1/4 | {-1,4,-10,57,19,-7,3,-1} | 13 | 64 | 6 | 7 |
| 1/2 | { -1,4,-11, 40,40,-11,4,-1} | 13 | 64 | 3 | 7 |

Together with HM3.0 8 taps filter 6 and 7 taps are tested in this proposal. Filter coefficients generation parameters are show in Table 2. The number of operations needed for each filter realization is shown as well.

Table 2. Generation parameters for additional filter used in this contribution.

|  | Filter coefficients for reference pixels { p-2, p-1, p0, p1, p2, p3} | N window size | S | “*” | “+” |
| --- | --- | --- | --- | --- | --- |
| 1/4 | { 2, -9, 57, 19, -7, 2} | 13 | 64 | 5 | 5 |
| 1/2 | { 1, -7, 38, 38, -7, 1} | 7 | 64 | 2 | 5 |
| 1/2 | {2, -9, 39, 39, -9, 2,} | 13 | 64 | 3 | 5 |
|  | Filter coefficients for reference pixels {p-3, p-2, p-1, p0, p1, p2, p3} | N window size | S | “*” | “+” |
| 1/4 | {-2, 5, -11, 58, 18, -6, 2} | no | 64 | 6 | 6 |
|  | Filter coefficients for reference pixels {p-3, p-2, p-1, p0, p1, p2, p3, p4,} | N window size | S | “*” | “+” |
| 1/4 | { -1, 4, -11, 57, 20, -7, 3, -1}, | no | 64 | 6 | 7 |
| 1/2 | { -1, 4, -10, 39, 39, -10, 4, -1}, | 10.3 | 64 | 3 | 7 |

# Test description and results

Test 2.1. Pure 6-taps filter. The following filter coefficients were used in this test:

1/4: { 2,  -9,  57,  19,  -7,   2}

1.2: { 1,  -7,  38,  38,  -7,   1}

This filter can be used for essential complexity reduction of MC process.

Because only one filter is used motion estimation and motion compensation stages are synchronized. Test results under test conditions recommended in [2] and [3] are shown in Table 3.

Table 3. Pure 6 taps DCTIF.

|  | Random access |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | -0.1 | -0.1 | -0.4 | 0.4 | 0.5 |
| Class B | 0.3 | 0.0 | 0.2 | 0.0 | 0.0 | -0.1 |
| Class C | 1.4 | 0.8 | 0.8 | 2.1 | 0.8 | 0.8 |
| Class D | 2.9 | 1.7 | 1.6 | 4.8 | 2.2 | 2.0 |
| Class E |  |  |  |  |  |  |
| All | 1.1 | 0.6 | 0.6 | 1.6 | 0.8 | 0.8 |
|  | Low delay |  |  | Low delayLC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.3 | 0.3 | 0.0 | 0.0 | 0.0 |
| Class C | 1.1 | 1.0 | 1.1 | 2.0 | 1.2 | 1.2 |
| Class D | 2.3 | 2.3 | 2.2 | 4.7 | 2.3 | 2.6 |
| Class E | 0.1 | 0.2 | -0.2 | -1.4 | -0.9 | -0.7 |
| All | 0.9 | 0.9 | 0.9 | 1.4 | 0.7 | 0.8 |
|  | Low delay P |  |  | Low delay LCP |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.4 | 0.2 | 0.0 | -0.8 | -0.1 | 0.3 |
| Class C | 1.5 | 1.3 | 1.2 | 1.8 | 0.8 | 1.1 |
| Class D | 3.1 | 2.5 | 2.0 | 4.2 | 2.3 | 2.6 |
| Class E | 0.1 | -0.1 | -0.4 | -1.9 | -1.3 | -0.5 |
| All | 1.3 | 1.0 | 0.7 | 0.9 | 0.5 | 0.9 |

An average across 6 test cases drop is 1.19%. The maximum drop 14% was observed for (BqSquare, RA-LC), sometimes this filter provides gain vs 8 taps HM3.0 filter (3.5% Vidyo3, LD(P)-LC). Complexity reduction due to this change will be shown below.

Test 2.2. 6-taps filter for bi-prediction and 8-tasp for uni-directional prediction. The same filter as in Test 2.1 was applied for bi-directional prediction in this case (all fractional positions, all block sizes). For uni-directional prediction 8 taps HM3.0 filter was used.

This combination reduces complexity of bi-prediction, which uses interpolation twice. For this complicated cases short length filter is used. ME uses 8-taps filter only on a first stage, but use actual MC interpolation of mode-decision stage. So encoding process in this test is slightly different from real application scenario.

Table 4. 6 taps DCTIF for bi-prediction and 8-taps for uni-prediction.

|  | Random access |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | -0.1 | 0.4 | -0.3 | 0.2 | 0.4 |
| Class B | 0.3 | 0.1 | 0.2 | 0.0 | 0.1 | 0.1 |
| Class C | 1.1 | 0.7 | 0.7 | 1.4 | 0.6 | 0.6 |
| Class D | 2.5 | 1.7 | 1.5 | 3.4 | 1.7 | 1.7 |
| Class E |  |  |  |  |  |  |
| All | 0.9 | 0.6 | 0.6 | 1.1 | 0.6 | 0.7 |
|  | Low delay |  |  | Low delayLC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.3 | 0.2 | -0.4 | 0.2 | -0.1 |
| Class C | 0.6 | 0.7 | 0.7 | 0.7 | 0.6 | 0.5 |
| Class D | 1.3 | 1.4 | 1.9 | 2.1 | 1.2 | 0.9 |
| Class E | -0.3 | -0.1 | -0.1 | -1.9 | -0.5 | -0.5 |
| All | 0.4 | 0.6 | 0.7 | 0.2 | 0.4 | 0.2 |
|  | Low delay P |  |  | Low delay LCP |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |

An average across 6 test cases drop is lower than in Test 2.1 (0.44%). The maximum drop 10% was observed, sometimes this filter provides gain vs 8 taps HM3.0 filter (-3.54%, Vidyo3, LD(B)-LC). There  is performance change in LD-P test case because there is no filter change for uni-prediction compare to HM3.0.

Complexity analysis for this combination is shown in section 3.

Test 2.3. 6-taps filter for 1/4 pel positions in bi-prediction and 8-tasp for remaining cases. Compare to Test 2.2 hit-ratio of 6-taps filter was reduced here. Half-pel filter is symmetrical and requires less number of operations so we allow 8 taps 1/2-pel filter for bi-perfection in this test. All comments about non-real encoding scenario are valid in this case as well.

Table 5. 6 taps DCTIF for 1/4 pel bi-prediction and 8-taps for remaining cases.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | 0.2 | 0.2 | 0.1 | 0.4 |
| Class B | 0.1 | 0.1 | 0.2 | 0.0 | 0.1 | 0.1 |
| Class C | 0.4 | 0.4 | 0.3 | 0.7 | 0.3 | 0.3 |
| Class D | 0.9 | 0.7 | 0.6 | 1.5 | 0.7 | 0.8 |
| Class E |  |  |  |  |  |  |
| All | 0.3 | 0.2 | 0.3 | 0.5 | 0.3 | 0.4 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | 0.2 | 0.1 | -0.1 | 0.1 | 0.1 |
| Class C | 0.3 | 0.4 | 0.5 | 0.6 | 0.2 | 0.2 |
| Class D | 0.5 | 1.1 | 0.9 | 1.1 | 0.6 | 0.5 |
| Class E | -0.1 | -0.5 | -0.2 | -0.4 | -0.3 | -0.6 |
| All | 0.1 | 0.4 | 0.3 | 0.3 | 0.2 | 0.1 |

An average across 6 test cases drop is again lower than before (0.22%). The maximum drop 4.8% was observed, sometimes this filter provides gain vs 8 taps HM3.0 filter (-0.7%). No performance changes in LD-P test case because there is no filter change for uni-prediction compare to HM3.0.

Complexity analysis for this combination is shown in section 3.

Test 2.4. 7 taps filter for 1/4 and 6 taps filter for 1/2

This test is targeting for complexity reduction. Interpolation filter doesn’t depend of  bi- or uni-directional prediction or position in a block. All half-pel positions are interpolated using 6-taps filter, and all quarter-pel positions are interpolated by 7 taps filter:

1/4: {-2,  5,  -11,  58,   18,  -6,   2, 0}

1.2: {0,  2,   -9,  39,   39,  -9,   2, 0}

There is no need to discuss ME and MC synchronization.

Table 6. Low complexity interpolation filter (7 taps 1/3-pel and 6-taps, 1/2-pel).

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | -0.3 | -0.2 | -0.2 | 0.0 | 0.1 |
| Class B | 0.0 | -0.1 | 0.0 | 0.0 | 0.1 | 0.0 |
| Class C | 0.1 | 0.1 | 0.1 | 0.2 | 0.1 | 0.1 |
| Class D | 0.1 | 0.1 | 0.1 | 0.5 | 0.2 | 0.3 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | -0.1 | 0.0 | 0.1 | 0.1 | 0.1 |
| Enc Time[%] | 96% |  |  | 95% |  |  |
| Dec Time[%] | 97% |  |  | 96% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | 0.1 | -0.4 | -0.3 | -0.6 |
| Class C | 0.2 | 0.0 | 0.1 | 0.2 | 0.1 | 0.1 |
| Class D | 0.5 | 0.3 | 0.4 | 0.8 | 0.4 | 0.6 |
| Class E | -0.2 | -0.7 | -0.4 | -1.0 | -0.8 | -0.7 |
| Overall | 0.2 | 0.0 | 0.1 | -0.1 | -0.1 | -0.1 |
| Enc Time[%] | 95% |  |  | 94% |  |  |
| Dec Time[%] | 97% |  |  | 96% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2 | 0.1 | -0.1 | -0.8 | -0.2 | 0.1 |
| Class C | 0.6 | 0.5 | 0.3 | 0.2 | 0.3 | 0.3 |
| Class D | 1.2 | 1.1 | 0.6 | 1.0 | 0.8 | 0.6 |
| Class E | 0.0 | 0.1 | -0.5 | -1.8 | -1.1 | -1.0 |
| Overall | 0.5 | 0.4 | 0.1 | -0.3 | 0.0 | 0.1 |
| Enc Time[%] | 97% |  |  | 95% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

An average (6 test cases) performance change with anchor is 0.07%/ 0.06%/0.04% (Y/U/V). The maximum drop was observed is 3%(BQSquare, LDP-HE), the maximum gain was observed in this test is 3.5% (Vydio3, LDP-LC).

Complexity analysis for this filter will be shown in section 3.

Test 2.5. Combination of 8 taps filter for uni- and bi-prediction.

In the last test two different 8 taps filters were applied for uni- and bi-directional predictions. Coefficients are only slightly different from HM3.0:

Uni-directional predictions:

Q:	{  -1,   4, -10,  57,  19,  -7,   3,  -1},

H:	{  -1,   4, -10,  39,  39, -10,   4,  -1},

Bi-directional predictions:

Q:	{  -1,   4, -11,  57,  20,  -7,   3,  -1},

H:	{  -1,   4, -11,  40,  40, -11,   4,  -1},

All comments about non-real encoding scenario are valid in this case as well. Test results for this test are shown in Table 7.

Table 7. Combination of two different 8-taps filters (bi-/uni-directional prediction filter switch).

|  | Random access |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.1 | 0.1 | 0.2 | 0.3 |
| Class B | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | -0.1 | -0.2 | -0.1 | -0.2 |
| Class D | -0.1 | -0.2 | -0.2 | -0.4 | -0.1 | -0.2 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | -0.1 | -0.2 | 0.0 | 0.0 |
|  | Low delay |  |  | Low delayLC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.3 | 0.4 | -0.2 | 0.1 | -0.2 |
| Class C | 0.0 | 0.0 | 0.0 | -0.2 | 0.2 | -0.1 |
| Class D | 0.1 | -0.1 | 0.5 | -0.1 | -0.1 | 0.2 |
| Class E | 0.0 | 0.1 | -0.2 | -0.5 | -0.3 | -0.9 |
| All | 0.0 | 0.1 | 0.2 | -0.2 | 0.0 | -0.2 |
|  | Low delay P |  |  | Low delay LCP |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | -0.6 | -0.3 | -0.3 | -0.5 |
| Class C | -0.1 | -0.1 | -0.1 | -0.2 | -0.2 | -0.5 |
| Class D | 0.1 | 0.0 | -0.2 | 0.2 | -0.2 | -0.1 |
| Class E | -0.3 | 0.1 | -0.6 | -1.3 | -1.1 | -0.6 |
| All | 0.0 | 0.0 | -0.4 | -0.3 | -0.4 | -0.4 |

Average performance improvement in this test is 0.12% for Luma and 0.1% for Chroma component.

# Complexity analysis

As it was shown above performance difference between different interpolation filter schemes is not that big. The accurate complexity estimation is very important.

Unfortunately there is almost no difference in encoding/decoding time between anchor and modifications. Complexity estimation traditionally used in CE3 assumes that all block sizes and all fractional positions are equiprobable. This is not true in real life and interpolation filter change (for example, application of simpler filter for some specific cases) may change hit-ratio of these cases essentially.

In order to estimate complexity as much accurate as possible we count actual number of calls for each interpolation filter. In order to do it reference s/w was slightly modified: counter for total number of pixels (g_d_num_MC_pel), predicted by motion compensation, and counters for number of mults (g_d_num_mults), additions (g_d_num_adds) and number of neighbour pixels need to acces (g_d_num_mem_band) in MC part were included. Introduced counters automatically take into account block extension in case of 2D interpolation.

Luma and Chroma components share the same MV so modification of Luma filter which increase (for example) hit-ratio of fractional positions leads to complexity increment of Chroma MC. So numbers of computations and memory access are accumulated for both Luma and Chroma components.

Code changes are summarized in in Table 8. More details can be found in attached code.

Table 8. Code modification for MC statistics collection.

Start of decoding process: g_d_num_MC_pel =0; g_d_num_adds=0; g_d_num_mults=0; g_d_num_mem_band=0;

At the beginning of motionCompensation : g_d_num_MC_pel +=iHeight*iWidth;

At the beginning of xPredInterLumaBlk_ha and xPredInterLumaBlk : { Int iPoz = ixFrac + 4 * iyFrac; case 0: g_d_num_mem_band+=iWidth *iHeight; break; case 2: g_d_num_mults+=iHeight*iWidth*LUMA_H_L; g_d_num_adds+=iHeight*iWidth*(LUMA_H_L-1); g_d_num_mem_band+=(iWidth +LUMA_H_L-1)*iHeight; break; case 8: g_d_num_mults+=iHeight*iWidth*LUMA_H_L; g_d_num_adds+=iHeight*iWidth*(LUMA_H_L-1); g_d_num_mem_band+=(iHeight+LUMA_H_L-1)*iWidth; break; case 1: case 3: g_d_num_mults+=iHeight*iWidth*LUMA_Q_L; g_d_num_adds+=iHeight*iWidth*(LUMA_Q_L-1); g_d_num_mem_band+=iHeight*(iWidth+LUMA_Q_L-1); break; case 4: case 12: g_d_num_mults+=iHeight*iWidth*LUMA_Q_L; g_d_num_adds+=iHeight*iWidth*(LUMA_Q_L-1); g_d_num_mem_band+=(iHeight+LUMA_Q_L-1)*iWidth; break; case 10: g_d_num_mults+=iHeight*(iWidth+LUMA_H_L-1)*LUMA_H_L+iHeight*iWidth*LUMA_H_L; g_d_num_adds+=iHeight*(iWidth+LUMA_H_L-1)*(LUMA_H_L-1)+iHeight*iWidth*(LUMA_H_L-1); g_d_num_mem_band+=(iHeight+LUMA_H_L-1)*(iWidth+LUMA_H_L-1); break; case 9: case 11: g_d_num_mults+=iHeight*(iWidth+LUMA_Q_L-1)*LUMA_H_L+iHeight*iWidth*LUMA_Q_L; g_d_num_adds+=iHeight*(iWidth+LUMA_Q_L-1)*(LUMA_H_L-1)+iHeight*iWidth*(LUMA_Q_L-1); g_d_num_mem_band+=(iHeight+LUMA_H_L-1)*(iWidth+LUMA_Q_L-1); break; case 6: case 14: g_d_num_mults+=iHeight*(iWidth+LUMA_H_L-1)*LUMA_Q_L+iHeight*iWidth*LUMA_H_L; g_d_num_adds+=iHeight*(iWidth+LUMA_H_L-1)*(LUMA_Q_L-1)+iHeight*iWidth*(LUMA_H_L-1); g_d_num_mem_band+=(iHeight+LUMA_Q_L-1)*(iWidth+LUMA_H_L-1); break; case 5: case 7: case 13: case 15: g_d_num_mults+=iHeight*(iWidth+LUMA_Q_L-1)*LUMA_Q_L+iHeight*iWidth*LUMA_Q_L; g_d_num_adds+=iHeight*(iWidth+LUMA_Q_L-1)*(LUMA_Q_L-1)+iHeight*iWidth*(LUMA_Q_L-1); g_d_num_mem_band+=(iHeight+LUMA_Q_L-1)*(iWidth+LUMA_Q_L-1); break; default: assert(0); break; }

At the beginning of xDCTIF_FilterC_ha and xDCTIF_FilterC : if (ixFrac ==0 && iyFrac ==0) g_d_num_mem_band+=iWidth*iHeight; else if (ixFrac ==0) g_d_num_mem_band+=(iWidth +CHROMA_L-1)*iHeight; else if (iyFrac ==0) g_d_num_mem_band+=(iHeight +CHROMA_L-1)*iWidth; else g_d_num_mem_band+=(iHeight +CHROMA_L-1)*(iWidth+CHROMA_L-1);

At the beginning of each Chroma IF functions (like xCTI_Filter_VPS04_C_HAL) g_d_num_mults+=CHROMA_L; g_d_num_adds+=(CHROMA_L-1);

End of decoding process: printf("\n MultsPerPixel:: %12.4f\n", g_d_num_mults/g_d_num_MC_pel); printf("\n AddsPerPixel:: %12.4f\n", g_d_num_adds/g_d_num_MC_pel); printf("\n MemBandPerPixel:: %12.4f\n", g_d_num_mem_band/g_d_num_MC_pel);

At the end of decoding process the number of multiplications per pixel, additions per pixel and memory access per pixel is printed-out into the log file, together with decoding time. So collection statistics for each filter type is as easy as parsing decoding time from log-files.

The complexity of each filter is determined by number of taps. This means that number of multiplication is equal to number of taps, and number of additions is equal to number of taps minus 1. This doesn’t take into account optimization based on symmetry of filter. If some coefficients are equal to 1 then actual multiplication is not needed. This hand-optimization can be applied or not applied for each interpolation filter which is highly dependent on particular implementation. Proposed complexity metric doesn’t take into account this hand-optimization.

Complexity metric described here was agreed on BoG meeting on interpolation MC. And complexity measurement was performed for the most part of proposals in CE3 and related contribution by additional decoder run. Report for the worst case memory band was agreed to be mandatory as well because this number shows implementation cost of analyzed interpolation filter solutions.

Summary of complexity / performance data obtained in these additional experiments is presented by Table 9.

Table 9. Comparison of different proposals from view point of proposed complexity metric.

| JCTVC-F | 468 | 100 | 100 | 100 | 101 | 315 | 248 | 599 | 599 | 574* | 247 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AIF | (1) | (2) | (3) | Uni/bi | 8/6 | 8/4 | 8h8q | 8h7q | 6h7q | 6h7q |
| Luma BD-rate |  |  |  |  |  |  |  |  |  |  |  |
| Avg 6 | -0,47 | -0,03 | 0,07 | 0,17 | -0,36 | 0,13 | 0,05 | -0,38 | -0,48 | 0.05 | 0,07 |
| Min | -8,9 | -1,0 | -0,8 | -0,3 | -9,9 | -0,9 | -0,68 | -4,3 | -5,9 | -7,8 | -3,5 |
| Max | 2.5 | 0,4 | 1,3 | 1,3 | 2,0 | 2,8 | 0,84 | 1,2 | 2,0 | 8,5 | 3,0 |
| Average complexity |  |  |  |  |  |  |  |  |  |  |  |
| Mult | 87% | 97% | 94% | 93% | 96% | 94% | 99% | 108% | 101% | 93% | 85% |
| Adds | 84% | 97% | 94% | 93% | 96% | 93% | 99% | 108% | 99% | 91% | 83% |
| MemBand | 95% | 93% | 93% | 87% | 98% | 98% | 99% | 102% | 100% | 97% | 95% |
| The Worst Case |  |  |  |  |  |  |  |  |  |  |  |
| MemBand | 83% | 67% | 64% | 40% | 100% | 100% | 50% | 100% | 100% | 83% | 83% |

(*) This test shows 0.8% drop of Chroma BD-rate.

This complexity analysis shows interesting things which were not visible in old complexity metric used in CE3 (assuming equal probability of each fractional positions). For example 8H+7Q filter proposed in F-599 uses shorter filter but doesn’t bring any complexity reduction. Proposal F-248 cuts the half of memory access in the worst case by using twice shorter interpolation filters for small bi-predictive PUs. There is almost no effect on coding efficiency under common test conditions. But this complexity metric shows that actual complexity reduction is ~1% only this means that hit-ratio of blocks small bi-predictive PUs is very small. Under another test conditions (like LCU size 16) an effect of this modification will be higher.

We conclude that proposed complexity metric shows clearly not only average gain but an average complexity as well. This analysis is very useful in order to determine better trade-off between complexity and performance.

Modified template for CE on motion compensation results presentation was also agreed during BG discussion. New template is used for test results representation in recent revision of this proposal.

# Conclusion

Among tested interpoaltion scheme solid (non-switchable) 7 taps 1/4-pel and 6 taps 1/2-pel filter:

Q: {-2,  5,  -11,  58,   18,  -6,   2}

H: {2,   -9,  39,   39,  -9,   2,}

shows the best trade-off between performance and complexity :

- minor drop 0.07% of Luma BD-rate with 16% average rediction of computations

- 17% reduction of memory band-width in the worst case

- no degradation for Chroma component BD-rate

We propose to adopt this interpolation filter for next version of HM s/w and WD.

We propose to use described complexity metric in future core experiments on interpolation filters.

# References

- Elena Alshina, Jianle Chen, Alexander Alshin, Nikolay Shlyakhov,Woo-Jin Han, “CE3: Experimental results of DCTIF by Samsung,” JCTVC-D344, Daegu, Korea, Jun. 2011.

- F. Bossen Common HM test conditions and software reference configurations JCTVC-E700, 5th. JCTVC, Meeting, Geneva, Mar. 2011.

- Takeshi Chujoh, Elena Alshina “Core Experiment 3: Motion Compensation Interpolation”, JCTVC-E703, 5th. JCTVC, Meeting, Geneva, Mar. 2011.

- Kenji Kondo, Teruhiko Suzuki, “CE3: Results on MC boundary filter (tool 4)” JCTVC-E100, 6th. JCTVC, Meeting, Turino, July. 2011.

- G. Bjøntegaard, “Calculation of Average PSNR Differences between RD curves,” VCEG-M33, 13th VCEG Meeting, Austin, USA, April 2001

# Patent rights declaration(s)

Samsung Electronics, Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).