| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F251 |
| --- | --- |

| Title: | CE10: Full-factorized core transform proposal by Samsung/FastVDO. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | E.Alshina A.Alshin, Il-Koo Kim #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea Pankaj Topiwala Columbia, MD. Cocoa Beach, FL. USA | Tel: Email: | +82 10 30261305 elena_a.alshina@samsung.com alexander_b.alshin@samsung.com ilkoo.kim@samsung.com pankaj@fastvdo.com |
| Source: | Samsung Electronics, Ltd, FastVDO |  |  |

_____________________________

# Abstract

This is the CE10 submission by Samsung and FastVDO. It is claimed that the proposed scheme supports all three forms of implementation: matrix multiplication, partial butterfly and full-factorization. All three forms are reportedly identical in terms of performance.  This reported equivalence allows choosing the implementation according to platform: i.e. matrix multiplication form may be preferred for SIMD implementations while full-factorization may be preferred for platforms with strict limitations for number of multiplications. By utilizing Loeffler’s factorization in the 32pt transform, it is reported that a 4X reduction in the number of multiplications compared to partial butterfly, and a 12X reduction compared to matrix multiplication, is achieved. Performance testing under common test conditions appears to show no performance change compare to HM3.0 for Luma component (0.0%) with small gain for Chroma component (-0.1%/-0.2%). Scalar quantization is reportedly unchanged compared to HM3.0  for all transform sizes.

# Introduction

HM3.0 uses core transforms introduced by [1]. This transform is an approximation of the DCT. It is implemented in two equivalent forms in HM3.0: as matrix multiplications and as so-called partial butterfly. This convertibility allows a variety of implementations according to platform. Partial butterfly is fast transform implementation providing about 3 times reduction for number of multiplications compared to matrix multiplications form. Quantization process introduced in [1] uses scalar quantization which essentially reduces memory for quantization matrices.

In this proposal we keep all these advantages but use different approximations for DCT transform which additionally allows full Loeffler’s factorization. Briefly, proposed changes of core transforms compared to [1] can be described in 2 steps. At first step transform matrix elements were multiplied by 64. Each 1D transform in HM3.0 uses its own de-scaling left shift (in order to keep output in predetermined range). Values of all these shifts were increased by 6 in order to compensate transform matrix element increment. So dynamic range of each buffer for temporal data storage is not changed compare to HM3.0. First step of our modifications does not change performance. At second step scaled by 64 transform matrix elements were modified in absolute values in order to allow higher degree of factorization compare to partial butterfly. These modifications lead to performance change.

Modified approximations of DCT matrices keeps fundamental properties of it’s symmetry. In particular 4x4 transform matrix is embedded into 8x8, 16x16 and 32x32 matrices, 8x8 is embedded into 16x16 and 32x32 and 16x16 transform matrix is part of 32x32 transform.

Transform matrix for 4x4 is not modified compared to [1] in our proposal.

# Three equivalent forms

The proposed transforms can be implemented in three ways:

- matrix multiplications,

- partial butterfly,

- full-factorization form.

First and second form are also supported by HM3.0, while the third one becomes available after proposed changes for transform matrices. The code for matrix multiplication and partial butterfly is similar to HM3.0 but uses different values of transform matrices elements. The HM3.0 transform matrix elements are represented in 8 bits. For this proposal, transform matrix elements are represented in 14 bits.

All equivalent forms use multiplications. In matrix multiplications and partial butterfly forms first multiplier is transform matrix element (14 bits in our design) and another multiplier is de-quantized coefficient or content of temporal buffer after 1st inverse transform. Both are supposed to be within 16 bits. If proposed here transform is implemented as matrix multiplication or partial butterfly then both multipliers don’t exceed 16 bits. Matrix multiplication form is targeting to SIMD implementation. Partial butterfly form is targeting for platform sensitive to bit-depth of each multipliers (16 bits are guaranteed) but with higher cost for multiplication operation (the number of multiplications is ~3 times smaller for partial butterfly).

The third form (full-factorization) is targeting for h/w platforms with strict restrictions for number of multiplications (~4 times smaller than partial butterfly). The full-factorization form may have up to two consecutive multiplications. One multiplier is 8 bits number in proposed design. Another multiplier is 16 bits number (as well as for partial butterfly) for first multiplication in a path. But for second multiplications in a path one multiplier can be over 16 bits. H/w implementation analysis was done in CE10, and shows that the gate count for the full-factorized transform is 20%-38% (depends on frequency) smaller than for partial butterfly, despite using higher bit-depth in multiplications. So full-factorization form looks preferable for h/w implementation.

The source code attached to this contribution supports all three equivalent implementations. Switching between these three implementations is controlled by these two macros, MATRIX_MULT and PARTIAL_BUTTERFLY, as follows:

- matrix multiplication; MATRIX_MULT=1; PARTIAL_BUTTERFLY=0;

- partiall butterfly: MATRIX_MULT=0; PARTIAL_BUTTERFLY=1;

- full factorization: MATRIX_MULT=0; PARTIAL_BUTTERFLY=0;

# Transform flow-graph in case of full-factorization

There is proposed forward transform flow-chart on Fig.1. Elements of this flow-chart are explained in Table 1. The whole transform process is split on elemenatry parts. Each part operates with no more than two values: X,Y – input and  U,V – output. Bold font indicates predetermined constant values. Numbers next to each Given’s rotations in this scheme specify parameters of each rotation (exact value of parameters can be found in Appendix A).

Proposed algorithm is fully determined by 37 parameters (mentioned in Table 6). Most of them are cosine values, scaled by 64 and rounded. While choosing the rounding for these parameters we minimized distortion between inverse and transpose matrices (moved toward orthogonal transform).

Fig. 1 Flow-chart of forward transform.

Table 1 Explanation for elementary parts of flow-graphs .

| Notation on a graph | Meaning | Implementation | Operations number and type |
| --- | --- | --- | --- |
|  | Given’s rotation: U=C1* X- C0* Y V= C0*X+ C1*Y | T = C0*(X+Y) U=C1_p_C0* X- T V= T+ C1_m_C0*Y (C1_p_C0= C1+C0 C1_m_C0= C1-C0) | “*” 3, “+” 3 |
|  | U= X+ Y V= X-Y |  | “+” 2 |
|  | U = X << 6 |  | “<<” 1 |
|  | U = X << 12 |  | “<<” 1 |
|  | U = X *90 |  | “*” 1 |
|  | U = X -Y |  | “+” 1 |
|  | U = X + Y |  | “+” 1 |

Fig. 1 clearly shows that in proposed full-factorized transform there are no more than 2 consecutive operations increasing bit-depth (multiplications or left shifts) in each line. In Samsung and FastVDO previous proposal factorized transform used up to 3 operations increasing bit-depth in some lines [3]. In order to avoid overflow in old design it was necessary to apply de-scaling (right-shifts) after intermediate stages. With no more than 2 multiplications in each line we don’t need right-shift at intermediate stages.

After first stage process is split into so-called even and odd parts. Even part of 32-point transform is 16-point trasnfrom. Even part of 16-point transform is 8-point transform and even part of 8-point transform is 4-point transform. This is results of keeping embedded structure of transform matrices.

# Inverse transform flow graph in case of full-factorization.

The flow-chart of inverse transform looks slightly less elegant. It is shown on Fig. 2. It contains the same elements as forward transform (Table 1). Numbers next to each Given’s rotations in this scheme specify parameters of each rotation as well as on Fig 1(but the same number s on Fig.1 and 2 not mandatory means the same parameters of rotation.

The number of operations in forward and inverse transform is the same.

# The Number of operations

As shown in third column of Table 1 Given’s rotations are implemented using 3 multiplications and 3 additions only. There are 27 Given’s rotations in forward and inverse proposed DCT-like trasnfrom.

The total number of opeartions for  8 pt, 16 pt and 32 pt proposed forward full-factorized transform is shown in Table 2.

The number of multiplications for 32pt transform compare to partial butterfly is almost 4 times smaller. Overall number of operations including all block sizes is about twice smaller than in HM3.0 partial butterfly.

Table 2. The number of operations for proposed factorized transform compare to partial butterfly (HM3.0).

|  |  | HM 3.0 |  | Proposed |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Matrix multiplication | Partial-butterfly | Matrix multiplication | Partial-butterfly | Full-factorization |
| 8-point | Additions | 896 | 448 | 896 | 448 | 464 |
|  | Multiplications | 1024 | 384 | 1024 | 384 | 176 |
| 16-point | Additions | 7680 | 3200 | 7680 | 3200 | 2592 |
|  | Multiplications | 8192 | 2816 | 8192 | 2816 | 992 |
| 32-point | Additions | 63488 | 23808 | 63488 | 23808 | 14656 |
|  | Multiplications | 65536 | 22016 | 65536 | 22016 | 5568 |

# Dynamic range analysis

Following CE10 description we’ve analyzed dynamic range of proposed inverse transform at any stage of decoding process. The software framework provided by Cisco/TI was used as agreed in CE10. Results of this analysis are shown in Table 3. There no data out of 16 bits at any stage of proposed inverse transform.

Table 3. Bit-depth at various stage of inverse transform.

|  | IBDI ON |  |  |  | IBDI OFF |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4×4 | 8×8 | 16×16 | 32×32 | 4×4 | 8×8 | 16×16 | 32×32 |
| Output of inverse quantization | 12 | 13 | 14 | 13 | 12 | 13 | 14 | 13 |
| Input to the1st stage of inverse transform | 16 | 16 | 16 | 14 | 16 | 16 | 16 | 14 |
| Input to the 2nd stage of inverse transform | 15 | 16 | 16 | 15 | 15 | 16 | 16 | 15 |
| Output of the 2nd stage of inverse transform | 11 | 12 | 12 | 12 | 9 | 10 | 10 | 10 |

# Number of sequential operations

As mentioned in chapters 2-3 there is no more than 2 multiplications in the longest execution path. In the worst case execution path uses 2 multiplications and 5 additions. There are no left (de-scaling) shifts at any intermediate stages.

# Quantization/dequantization matrices

There is no change of quantization process compare to HM3.0:  6 scalar values for quantization and 6 scalar values for de-quantization.

# H/w implementation analysis

H/W implementation analysis was done for all proposals in CE10. Automatic RTL generation tool from C code, Catapult C of Mentor Graphics, is used to minimize the influence of level of optimization. Results for proposed core transform implemented in full-factorized form compare to HM3.0 partial butterfly are shown in Table 4.

Table 4. H/w implementation analysis.

| Frequency | Gate Count(Design Compiler) |  |  |  |
| --- | --- | --- | --- | --- |
|  | HM3.0 (partial butterfly) |  | FULL_FACTORIZATION |  |
| 150MHz | 178,803 | 100% | 135,480 | 76% |
| 180MHz | 205,971 | 100% | 141,171 | 69% |
| 200MHz | 238,769 | 100% | 147,153 | 62% |

| Frequency | Latency |  |  | Throughput Cycle |
| --- | --- | --- | --- | --- |
|  | HM3.0 (partial butterfly) | FULL_FACTORIZATION | The same for all |  |
| 150MHz | 4 | 6 | 1 |  |
| 180MHz | 4 | 5 | 1 |  |
| 200MHz | 6 | 5 | 1 |  |

The proposed algorithm was implemented in HM3.0. Only odd part of partial butterfly in HM3.0 is modified by this proposal. All other things are the same with [1]. Even 4 pt transform matrix was not modified compare to [1].

There is no quantization part change compare to HM3.0 (in other words we use flat quantization).

According to CE10 requirements proposed modification of HM3.0 transform was tested at normal, low and high QP range [2] for 6 test cases (AI-HE, AI-LC, RA-HE, RA-LC, LD(B)-HE, LD(B)_LC). Test results are presented in Tables 5 (normal), 6 (low), 7 (high QP).

Table 5. Normal QP test (full-length simulation).

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2 | -0.1 | -0.1 | 0.3 | -0.1 | -0.2 |
| Class B | 0.0 | 0.0 | -0.1 | 0.1 | -0.1 | -0.1 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class D | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | -0.1 | 0.0 | 0.1 | -0.1 | 0.0 |
| All | 0.1 | -0.1 | -0.1 | 0.1 | -0.1 | -0.1 |
| Enc Time[%] | 100% |  |  | 97% |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  |
|  | Random access |  |  |  | Random access LoCo |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | -0.2 | -0.5 | 0.1 | -0.2 | -0.2 |
| Class B | 0.0 | 0.0 | 0.1 | 0.0 | -0.1 | -0.1 |
| Class C | 0.0 | 0.2 | -0.2 | 0.0 | -0.1 | -0.1 |
| Class D | 0.0 | -0.1 | -0.2 | 0.0 | -0.3 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | -0.2 | 0.0 | -0.2 | -0.1 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | -0.2 | 0.0 | -0.1 | -0.6 |
| Class C | 0.0 | -0.1 | -0.2 | 0.0 | 0.1 | -0.2 |
| Class D | -0.1 | -0.1 | 0.2 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | -0.8 | 0.4 | 0.0 | -0.2 | -1.4 |
| All | 0.0 | -0.2 | 0.0 | 0.0 | -0.1 | -0.5 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 97% |  |  |

In average there is no performance difference with HM3.0 for Luma component (0.0%), and there is small average gain for U (0.1%) and V (0.2%) components in this test.

Table 6. Low QP test (2sec simulation).

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.76 | 0.33 | 0.34 | 0.97 | 0.67 | 0.74 |
| Class B | 0.26 | 0.05 | 0.03 | 0.30 | 0.14 | 0.00 |
| Class C | 0.13 | 0.01 | 0.01 | 0.74 | 0.32 | 0.27 |
| Class D | 0.12 | -0.01 | -0.01 | 0.77 | 0.40 | 0.36 |
| Class E | 0.13 | -0.04 | -0.02 | 0.20 | -0.26 | -0.24 |
| All | 0.29 | 0.07 | 0.07 | 0.60 | 0.27 | 0.24 |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.53 | 0.08 | 0.09 | 0.64 | 0.06 | 0.07 |
| Class B | 0.13 | 0.02 | 0.01 | 0.13 | -0.07 | -0.11 |
| Class C | 0.08 | -0.05 | -0.09 | 0.20 | 0.06 | -0.02 |
| Class D | 0.04 | -0.07 | -0.07 | 0.16 | -0.02 | 0.08 |
| Class E |  |  |  |  |  |  |
| All | 0.19 | 0.00 | -0.01 | 0.27 | 0.00 | 0.00 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | #VALUE! |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.13 | 0.00 | -0.01 | 0.15 | -0.03 | -0.06 |
| Class C | 0.08 | -0.08 | -0.08 | 0.17 | -0.01 | 0.00 |
| Class D | 0.02 | -0.02 | -0.07 | 0.08 | -0.10 | 0.01 |
| Class E | 0.03 | -0.08 | -0.11 | 0.04 | 0.10 | -0.05 |
| All | 0.07 | -0.04 | -0.06 | 0.12 | -0.02 | -0.03 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

In average there is 0.26% average drop compare to HM3.0 for Luma component in this test. Drop mainly comes due to 2 10 bits test sequences (Nebuta and Steam Locomotive).

Table 7. High QP test (2sec simulation).

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | -0.2 | 1.5 | 0.1 | -0.2 | -0.2 |
| Class B | 0.0 | -0.1 | -0.4 | 0.0 | -0.2 | 0.0 |
| Class C | 0.0 | -0.5 | 0.2 | 0.0 | -0.2 | 0.0 |
| Class D | -0.1 | 2.2 | 0.3 | 0.1 | -0.1 | -0.1 |
| Class E | 0.0 | -0.2 | -0.1 | 0.0 | 0.1 | -0.1 |
| All | 0.0 | 0.3 | 0.3 | 0.0 | -0.1 | -0.1 |
| Enc Time[%] | 100% |  |  | 97% |  |  |
| Dec Time[%] | 95% |  |  | 92% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1 | -0.5 | 1.1 | 0.0 | 0.4 | -2.3 |
| Class B | 0.1 | -1.4 | 0.3 | -0.2 | 0.5 | 0.1 |
| Class C | -0.3 | -1.3 | 1.5 | -0.1 | -2.2 | 0.3 |
| Class D | -0.1 | 0.0 | -0.3 | -0.1 | -1.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | -0.9 | 0.6 | -0.1 | -0.5 | -0.5 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.6 | -2.8 | 0.0 | 0.5 | -1.4 |
| Class C | -0.1 | -0.4 | 0.0 | -0.1 | -1.1 | -0.7 |
| Class D | 0.4 | 3.6 | -0.5 | -0.1 | 3.1 | -0.7 |
| Class E | 0.1 | -2.3 | 0.2 | -0.4 | 1.6 | 0.7 |
| All | 0.1 | 0.5 | -1.0 | -0.1 | 0.9 | -0.7 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 97% |  |  |

An average performance difference in this test compare to HM3.0 is 0.0%/0.0%/-0.2% (Y/U/V).

# Description for Working Draft

This description was borrowed from F-446 to emphasize how minor are proposed changes compare to HM3.0 of transform design.

10.1. Basis vectors

The basis vectors of the 4x4 and 8x8 transforms are shown below. The basis vectors of the 16x16 and 32x32 transforms are shown in Appendix B.

4x4 transform:

{ 4096, 4096, 4096, 4096},

{ 5312, 2304,-2304,-5312},

{ 4096,-4096,-4096, 4096},

{ 2304,-5312, 5312,-2304}

8x8 transform:

{ 4096, 4096, 4096, 4096, 4096, 4096, 4096, 4096,},

{ 5696, 4800, 3264, 1088,-1088,-3264,-4800,-5696,},

{ 5312, 2304,-2304,-5312,-5312,-2304, 2304, 5312,},

{ 4770,-1080,-5670,-3240, 3240, 5670, 1080,-4770,},

{ 4096,-4096,-4096, 4096, 4096,-4096,-4096, 4096,},

{ 3240,-5670, 1080, 4770,-4770,-1080, 5670,-3240,},

{ 2304,-5312, 5312,-2304,-2304, 5312,-5312, 2304,},

{ 1088,-3264, 4800,-5696, 5696,-4800, 3264,-1088,},

10.2 Intermediate scaling and quantization/dequantization.

In this subsection, intermediate scaling and quantization/de-quantization are described in more detail. In addition to specifying the scaling in the de-quantization and the inverse transform, we also specify the forward transform and quantizer that we have used for our simulation results.

Definitions

B	= internal bit depth (as specified by InternalBitDepth in the common config files)

N 	= transform size

M	= log2(N)

Q	= f(QP%6), 	where f(x)   =   {26214,23302,20560,18396,16384,14564}, x=0,…,5

IQ 	= g(QP%6), 	where g(x)  =   {40,45,51,57,64,72}, x=0,…,5

Forward transform

The following applies to the implementation of the forward transform.

- The forward transform is specified as a matrix multiplication in two stages, horizontal and vertical.

- After the first stage, the resulting transform coefficients are scaled as follows

y  =  (x + offset)>>(M-1+(B-8)+6),		offset = (1<<(M-2+ (B-8)+6))

- After the second stage, the resulting transform coefficients are scaled as follows

y  =  (x + offset)>>(M+6+6),		offset = (1<<(M+5+6))

This ensures a bit width of 16 after each transform stage.

Quantization

Quantization can be implemented as follows (RDOQ=off):

level  =  (coeff*Q + offset)>>(21+QP/6–M–(B-8))

For RDOQ, the scaling and cost function calculations are adjusted accordingly.

Dequantization

The dequantizer is specified as follows

coeffQ 	=  ((level*IQ << (QP/6)) + offset)>>(M-1+(B-8)),  offset = 1<<(M-2+(B-8))

coeffQ = min(32767,max(-32768,coeffQ))

The clipping ensures that the value of the quantized transform coefficients, coeffQ, can be represented with 16 bits. The scaling is chosen so that clipping only occur for extreme input values and/or unreasonable quantizer behaviour.

Inverse transform

The following applies to the implementation of the inverse transform.

- The inverse transform is specified as a matrix multiplication in two stages, horizontal and vertical.

- After the first stage, the resulting transform coefficients are scaled as follows

y  =  (x + offset)>>(7+6)			offset = 1<<(7-1+6)

This ensures a bit width of 16 after the first inverse transform stage assuming the quantization/dequantization scheme above.

- After the second stage, the resulting transform coefficients are scaled as follows

y  =  (x + offset)>>(12-(B-8)+6)		offset = 1<<(11-(B-8)+6)

Quantization step size and QP

The quantizer multipliers, f(x) and dequantizer multipliers, g(x) are chosen so that

f(x) * g(x)  (1<<20), 	x = 0,1,…,5

Furthermore, assume that we have an N*N DC block with pixel values equal to X.  If this block is input to an orthonormal transformation containing a DC basis vector (e.g. the Discrete Cosine Transform), the value of the DC coefficient is equal to X*N.  With the proposed transforms and quantization/ dequantiztion scheme, the same relationship between input pixel values and the value of level applies when QP = 4. In other words, if all pixel values in an NxN block are equal to X, and QP=4, this will result in a DC coefficient with level = X*N after quantization. The range of QP values maps to approximately the same range of quantization step sizes as in HM2.0.

# Conclusion

# Based on test results:

# 0.0%/-0.1%/-0.2% (average performance across all 6 test cases in normal, low and high QP tests)

# we would like to recommend adoption for proposed core transform modification for HM4.0 and WD.

# This will keep possibility of 2 equivalent implementations:

# matrix multiplication

# partial butterfly

# available in HM3.0 and add one more equivalent implementation

# full-factorized form

# which uses 4 times smaller number of multiplications in 32 pt transform compare to default HM3.0 configuration (partial butterfly).

# References

[1] A. Fuldseth, G. Bjøntegaard (Cisco), M. Sadafale, M. Budagavi (TI), “Transform design for HEVC with 16 bit intermediate data representation”, JCTVC-E243, 5th. JCTVC, Meeting, Geneva, Mar. 2011

[2] P. Topiwala, M. Budagavi, A. Fuldseth, R. Joshi, I.-K. Kim, “Description of Core Experiment 10 (CE10): Core Transform Design”JCTVC-E710, 5th. JCTVC, Meeting, Geneva, Mar. 2011

[3] I.-K. Kim, T. Lee, J. Chen, V. Seregin, Y. Hong, W.-J. Han (Samsung), P. Topiwala (FastVDO), “CE10: Fast integer transform based on modified Loeffler’s factorization”, JCTVC-E700, 5th. JCTVC, Meeting, Geneva, Mar. 2011

# Patent rights declaration(s)

Samsung Electronics, Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Fast VDO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

APPENDIX  A

Table 6. Notations for input parameters of proposed algorithm.

| Addreviation | Explanation | Example Value |
| --- | --- | --- |
| S _BIT | S = 1<< S_BIT | 6 |
| SQRT_2 | int(sqrt(2.0) * S ) | 90 |
| COS8_SQRT_2 | int(cos( 8*pi/64) *sqrt(2.0) * S) | 83 |
| COS16_SQRT_2 | int(cos( 16*pi/64) *sqrt(2.0) * S) | 64 |
| COS24_SQRT_2 | int(cos( 24*pi/64) *sqrt(2.0) * S) | 36 |
| COS4_SQRT_2 | int(cos( 4*pi/64) * S) | 63 |
| COS12_SQRT_2 | int(cos( 12*pi/64) * S) | 53 |
| COS20_SQRT_2 | int(cos( 20*pi/64) * S) | 36 |
| COS28_SQRT_2 | int(cos( 28*pi/64) * S) | 12 |
| COS2_SQRT_2 | int(cos(2*pi/64) * S) | 64 |
| COS6_SQRT_2 | int(cos( 6*pi/64) * S) | 61 |
| COS10_SQRT_2 | int(cos( 10*pi/64) * S) | 56 |
| COS14_SQRT_2 | int(cos( 14*pi/64) * S) | 49 |
| COS18_SQRT_2 | int(cos( 18*pi/64) * S) | 41 |
| COS22_SQRT_2 | int(cos( 22*pi/64) * S) | 30 |
| COS26_SQRT_2 | int(cos( 26*pi/64) * S) | 19 |
| COS30_SQRT_2 | int(cos( 30*pi/64) * S) | 6 |
| COS1_SQRT_2 | int(cos( 1*pi/64) * S) | 64 |
| COS3_SQRT_2 | int(cos( 3*pi/64) * S) | 63 |
| COS5_SQRT_2 | int(cos( 5*pi/64) * S) | 62 |
| COS7_SQRT_2 | int(cos( 7*pi/64) * S) | 60 |
| COS9_SQRT_2 | int(cos( 9*pi/64) * S) | 58 |
| COS11_SQRT_2 | int(cos( 11*pi/64) * S) | 55 |
| COS13_SQRT_2 | int(cos( 13*pi/64) * S) | 51 |
| COS15_SQRT_2 | int(cos(15*pi/64) * S) | 47 |
| COS17_SQRT_2 | int(cos( 17*pi/64) * S) | 43 |
| COS19_SQRT_2 | int(cos( 19*pi/64) * S) | 38 |
| COS21_SQRT_2 | int(cos( 21*pi/64) * S) | 33 |
| COS23_SQRT_2 | int(cos( 23*pi/64) * S) | 27 |
| COS25_SQRT_2 | int(cos( 25*pi/64) * S) | 22 |
| COS27_SQRT_2 | int(cos( 27*pi/64) * S) | 16 |
| COS29_SQRT_2 | int(cos( 29*pi/64) * S) | 9 |
| COS31_SQRT_2 | int(cos( 31*pi/64) * S) | 3 |
| COS4_SQRT_2 | int(cos( 4*pi/64) *sqrt(2.0) * S) | 89 |
| COS12_SQRT_2 | int(cos( 12*pi/64) *sqrt(2.0) * S) | 75 |
| COS20_SQRT_2 | int(cos( 20*pi/64) *sqrt(2.0) * S) | 50 |
| COS28_SQRT_2 | int(cos( 28*pi/64) *sqrt(2.0) * S) | 18 |

Table 7. Parameters values for Given’s rotations in forward factorized DCT-like trasnform.

| Given’s rotation | C0 | C1_p_C0 | C1_m_C0 |
| --- | --- | --- | --- |
| #1 | COS8_SQRT_2<<S_BIT | (COS24_SQRT_2<<S_BIT)+ (COS8_SQRT_2<<S_BIT) | (COS24_SQRT_2<<S_BIT)- (COS8_SQRT_2<<S_BIT) |
| #2 | COS20 | COS20+COS12 | -COS20+COS12 |
| #3 | COS28 | COS28+COS4 | -COS28+COS4 |
| #4 | COS18 | COS18+COS14 | -COS18+COS14 |
| #5 | -COS10 | -COS10+COS22 | COS10+COS22 |
| #6 | COS26 | COS26+COS6 | -COS26+COS6 |
| #7 | -COS2 | -COS2+COS30 | COS2+COS30 |
| #8 | COS8_SQRT_2 | COS24_SQRT_2+COS8_SQRT_2 | COS24_SQRT_2-COS8_SQRT_2 |
| #9 | -COS24_SQRT_2 | -COS24_SQRT_2+COS8_SQRT_2 | COS24_SQRT_2+COS8_SQRT_2 |
| #10 | -COS15 | -COS15+COS17 | COS15+COS17 |
| #11 | COS19 | COS19+COS13 | COS19-COS13 |
| #12 | -COS11 | - COS11+COS21 | COS11+COS21 |
| #13 | COS23 | COS23+ COS9 | COS23- COS9 |
| #14 | -COS7 | -COS7+COS25 | COS7+COS25 |
| #15 | COS27 | COS27+ COS5 | COS27- COS5 |
| #16 | -COS3 | -COS3+ COS29 | -COS3+ COS29 |
| #17 | COS31 | COS31+ COS1 | COS31- COS1 |
| #18 | -COS20 | -COS20+COS12 | COS20+COS12 |
| #19 | -COS28 | -COS28+COS4 | COS28+COS4 |
| #20 | COS28 | COS28+COS4 | -COS28+COS4 |
| #21 | COS20 | COS20+COS12 | -COS20+COS12 |
| #22 | -COS24_SQRT_2 | COS8_SQRT_2-COS24_SQRT_2 | COS8_SQRT_2+COS24_SQRT_2 |
| #23 | -COS24_SQRT_2 | COS8_SQRT_2-COS24_SQRT_2 | COS8_SQRT_2+COS24_SQRT_2 |
| #24 | -COS20_SQRT_2 | -COS20_SQRT_2+COS12_SQRT_2 | COS20_SQRT_2+COS12_SQRT_2 |
| #25 | -COS28_SQRT_2 | -COS28_SQRT_2+COS4_SQRT_2 | COS28_SQRT_2+COS4_SQRT_2 |
| #26 | COS28_SQRT_2 | COS28_SQRT_2+COS4_SQRT_2 | -COS28_SQRT_2+COS4_SQRT_2 |
| #27 | COS20_SQRT_2 | COS20_SQRT_2+COS12_SQRT_2 | -COS20_SQRT_2+COS12_SQRT_2 |

Table 8. Parameters values for Given’s rotations in inverse factorize DCT-like trasnform.

| Given’s rotation | C0 | C1_p_C0 | C1_m_C0 |
| --- | --- | --- | --- |
| #1 | COS8_SQRT_2<<S_BIT | (COS24_SQRT_2<<S_BIT)+ (COS8_SQRT_2<<S_BIT) | (COS24_SQRT_2<<S_BIT)- (COS8_SQRT_2<<S_BIT) |
| #2 | -COS20 | -COS20+COS12 | COS20-COS12 |
| #3 | COS4 | COS28+COS4 | COS28-COS4 |
| #4 | COS24_SQRT_2 | COS24_SQRT_2+COS8_SQRT_2 | -COS24_SQRT_2+COS8_SQRT_2 |
| #5 | COS24_SQRT_2 | COS24_SQRT_2+COS8_SQRT_2 | -COS24_SQRT_2+COS8_SQRT_2 |
| #6 | -COS18 | -COS18+COS14 | COS18+COS14 |
| #7 | -COS22 | COS10-COS22 | COS10+COS22 |
| #8 | -COS30 | COS2-COS30 | COS2+COS30 |
| #9 | -COS26 | -COS26+COS6 | COS26+COS6 |
| #10 | COS20_SQRT_2 | COS20_SQRT_2-COS12_SQRT_2 | -COS20_SQRT_2-COS12_SQRT_2 |
| #11 | COS28_SQRT_2 | COS28_SQRT_2+COS4_SQRT_2 | -COS28_SQRT_2+COS4_SQRT_2 |
| #12 | COS4_SQRT_2 | COS28_SQRT_2+COS4_SQRT_2 | COS28_SQRT_2-COS4_SQRT_2 |
| #13 | COS20_SQRT_2 | COS20_SQRT_2+COS12_SQRT_2 | -COS20_SQRT_2+COS12_SQRT_2 |
| #14 | COS4 | COS28+COS4 | COS28-COS4 |
| #15 | COS12 | -COS20+COS12 | -COS20-COS12 |
| #16 | COS24_SQRT_2 | COS8_SQRT_2+COS24_SQRT_2 | COS8_SQRT_2-COS24_SQRT_2 |
| #17 | -COS24_SQRT_2 | -COS8_SQRT_2-COS24_SQRT_2 | -COS8_SQRT_2+COS24_SQRT_2 |
| #18 | COS12 | COS20+COS12 | COS20-COS12 |
| #19 | -COS28 | -COS28-COS4 | COS28-COS4 |
| #20 | COS23 | COS23-COS9 | -COS23- COS9 |
| #21 | -COS21 | -COS11-COS21 | -COS11+COS21 |
| #22 | COS19 | COS19+COS13 | -COS19+COS13 |
| #23 | -COS17 | COS15-COS17 | COS15+COS17 |
| #24 | COS31 | COS31- COS1 | -COS31- COS1 |
| #25 | -COS29 | -COS3- COS29 | -COS3+ COS29 |
| #26 | COS27 | COS27+ COS5 | -COS27+ COS5 |
| #27 | COS25 | COS7+COS25 | COS7-COS25 |

APPENDIX B

4x4 transform:

{ 4096, 4096, 4096, 4096},

{ 5312, 2304,-2304,-5312},

{ 4096,-4096,-4096, 4096},

{ 2304,-5312, 5312,-2304}

8x8 transform:

{ 4096, 4096, 4096, 4096, 4096, 4096, 4096, 4096,},

{ 5696, 4800, 3264, 1088,-1088,-3264,-4800,-5696,},

{ 5312, 2304,-2304,-5312,-5312,-2304, 2304, 5312,},

{ 4770,-1080,-5670,-3240, 3240, 5670, 1080,-4770,},

{ 4096,-4096,-4096, 4096, 4096,-4096,-4096, 4096,},

{ 3240,-5670, 1080, 4770,-4770,-1080, 5670,-3240,},

{ 2304,-5312, 5312,-2304,-2304, 5312,-5312, 2304,},

{ 1088,-3264, 4800,-5696, 5696,-4800, 3264,-1088,},

16x16 transform:

{	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	},

{	5760,	5504,	5120,	4480,	3712,	2688,	1664,	512,	-512,	-1664,	-2688,	-3712,	-4480,	-5120,	-5504,	-5760,	},

{	5696,	4800,	3264,	1088,	-1088,	-3264,	-4800,	-5696,	-5696,	-4800,	-3264,	-1088,	1088,	3264,	4800,	5696,	},

{	5543,	3568,	619,	-2802,	-5096,	-5747,	-4506,	-1639,	1639,	4506,	5747,	5096,	2802,	-619,	-3568,	-5543,	},

{	5312,	2304,	-2304,	-5312,	-5312,	-2304,	2304,	5312,	5312,	2304,	-2304,	-5312,	-5312,	-2304,	2304,	5312,	},

{	5167,	474,	-4379,	-5528,	-1806,	3773,	5728,	2591,	-2591,	-5728,	-3773,	1806,	5528,	4379,	-474,	-5167,	},

{	4770,	-1080,	-5670,	-3240,	3240,	5670,	1080,	-4770,	-4770,	1080,	5670,	3240,	-3240,	-5670,	-1080,	4770,	},

{	4410,	-2700,	-5490,	540,	5760,	1710,	-5040,	-3690,	3690,	5040,	-1710,	-5760,	-540,	5490,	2700,	-4410,	},

{	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	},

{	3690,	-5040,	-1710,	5760,	-540,	-5490,	2700,	4410,	-4410,	-2700,	5490,	540,	-5760,	1710,	5040,	-3690,	},

{	3240,	-5670,	1080,	4770,	-4770,	-1080,	5670,	-3240,	-3240,	5670,	-1080,	-4770,	4770,	1080,	-5670,	3240,	},

{	2591,	-5728,	3773,	1806,	-5528,	4379,	474,	-5167,	5167,	-474,	-4379,	5528,	-1806,	-3773,	5728,	-2591,	},

{	2304,	-5312,	5312,	-2304,	-2304,	5312,	-5312,	2304,	2304,	-5312,	5312,	-2304,	-2304,	5312,	-5312,	2304,	},

{	1639,	-4506,	5747,	-5096,	2802,	619,	-3568,	5543,	-5543,	3568,	-619,	-2802,	5096,	-5747,	4506,	-1639,	},

{	1088,	-3264,	4800,	-5696,	5696,	-4800,	3264,	-1088,	-1088,	3264,	-4800,	5696,	-5696,	4800,	-3264,	1088,	},

{	512,	-1664,	2688,	-3712,	4480,	-5120,	5504,	-5760,	5760,	-5504,	5120,	-4480,	3712,	-2688,	1664,	-512,	},

32x32 transform:

{	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	},

{	5760,	5696,	5632,	5440,	5248,	4992,	4608,	4288,	3904,	3456,	2944,	2432,	1984,	1408,	832,	256,	-256,	-832,	-1408,	-1984,	-2432,	-2944,	-3456,	-3904,	-4288,	-4608,	-4992,	-5248,	-5440,	-5632,	-5696,	-5760,	},

{	5760,	5504,	5120,	4480,	3712,	2688,	1664,	512,	-512,	-1664,	-2688,	-3712,	-4480,	-5120,	-5504,	-5760,	-5760,	-5504,	-5120,	-4480,	-3712,	-2688,	-1664,	-512,	512,	1664,	2688,	3712,	4480,	5120,	5504,	5760,	},

{	5675,	5223,	4301,	3000,	1350,	-308,	-1935,	-3425,	-4650,	-5445,	-5806,	-5600,	-4925,	-3927,	-2464,	-875,	875,	2464,	3927,	4925,	5600,	5806,	5445,	4650,	3425,	1935,	308,	-1350,	-3000,	-4301,	-5223,	-5675,	},

{	5696,	4800,	3264,	1088,	-1088,	-3264,	-4800,	-5696,	-5696,	-4800,	-3264,	-1088,	1088,	3264,	4800,	5696,	5696,	4800,	3264,	1088,	-1088,	-3264,	-4800,	-5696,	-5696,	-4800,	-3264,	-1088,	1088,	3264,	4800,	5696,	},

{	5622,	4249,	2002,	-933,	-3378,	-5246,	-5760,	-4953,	-3039,	-270,	2478,	4710,	5727,	5456,	3893,	1332,	-1332,	-3893,	-5456,	-5727,	-4710,	-2478,	270,	3039,	4953,	5760,	5246,	3378,	933,	-2002,	-4249,	-5622,	},

{	5543,	3568,	619,	-2802,	-5096,	-5747,	-4506,	-1639,	1639,	4506,	5747,	5096,	2802,	-619,	-3568,	-5543,	-5543,	-3568,	-619,	2802,	5096,	5747,	4506,	1639,	-1639,	-4506,	-5747,	-5096,	-2802,	619,	3568,	5543,	},

{	5449,	2865,	-759,	-4329,	-5772,	-4570,	-1521,	2553,	5204,	5553,	3560,	-334,	-3842,	-5753,	-4990,	-1877,	1877,	4990,	5753,	3842,	334,	-3560,	-5553,	-5204,	-2553,	1521,	4570,	5772,	4329,	759,	-2865,	-5449,	},

{	5312,	2304,	-2304,	-5312,	-5312,	-2304,	2304,	5312,	5312,	2304,	-2304,	-5312,	-5312,	-2304,	2304,	5312,	5312,	2304,	-2304,	-5312,	-5312,	-2304,	2304,	5312,	5312,	2304,	-2304,	-5312,	-5312,	-2304,	2304,	5312,	},

{	5261,	1318,	-3377,	-5786,	-3986,	904,	4905,	5420,	2055,	-3015,	-5722,	-4188,	153,	4719,	5601,	2353,	-2353,	-5601,	-4719,	-153,	4188,	5722,	3015,	-2055,	-5420,	-4905,	-904,	3986,	5786,	3377,	-1318,	-5261,	},

{	5167,	474,	-4379,	-5528,	-1806,	3773,	5728,	2591,	-2591,	-5728,	-3773,	1806,	5528,	4379,	-474,	-5167,	-5167,	-474,	4379,	5528,	1806,	-3773,	-5728,	-2591,	2591,	5728,	3773,	-1806,	-5528,	-4379,	474,	5167,	},

{	4957,	-300,	-5225,	-4676,	878,	5450,	4275,	-1419,	-5642,	-3825,	1900,	5736,	3447,	-2475,	-5725,	-2981,	2981,	5725,	2475,	-3447,	-5736,	-1900,	3825,	5642,	1419,	-4275,	-5450,	-878,	4676,	5225,	300,	-4957,	},

{	4770,	-1080,	-5670,	-3240,	3240,	5670,	1080,	-4770,	-4770,	1080,	5670,	3240,	-3240,	-5670,	-1080,	4770,	4770,	-1080,	-5670,	-3240,	3240,	5670,	1080,	-4770,	-4770,	1080,	5670,	3240,	-3240,	-5670,	-1080,	4770,	},

{	4626,	-1887,	-5808,	-1417,	4966,	4362,	-2538,	-5747,	-821,	5184,	3834,	-2978,	-5621,	-330,	5451,	3452,	-3452,	-5451,	330,	5621,	2978,	-3834,	-5184,	821,	5747,	2538,	-4362,	-4966,	1417,	5808,	1887,	-4626,	},

{	4410,	-2700,	-5490,	540,	5760,	1710,	-5040,	-3690,	3690,	5040,	-1710,	-5760,	-540,	5490,	2700,	-4410,	-4410,	2700,	5490,	-540,	-5760,	-1710,	5040,	3690,	-3690,	-5040,	1710,	5760,	540,	-5490,	-2700,	4410,	},

{	4230,	-3420,	-4950,	2430,	5400,	-1440,	-5670,	270,	5760,	810,	-5580,	-1980,	5220,	2970,	-4590,	-3870,	3870,	4590,	-2970,	-5220,	1980,	5580,	-810,	-5760,	-270,	5670,	1440,	-5400,	-2430,	4950,	3420,	-4230,	},

{	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	},

{	3870,	-4590,	-2970,	5220,	1980,	-5580,	-810,	5760,	-270,	-5670,	1440,	5400,	-2430,	-4950,	3420,	4230,	-4230,	-3420,	4950,	2430,	-5400,	-1440,	5670,	270,	-5760,	810,	5580,	-1980,	-5220,	2970,	4590,	-3870,	},

{	3690,	-5040,	-1710,	5760,	-540,	-5490,	2700,	4410,	-4410,	-2700,	5490,	540,	-5760,	1710,	5040,	-3690,	-3690,	5040,	1710,	-5760,	540,	5490,	-2700,	-4410,	4410,	2700,	-5490,	-540,	5760,	-1710,	-5040,	3690,	},

{	3452,	-5451,	-330,	5621,	-2978,	-3834,	5184,	821,	-5747,	2538,	4362,	-4966,	-1417,	5808,	-1887,	-4626,	4626,	1887,	-5808,	1417,	4966,	-4362,	-2538,	5747,	-821,	-5184,	3834,	2978,	-5621,	330,	5451,	-3452,	},

{	3240,	-5670,	1080,	4770,	-4770,	-1080,	5670,	-3240,	-3240,	5670,	-1080,	-4770,	4770,	1080,	-5670,	3240,	3240,	-5670,	1080,	4770,	-4770,	-1080,	5670,	-3240,	-3240,	5670,	-1080,	-4770,	4770,	1080,	-5670,	3240,	},

{	2981,	-5725,	2475,	3447,	-5736,	1900,	3825,	-5642,	1419,	4275,	-5450,	878,	4676,	-5225,	300,	4957,	-4957,	-300,	5225,	-4676,	-878,	5450,	-4275,	-1419,	5642,	-3825,	-1900,	5736,	-3447,	-2475,	5725,	-2981,	},

{	2591,	-5728,	3773,	1806,	-5528,	4379,	474,	-5167,	5167,	-474,	-4379,	5528,	-1806,	-3773,	5728,	-2591,	-2591,	5728,	-3773,	-1806,	5528,	-4379,	-474,	5167,	-5167,	474,	4379,	-5528,	1806,	3773,	-5728,	2591,	},

{	2353,	-5601,	4719,	-153,	-4188,	5722,	-3015,	-2055,	5420,	-4905,	904,	3986,	-5786,	3377,	1318,	-5261,	5261,	-1318,	-3377,	5786,	-3986,	-904,	4905,	-5420,	2055,	3015,	-5722,	4188,	153,	-4719,	5601,	-2353,	},

{	2304,	-5312,	5312,	-2304,	-2304,	5312,	-5312,	2304,	2304,	-5312,	5312,	-2304,	-2304,	5312,	-5312,	2304,	2304,	-5312,	5312,	-2304,	-2304,	5312,	-5312,	2304,	2304,	-5312,	5312,	-2304,	-2304,	5312,	-5312,	2304,	},

{	1877,	-4990,	5753,	-3842,	334,	3560,	-5553,	5204,	-2553,	-1521,	4570,	-5772,	4329,	-759,	-2865,	5449,	-5449,	2865,	759,	-4329,	5772,	-4570,	1521,	2553,	-5204,	5553,	-3560,	-334,	3842,	-5753,	4990,	-1877,	},

{	1639,	-4506,	5747,	-5096,	2802,	619,	-3568,	5543,	-5543,	3568,	-619,	-2802,	5096,	-5747,	4506,	-1639,	-1639,	4506,	-5747,	5096,	-2802,	-619,	3568,	-5543,	5543,	-3568,	619,	2802,	-5096,	5747,	-4506,	1639,	},

{	1332,	-3893,	5456,	-5727,	4710,	-2478,	-270,	3039,	-4953,	5760,	-5246,	3378,	-933,	-2002,	4249,	-5622,	5622,	-4249,	2002,	933,	-3378,	5246,	-5760,	4953,	-3039,	270,	2478,	-4710,	5727,	-5456,	3893,	-1332,	},

{	1088,	-3264,	4800,	-5696,	5696,	-4800,	3264,	-1088,	-1088,	3264,	-4800,	5696,	-5696,	4800,	-3264,	1088,	1088,	-3264,	4800,	-5696,	5696,	-4800,	3264,	-1088,	-1088,	3264,	-4800,	5696,	-5696,	4800,	-3264,	1088,	},

{	875,	-2464,	3927,	-4925,	5600,	-5806,	5445,	-4650,	3425,	-1935,	308,	1350,	-3000,	4301,	-5223,	5675,	-5675,	5223,	-4301,	3000,	-1350,	-308,	1935,	-3425,	4650,	-5445,	5806,	-5600,	4925,	-3927,	2464,	-875,	},

{	512,	-1664,	2688,	-3712,	4480,	-5120,	5504,	-5760,	5760,	-5504,	5120,	-4480,	3712,	-2688,	1664,	-512,	-512,	1664,	-2688,	3712,	-4480,	5120,	-5504,	5760,	-5760,	5504,	-5120,	4480,	-3712,	2688,	-1664,	512,	},

{	256,	-832,	1408,	-1984,	2432,	-2944,	3456,	-3904,	4288,	-4608,	4992,	-5248,	5440,	-5632,	5696,	-5760,	5760,	-5696,	5632,	-5440,	5248,	-4992,	4608,	-4288,	3904,	-3456,	2944,	-2432,	1984,	-1408,	832,	-256,	},