| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F252 |
| --- | --- |

| Title: | Block-size and pixel position independent boundary smoothing for non-directional Intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A.Alshin, E.Alshina #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +82 10 30261305 alexander_b.alshin@samsung.com elena_a.alshina@samsung.com |
| Source: | Samsung Electronics, Ltd |  |  |

_____________________________

# Abstract

This proposal introduces test results if boundary smoothing which is applied in HM3.0 for non-directional Intra prediction was changed to use the same smoothing algorithm for all PU sizes and all pixels regardless position in a block. No performance drop reported in all-intra test for both Luma and Chroma components. Additionally extension of boundary smoothing for 32x32 Intra non-directional predictions was tested. No performance change for Luma component and 0.1% gain for Chroma component was observed in this test.

# Introduction

HM3.0 uses LUT-based adaptive filtering on intra prediction samples introduced by [1]. This is smoothing which is applied for boundary pixels in case of non-directional predictions. This tool is targeting for block artifacts removing. Smoothing filter is different according to block size and pixel position in a block. Fig 1-2 explain filter selection rules in HM3.0 (picture was borrowed from [1]). This boundary smoothing is applied for Intra PU with sizes from 4x4 to 16x16 in HM3.0. No boundary smoothing is applied for 32x32 Intra Pus.

Figure 1. Non-directional prediction samples to be filtered

Figure 2. Filter coefficients for non-directional prediction edges except the top-left corner sample

Figure 3. Filter coefficients for top-left corner sample of non-directional prediction

# Unification of boundary smoothing

First we checked performance change if the same smoothing (Fig. 3 , Filter #B) filter is applied for all boundary pixels regardless position for non-directional Intra PU. This filter can be implemented with 1 shift and 2 adds operations (no multiplications compare to other variants).

We keep application for this tool for PU sizes from 4x4 to 16x16.

Test results for all-Intra test are shown in Table 1. No performance change in average was observed both for Luma and Chroma components. Small gain (0.1%) appears due to this unification in Chroma component for high resolution videos.

Table 1. Unification of boundary smoothing.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | -0.1 | -0.1 | 0.0 | -0.1 | -0.1 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

We would like to thank TI for verification for these test results.

# Extension for 32x32 Intra PU of boundary smoothing

As was noticed in previous test some gain comes from unification for high resolution videos. This indicates that this tool is more efficient for large blocks. So we extended this tool for 32x32 non-directional Intra PUs. In this design smoothing filter is the same for all PU sizes and all pixel positions.

Test results for all-Intra test are shown in Table 2. No performance change in average was observed both for Luma and 0,1% gain was observed for Chroma component due to this change.

There is no noticeable change of encoding time and small reduction of decoding time.

Table 2. Unification of boundary smoothing extended to 32x32 PU

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1 | -0.2 | -0.2 | -0.1 | -0.3 | -0.3 |
| Class B | 0.0 | -0.1 | -0.2 | 0.1 | -0.1 | -0.1 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class D | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | -0.1 | -0.1 | 0.0 | -0.1 | -0.1 |
| Overall | 0.0 | -0.1 | -0.1 | 0.0 | -0.1 | -0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

We would like to thank TI for verification for these test results.

Tested modification of DC mode with boundary smoothing can be described by following pseudo-code:

predSamples[ 0, 0 ] = ( p[ -1, 0 ] + (DCVal <<1)+  p[ 0, -1 ] + 2 ) >> 2	                (1)

predSamples[ x, 0 ] = ( p[ x, -1 ] + (DCVal<<1) + predSamples [x-1,0] + 2 ) >> 2, with x = 1..nS-1
predSamples[ 0, y ] = ( p[ -1, y ] + (DCVal<<1)+ predSamples [0,y-1] + 2 ) >> 2, with y = 1..nS-1
predSamples[ x, y ] = DCVal, with x, y = 1..nS-1

Here DCVal is prediction (the same in all points for DC mode), predSamples output array of DC prediction, p[x,y] are reference samples.

# Independent pixel processing

In the last test we apply 3-taps filter B (Fig 3) in corner position only. On sides 2 taps filter B (Fig 2) is used. This allows processing all PU pixels in parallel (in opposite to (1) where already smoothed neighbor pixels are used for current pixel prediction):

predSamples[ 0, 0 ] = ( p[ -1, 0 ] + (DCVal <<1)+  p[ 0, -1 ] + 2 ) >> 2	                (2)

predSamples[ x, 0 ] = ( p[ x, -1 ] + (DCVal<<1)+ DCVal  + 2 ) >> 2, with x = 1..nS-1
predSamples[ 0, y ] = ( p[ -1, y ] + (DCVal<<1)+ DCVal  + 2 ) >> 2, with y = 1..nS-1
predSamples[ x, y ] = DCVal, with x, y = 1..nS-1

Test results for all-Intra test are shown in Table 3.

Table 3. 3 taps filter in a corner, 2-taps on side boundaries.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0,2 | -0,3 | -0,2 | -0,1 | -0,2 | -0,2 |
| Class B | 0,0 | -0,1 | -0,2 | 0,0 | -0,1 | -0,1 |
| Class C | 0,0 | -0,1 | 0,0 | 0,0 | 0,0 | 0,0 |
| Class D | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| Class E | 0,0 | -0,1 | -0,1 | 0,0 | -0,1 | -0,1 |
| Overall | -0,04 | -0,12 | -0,12 | -0,02 | -0,09 | -0,10 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

We would like to thank Nokia for verification for these test results.

# Conclusion

We propose to modify boundary smoothing for non-directional Intra prediction and remove dependency of on pixel position and block size. Independ processing of pixels

predSamples[ 0, 0 ] = ( p[ -1, 0 ] + (DCVal <<1)+  p[ 0, -1 ] + 2 ) >> 2

predSamples[ x, 0 ] = ( p[ x, -1 ] + (DCVal<<1)+ DCVal  + 2 ) >> 2, with x = 1..nS-1
predSamples[ 0, y ] = ( p[ -1, y ] + (DCVal<<1)+ DCVal  + 2 ) >> 2, with y = 1..nS-1
predSamples[ x, y ] = DCVal, with x, y = 1..nS-1

is recommended. We propose adopt changes for new release of HM s/w and WD.

# Patent rights declaration(s)

Samsung Electronics, Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).