| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F254 |
| --- | --- |

| Title: | Multi-parameter probability up-date for CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A.Alshin, E.Alshina #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +82 10 30261305 alexander_b.alshin@samsung.com elena_a.alshina@samsung.com |
| Source: | Samsung Electronics, Ltd |  |  |

_____________________________

# Abstract

This contribution provides description of multi-parameter probability up-date for HEVC CABAC. Described idea implemented on-top of HM 3.0 and tested using common test conditions. Average performance improvement achieved is 0.8% with less than 1% encoding time and no increase of decoding time.

# Introduction

Entropy coding is the one of the most important parts for all advanced video compression schemes. Context-adaptive binary arithmetic coding (CABAC) is entropy coding used H.264/MPEG-4 AVC and HEVC standardization process.

Probability up-date is a key factor of CABAC making this arithmetic coder the content adaptive one. Proposed method affects only for probability up-date and interval (range) subdivision. All other parts of HEVC-CABAC are not modified.

# Idea description

The selection of probability up-date model is a key factor. Initial estimation of model parameters is based on a-priori information for compressed data. Then for every next step adaptation is going on for new continuously changing conditions. Intensive investigation of adaptive prediction methods was launched after well-known paper of C.C.Holt [1] devoted to problem of exponential smoothing in 1957. Modification and generalization of this model led to appearance whole family of adaptive models with different properties

Probability up-date. In modern arithmetic codices [2] for probability update the following formula is the most frequently used:

pnew = y+(1- ) pold.                (1)

Here y is equal to “zero” if current symbol matches with most probable symbol (MPS) otherwise y is equal to “one”. This formula provides estimating value for probability of least probable symbol (LPS).

The parameter N=1/ is a measure for number of previously encoded bins which have significant influence to the current up-date (“window size”). It is also possible to say that this value determines average memory of system. The choice of parameter which determines sensitivity of the model is difficult and important problem. Sensitive system quickly reacts to real changing but less sensitive model does not react to noise and random errors. Both properties are useful but contradictory. Using different control signals it is possible to change  during encoding however such approaches are very labor-consuming. Instead of this we propose to calculate simultaneously several values with different i:

pi new =  i y+(1- i) pi old                          (2)

and use weighted average as next bin probability prediction:

p new =i pi new.             (3)

Integerization.

Real calculations by formula (1) are carried out using look-up tables. Probabilities are represented one of 64 discrete values on exponential mesh pn=0.5(1-)n  covered interval from 0 to 1/2. In case of most probable symbol up-date by formula (1) on this mesh executes without error accumulation (exactly from point to point) only if LPS (more rare case) is coded we need some rounding for matching with closest mesh knots. However this mesh is very dense near 0 and sparse near 1/2

Fig. 1. Exponential mesh for probability.

therefore for data having probability near to 1/2 evaluation of probability can be done with significant error.

Also increase of probability bit depth leads to significant increase look-up table size. This is another disadvantage of exponential mesh.

Instead of look-up tables (m_aucNextStateMPS and m_aucNextStateLPS) and exponential mesh for probability which are utilized in AVC CABAC we proposed to use uniform mesh and explicit calculation with multiplication free formula for probability up-date.

Suppose that probability pi is represented by integer number Pi from 0 to 2k that is

pi=Pi/2k

Let’s i  be power of two (i =1/2Mi ) then we come to multiplication free formula for probability update:

Pi = (Y >> Mi) + P – (Pi >> Mi).

This formula predicts probability that next bin will be “1”. Here Y = 2k  if last coding bin is “1” and Y=0 if last coding bin is “0”, “>>Mi” is right arithmetic shift for Mi bits.

Our numerous experiments show that reasonable values of are {3, 4,…8}. This corresponds “window size” Ni =1/i {8, 16, 32, 64, 128, 256}. For comparison corresponding value for CABAC is approximately equal 19.69 . It is important that calculations for each Pi are independent and can be easily parallelized.

Two-probabilities update model. Keeping balance between complexity increase and performance improvement let’s consider that linear combination for probability estimation consists of only two summands corresponding N0=16 and N1=128:

P0 = (Y>>4) + P0 – (P0>>4)                (4)

P1 = (Y>>7) + P1 – (P0>>7)                (5)

P = (P0 + P1+1)>>1                        (6)

MPS/LPS concept. Floating point value which corresponds to AVC CABAC’s probability is always less or equal 1/2. If probability exceeds this limit LPS becomes MPS keeping probability inside interval mentioned above. This concept has some clear advantages one is reduction of look-up table size.

However in case of multi-parameter update model direct generalization of this method meets some difficulties. In practice one probability estimation can exceed limit another will be still less than 1/2. Therefore either we need MPS/LPS switching for each Pi or must do it for some average value. In both cases it introduces additional complexity without noticeable performance improvement. So we propose to increase permissible level of probability (in terms of float-point values) up to 1 and to forbid MPS/LPS switching.

Initialization. During encoding process so-called “reset entropy” occurs in the beginning of each slice. It means that arithmetic engine drops current probabilities and starts coding with predetermined a-priori known probability values. However there is no guarantee that this initial value is close to optimal for current content. Process of transition from initial value is shown on the figure.

Fig. 2. Probability up-date: Short-distance probability up-date model (B) and semi-sum (C).

On the figure special model example is represented: quasi-random sequence consisting of “1” and “0” with constant probability p=0.75 was evaluated by two probability update models with N0=16, and “semi sum” of  N0=16 and N0=128. Initial value for probability in this test was pini=0.25.

The model with short “window size” converges to optimal value very fast; however then this model is subject to fluctuations in a large measure. The model “semi- sum” tends to optimal value slowly but later reaction for small perturbations is not so noticeable.

We can draw the following conclusion from this observation. For short transition period only short distance prediction is preferable. But after stabilization near optimal value two-probability update model is more accurate for majority of context models.

We proposed to introduce a counter of updates since last initialization. After every up-date the counter increases by one. Until the counter exceeds some threshold only short “window size” model (4) will be used. When the counter reaches threshold we should switch to more accurate two-probability update model (6). Our experiments definitely show that optimal threshold belongs to diapason from 20 to 70 up-dates. All test results represented bellow obtained with threshold equal to 50.

It is necessary to notice that even if we don’t use second probability value with long distance prediction we need always update it in order to use correct value after the counter exceeds threshold.

The advantage of this approach mostly reveals for video with small size because “reset entropy” happens more often. It is necessary to emphasize that determining of initialization values for every context model is a case study that is why for honest comparison we used exactly same values that are utilized for initialization procedure in CABAC.

Interval (range) subdivision.

Interval subdivision can be done by direct calculation or look-up table technique. If probability has k bits representation than

L=(range P+2k-1)>>k      (7)

It is necessary to separate L from “0” and “range”

0<   L  range - 

Calculations by formula (7) can be substituted by look-up table.

# Test results

The proposed algorithm was implemented on the top of HM3.0 s/w and tested under common test conditions [3]. Three high efficiency test cases: All Intra, Random Access and Low Delay were investigated.

For stated test conditions the initialization may have influence to performance especially for small resolution video. For 128 possible states in CABAC one-by-one mapping with 128 corresponding points on uniform mesh was used. In order to minimize difference between corresponded procedures in HM 3.0 and proposed method only these values were used for the initialization.

Table 1. Test results for multi-parameters probability up-date model in HM3.0.

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.9 | 0.0 | 0.6 | -0.8 | 0.7 | 1.0 |  |  |  |
| Class B | -0.9 | -0.9 | -0.7 | -0.9 | -0.5 | 0.0 | -0.8 | 0.1 | 0.4 |
| Class C | -0.8 | -1.0 | -1.0 | -0.7 | -0.4 | -0.6 | -0.7 | -0.3 | -0.1 |
| Class D | -0.8 | -0.7 | -0.7 | -0.5 | -0.2 | -0.4 | -0.5 | 0.4 | 0.4 |
| Class E | -0.9 | -0.4 | -0.6 |  |  |  | -0.6 | 1.3 | -0.6 |
| Overall | -0.87 | -0.6 | -0.5 | -0.74 | -0.1 | 0.0 | -0.67 | 0.3 | 0.1 |
| Enc Time[%] | 100.7% |  |  | 100.4% |  |  | 100.3% |  |  |
| Dec Time[%] | 100.2% |  |  | 100.0% |  |  | 99.5% |  |  |

Test results are summarized in Table 1. Calculation by formula (7) was substituted by LUT (size 128x32). Thereby proposed technique provides 0.76% bit saving in average on 3 tests with 0.1% decoder complexity reduction and 0.5% encoder complexity increase. This data shows that proposed technique doesn’t affect significantly for encoder/decoder complexity. For realization of this algorithm we need allocate additional 4 bytes for each context model.

We would like to thank RIM for verification of this test results, deep analysis of our code and a lot of useful comments about design improvement.

# Conclusions

Based on presented test results :

avg. gain 0.8% ; avg. Enc/Dec time  100%

we propose to create core experiment for study of multi-parameters probability up-date for HEVC-CABAC or directly adopt proposed multi-parameters probability up-date for CABAC to the next version of HM s/w and WD.

# References

[1] Holt C.C. Forecasting trends and seasonal by exponentially weighted moving averages. //O.N.R. Memorandum, Carnegie Inst. Of Technology. 1957. №2.

[2] D. Marpe, H. Schwarz, and T. Wiegand: Context-Based Adaptive Binary Arithmetic Coding in the H.264 / AVC Video Compression Standard, IEEE Transactions on Circuits and Systems for Video Technology, Vol. 13, No. 7, pp. 620-636, July 2003.

[3] F. Bossen Common HM test conditions and software reference configurations JCTVC-E700, 5th. JCTVC, Meeting, Geneva, Mar. 2011.

# Patent rights declaration(s)

Samsung Electronics, Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).