| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F257 |
| --- | --- |

| Title: | About clip operation removal from de-quantization part of HM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A.Alshin, E.Alshina #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +82 10 30261305 alexander_b.alshin@samsung.com elena_a.alshina@samsung.com |
| Source: | Samsung Electronics, Ltd |  |  |

_____________________________

# Abstract

This proposal introduces restriction for quantized coefficients values depending on QP and transforms size. If Encoder doesn’t generate quantized coefficients with magnitude over this limit then clip operation can be removed from de-quantization part of HM transform. No overflow of 16 bits buffer is guaranteed in this case.

# Introduction

HM3.0 uses core transform introduced by [1]. Additionally for Intra 4x4 TU DST is applied according to [2]. The flow-chart of inverse transform part is shown on Fig.1.

Fig 1. Current design.

Out-put of swadowed block are storied in temporal buffer. This is desiarable to keep values to be storied within 16 bits.

# De-quantization clip removal

The clip operation is applied to out-put of de-Qunatizer:

iCoeffQ = ((piQCoef[n]*(Int) g_auiIQ [QP%6]<< m_cQP.m_iPer)+iAdd)>>iShift;   (1)

piCoef[n] = Clip3(-32768,32767,iCoeffQ);              (2)

Here iCoeffQ is de-Quantized coefficients and piCoef[n] is out-put of de-Quantizer. If p is input data bit depth (parameter uiBitDepth in HM3.0) then parameters values in (1) are:

iShift = p -9 + log2S; iAdd = 1 << (iShift-1);

m_cQP.m_iPer = QP/6+p-8;

uiQ = g_auiIQ[QP%6]; g_auiIQ = {40,45,51,57,64,72};

It was noticed that simple remove of this clip doesn’t affect to performance on common test conditions. There is no performance change even for Low and High QP test (following CE10 description [3]).

This means that pixel-wise clip operation doesn’t work in HM3.0 inverse trasnfrom part. Moreover even if such big value of quantized coefficient which will break in-equality:

-32768 iCoeffQ 32767                    (3)

will appear in bit-stream then this value will be cliped after de-qauntization and will not be used in decode-side. In order to reduce decode complexity and avoid extremely large values of quantized coefficients in bit-stream we would like to propose removal of clip (2).

But even the break of in-equality (3) doesn’t happen under common test conditions the overflow of short type is theoretically possible in de-Quantizer.

In order to guarantee no overflow of 16 bit buffer

MAX_C = 32767

we’ve calculated the maximum possible value for quantized coefficents:

iMax = MAX_C * 2 (log2S-1 – iQP/6 ) / g_auiIQ[QP%6];                  (4)

If qunatized coefficients are cliped into the range:

piQCoef[n] = Clip3(-iMax,iMax,piQCoef[n])                    (5)

then no overflow of (3) is possible and clip (2) can be safely remove. Maximum absolut value in (5) depends Qp and trasnfrom size.

Fig.2 Proposed design

# Test results.

This modification was implemented in HM3.0 and tested using 6 test confugurations under common test conditions [4]. Additionally Low and High QP 2 sec tests were done in order to prove that there is no performance change even in extreme cases.

Table 1: Test results for clip operation removed from de-quantizer.

Normal QP test:

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class B | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class C | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class D | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class E | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| All | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class B | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class C | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class D | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class E |  |  |  |  |  |  |
| All | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class C | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class D | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class E | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| All | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Low QP test.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class B | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class C | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class D | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class E | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| All | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 96% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class B | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class C | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class D | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class E |  |  |  |  |  |  |
| All | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class C | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class D | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class E | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| All | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

High QP test.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class B | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class C | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class D | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class E | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| All | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class B | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class C | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class D | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class E |  |  |  |  |  |  |
| All | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class C | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class D | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Class E | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| All | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

No performance change was observed in tests at Low, Normal and High QP. This is easy to uderstand. Our analysis of encoder process and forward trasnfrom shows that the quantized coefficients out of range (5) may happen at QP=0 in very rare case: several residual values in a block in sertain positions should maximum possible value (this means that in these points prediction is absolutely wrong: 0 is predicted by 255).

We would like to thank Cisco for verification of this test results.

# Conclusion

We propose to remove clip operation from de-quantization part of HM and use clip operation (5) with parameter (4) instead this. Encoder shall not generate quantized coefficients with magnitude over redetermined limit (4)-(5). This guidline for Encoder should be included into HEVC specification.

# Patent rights declaration(s)

Samsung Electronics, Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).