| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F259 |
| --- | --- |

| Title: | CE3: Cross-check report for test1 JCTVC-F315 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Kenji Kondo and Teruhiko Suzuki W12F 1-11-1 GCO Osaki, Shinagawa, 141-0032 Tokyo, Japan | Tel: Email: | +81-3-5435-3308 Kenji.Kondo@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-verification result of CE3 Subtest 1 described in JCTVC-F315. Cross-verification task has been successfully completed, and it is confirmed that the BD-rate results match those provided by proponent.

# Software analysis

The following modifications are confirmed through the analysis of software provided by TOSHIBA:

- Switch number of filter tap according prediction mode and sub pel location:

  - For bi-prediction

    - Sub pel position is quarter pel, 6 tap FIR filter is used.

    - Sub pel position is half pel, 8 tap as same as HM is used.

  - Otherwise, 8 tap as same as HM is used.

- If two sub pel positions of L0 and L1 are the same (e.g. e position for both L0 and L1 as shown in Figure 1), those referenced pixels of L0 and L1 are averaged at 1st step then averaged sample value is interpolated (e.g. as e position).

Figure 1	Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for quarter sample luma interpolation.

# Results

Software provided by Toshiba was successfully compiled by Gnu Compiler Collection (GCC) version 4.1.2-50 although minor warnings were found (e.g. unused variable).

Simulation was conducted according to the CE3 common test coding conditions based on JCTVC-E700. Attached Excel sheets, JCTVC-F259.xls, contain the detailed results. It was confirmed that BD-PSNR/Rate values obtained are the same as those of provided by proponent.

And encoding time and decoding time were measured on same platform machines that spec is Core i7 CPU 3.4GHz, and have 8 GB memory. And the Linux 64 bit (called Cent OS) is installed in the platform machine.

For encode, cluster that consists of above identical platform was used. For decode, only 1 computer in the cluster was used, and decoding time was measured without yuv output.

# Conclusion

It is confirmed that the CE3 test 1 results reported by proponent are accurate.

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).